7-NOV-2002

K.F. Johnson (kurtis@hep.fsu.edu)

FSU

Dear HEM-ers:

Following is some data on 3M’s HEM material.

# SPECTRA

Below is a plot of the emission and excitation spectra which I obtained using a Varian Cary Eclipse spectrophotometer/fluorimeter.   There are three curves on this plot:   the emission when the “blue” side is excited at 300 nm, the emission when the “yellow” side is excited, and the excitation scan of 420nm light.  Although the plot shows approximately equal heights of the emission curves, in fact the blue side curve is about 3 times higher than the yellow side curve.  I have re-scaled them to approximately equal heights so that comparison of their features is simpler.

The general features of the emission curves are fluorescence at ~425nm  and a FWHM of 60nm.

The peaks occur at slightly different wavelengths, 423 and 429nm.  To me this indicates that, at a guess, the blue fluorescence is generated by  (a)  layer(s)  on the blue side and partially absorbed by layer(s) on the yellow side.  This supposition would account for both the lower fluorescence strength and longer wavelength at the maximum of the yellow side.

The excitation curve shows the (relative) quantity of 420nm light produced when HEM is excited by light of various wavelengths, starting at 300nm and contnueing through 450nm (and, yes, I am aware that one should not see anything above 420nm).  The peak at 420nm should be just the scattered incoming excitation light.  The width of this peak gives us a feel for the instrument function, e.g. there is a “smoothing” of about 5nm of the excitation spectrum.

A few random comments which may or may not be of importance: HEM has a 1-dimensional crystaline structure (layered).  It seems likely that the emitted light will have different properties (intensity, wavelength) at different angles to the normal.  I haven’t checked this.  These measurements were done at an angle  of ~45 degrees between the excitation and emitted light.  Also, there may be additional fluorescence lines – I seem to remember seeing green when looking at HEM while at CERN – but I haven’t seen green yet here.

# DECAY TIME

The decay time was measured by flashing the HEM with a 337nm UV laser pulse (4ns width) and observing the resulting fluorescence with a back-biased photo-diode.  The signal was captured by a LeCroy digital oscilloscope ( type 9361c, 2.5GS/s  ).  The 10%-to-90% rise time is less than 10ns and the decay time is 55ns (90%-to-10%).  Note that each major time tick represents 20ns.