APPENDIX - D

MATLAB CODE FOR THE MATLAB MODEL OF THE MAIN DATA PATH

D.1 Main design file

% VHDL design and FPLD implementation of the Silicon Track Card

% Shweta Lolage

% this is the main batch file used to run the whole code

% the downloaded parameters are written in the download_para.m file

read_downloaded_parameter;

% this file is used to take the data from teh commented downloaded_parameter.m file and

%write the data stream into down_data.m file where it can be easily accessed by the modules

smt_filter;

% this file takes the downloaded parameters required - cav, dav, lnkrdy, delay, event_no

% it takes VTM data from vtm_data.m

% writes the processed data in binary form in smt_data.m

strip_reader;

% this file takes the downloaded parameters required - seqid , hdiid

% data type gain and offset are still taken from the user

% bad channel data is taken from the bad_chan_mem.m file

% takes data from smt_data.m

% writes the processed data into strip_data.m

cluster_finder;

% this file takes the downloaded parameters required -  two data-threshold values

% takes data from strip_data.m

% writes the processed data into cluster_data.m

centroid_calculator;

% this file takes the downloaded parameters required - three pulse-threshold values

% the cluster-type is taken as input from user

% takes data from cluster_data.m

% writes the calculated centroids into centroid_data.m

hit_filter;

% takes data from roads_data.m, which is road data LUT and centroid_data.m

% writes the processed hits into hits_data.m

D.2 Read downloaded parameter

%This program is used to convert the commented

%downloaded data into a bit and byte stream

% which can be read by the modules

clear all

clc

do = [];

fid = fopen ('downloaded_parameter.m','r');

t  = fread(fid);

t1 = size (t);

t2 = t1(1);

y = 1;

mark = 0;

while y <= t2,

if t(y) == 37

if mark == 0

mark = 1;

y = y + 1;

else

mark = 0;

y = y + 1;

end

else

if mark == 0

if t(y) == 13 |t(y) ==10 | t(y) = 32

y = y + 1;

else

do1 = t(y);

do = [do do1];

y = y + 1;

end

else

y = y + 1;

end

end

end

s = size(do);

s1 = s(2);

fid = fopen('down_data.m','w');

cluster = fwrite( fid,do);

st = fclose('all');

D.3 SMT Filter

% VHDL design and FPLD implementation of the Silicon Track Card

% Shweta Lolage

% this the code to realise the SMT data filter

clear all

clc

smt_out = [];

smt_out_p = [];

end_of_event = 0;

z= [];

error = [];

serr = 0;

event_frc = '00';

fid = fopen('down_data.m','r');

A1 = fread(fid);

o=hex2dec(A1);

st = fclose(fid);

cav = o(1);

dav = o(2);

ndav = not(dav);

lnkrdy = o(3);

nlnkrdy = not(lnkrdy);

disable = o(4);

ndisable = not(disable);

delay = o(5)*16 + o(6);

delay_b = dec2bin(delay,8);

event_in = o(7)*16 + o(8);

event_in_b = dec2bin (event_in, 8);

fid= fopen ('vtm_data.m','r');

x=fid;

A=fread(fid);

y=hex2dec(A);

st = fclose(fid);

kq=size(y);

kqq=kq(1);

for k=1:3:kqq

error1 = y(k);

error = [error error1];

z1=y(k+1)*16+y(k+2);

z=[z z1];

end

datasi=size(z);

datasize=datasi(2);

% the inputs to this block other than the vtm_data, test_data and test inputs are

% frc_start

% delay_in (8 bit value)

% disable

% cav

% dav

% lnkrdy

% event_in

% smt_error_in

% Some constants required for comparison to be converted into decimal%

zC0 = hex2dec('C0');

bC0 = dec2bin(zC0,8);

wrreq = 0;

usmt_out = [];

lsmt_out = [];

enable1 = and (cav,ndisable);

enable2 = and (ndav,nlnkrdy);

enable = and (enable1, enable2);

j = 1;

if enable == 1

while j <= datasize,

end_of_event = 0;

if z(j) == zC0

serr = and (smt_error_in_u, smt_error_in_l);

serr1 = dec2bin(serr,1);

smt_out1 = ['1' serr1 bC0 event_frc]; % this is the trailer%

smt_out1_d = bin2dec (smt_out1);

smt_out3 = dec2bin (smt_out1_d,32);

smt_out = [smt_out smt_out1];

smt_out_p = [smt_out_p smt_out3];

end_of_event = 1;

j = j + 1;

if z(j) == zC0

end_of_event = 1;

while j <= datasize

j = j + 1;

end

end

else

while delay ~= 0

delay = delay - 1;

frc_start = input ('input the frc_start status');

if frc_start == 1

event_frc = input('input the new event_no (in hex)');

delay = 0;

end

end

event_frc = event_in_b;

usmt_out = z(j);

smt_error_in_u = error(j);

j = j + 1;

lsmt_out = z(j);

smt_error_in_l = error(j);

j = j + 1;

busmt_out = dec2bin(usmt_out,8);

blsmt_out = dec2bin(lsmt_out,8);

smt_out1 = [smt_error_in_u smt_error_in_l busmt_out blsmt_out];

smt_out1_d = bin2dec (smt_out1);

smt_out2 = dec2bin (smt_out1_d,18);

smt_out3 = dec2bin (smt_out1_d,32);

smt_out = [smt_out smt_out2];

smt_out_p = [smt_out_p smt_out3];

wrreq = 1;

end

end

end

fid = fopen('smt_data.m','w'); % data is written to a file

databits = fwrite( fid,smt_out);

st = fclose ('all');

disp('SMT_data processed');

D.4 Strip Reader

% VHDL design and FPLD implementation of the Silicon Track Card

% Shweta Lolage

% this the code to realise the Strip reader control

clear all

channel= [];

data = [];

z=[];

output = [];

y = [];

q = [];

r = [];

m = [];

vstate=['s' 'h' 'c' 'z' 'a' 'v'];

fid = fopen('down_data.m','r');

A2 = fread(fid);

o1= hex2dec(A2);

seqid = o1(9)*16 + o1(10);

hdiid= o1(11)*16 + o1(12);

high_z = o1(66);

low_z = o1(67);

high_stereo = o1(68);

low_stereo = o1(69);

high_axial = o1(70);

low_axial =  o1(71);

st = fclose(fid);

fid = fopen ('bad_chan_mem.m','r');

C = fread(fid);

m = hex2dec(C);

st = fclose(fid);

fid = fopen ('gain_offset.m','r');

C1= fread(fid);

go = hex2dec(C1);

st = fclose (fid);

test = input ('set 1 if test data to be used : ');

if test == 1

fid= fopen('test_data.m','r');

else

fid= fopen('smt_data.m','r');

end

B=fread(fid);

p = hex2dec(B);

st = fclose(fid);

datasi=size(p);

datasize=datasi(1);

for k=1:18:datasize

q1=p(k)*2+ p(k+1);

q = [q q1];

r1 = p(k+2)*128 + p(k+3)*64 + p(k+4)*32 + p(k+5)*16 + p(k+6)*8 + p(k+7)*4 + p(k+8)*2 +p(k+9);

r=[r r1];

r1 = p(k+10)*128 + p(k+11)*64 + p(k+12)*32 + p(k+13)*16 + p(k+14)*8 + p(k+15)*4 + p(k+16)*2 +p(k+17);

r=[r r1];

end

matsize = size (r);

matrixsize = matsize(2);

errcounter1 = 0; %for the error in the seqid%

errcounter2 = 0; %for the error in the byte of zeros%

eof=hex2dec('C0');

newdata= 0;

eofdata= 0;

state = vstate(1);

j=1;

n = 1;

while (j<=matrixsize)

switch lower (state)

case 's'

eofdata=0;

sinter = [r(j)];

if sinter==seqid

state=vstate(2);

else

errcounter1 = errcounter1 + 1;

state=vstate(2);

end

j=j+1;

case 'h'

eofdata = 0;

hinter =[r(j)];

if hinter==hdiid

state=vstate(3);

newdata= 1;

else

errcounter1 = errcounter1 + 1;

state=vstate(3);

end

j=j+1;

case 'c'

cinter=[r(j)];

eofdata = 0;

if cinter >=128

chip=cinter;

chip_no = chip - 128;

gain_add = chip_no;

offset_add = chip_no + 32;

gain = go(gain_add)*16 + go(gain_add + 1);

offset = go(offset_add)* 16 + go(offset_add + 1);

if chip_no >= low_stereo & chip_no <= high_stereo

datatype = '01';

elseif chip_no >= low_axial & chip_no <= high_axial

datatype = '10';

elseif chip_no >= low_z & chip_no <= high_z

datatype = '11';

end

state=vstate(4);

else

state=vstate(1);

datata=cinter

end

j=j+1;

case 'z'

zinter =[r(j)];

if zinter==0

state=vstate(5);

else

errcounter2=errcounter2+1;

state=vstate(1);

end

j=j+1;

case 'a'

ainter =[r(j)];

eofdata = 0;

if  ainter<=127

channel=ainter;

state=vstate(6);

j=j+1;

else

if  ainter==eof

eofdata=1;

newb=dec2bin(newdata,1);

eofb=dec2bin(eofdata,1);

chipb=dec2bin(dchip,4);

chanb=dec2bin(channel,7);

j = j + 1;

event = r(j);

event_b = dec2bin(event,8);

output1 = [datatype newb eofb event_b chipb chanb];

output1_d = bin2dec(output1);

output1_b = dec2bin(output1_d,23);

output = [output output1_b];

state=vstate(1);

chan=ainter;

j=j+1;

else

state=vstate(3);

chann=ainter;

disp(chann);

pause

end

end

case 'v'

vinter =[r(j)];

v_data=vinter;

newb=dec2bin(newdata,1);

eofb=dec2bin(eofdata,1);

dchip=chip-128;

chipb=dec2bin(dchip,4);

chanb=dec2bin(channel,7);

add_bad_chan = (dchip -1)*128 + channel;

if m(add_bad_chan) == 0

data = v_data*gain + offset;

else

data = 0;

end

datab=dec2bin(data,8);

output1 = [datatype newb eofb datab chipb chanb];

output1_d = bin2dec(output1);

output1_b = dec2bin(output1_d,23);

output = [output output1_b];

state=vstate(5);

newdata = 0;

j=j+1;

otherwise

state=vstate(1);

end

end

fid = fopen('strip_data.m','w');

% the data is written out in 23-bit word format

datastrip = fwrite( fid,output);

st = fclose('all');

disp('Strip data written in the file');

D.5 Cluster finder

% VHDL design and FPLD implementation of the Silicon Track Card

% Shweta Lolage

% this the code to realise the Cluster finding algorithm

clear all

data = [];

chip_address = [];

chan_address = [];

address = [];

data_type = [];

new_data =[];

eof_data =[];

c_data_type = [];

rotate =['f' '1' '2' 'p' 't'];

d1 = 0; d2 = 0; d3 = 0; d4 = 0; d5 = 0;  % data buffers

add1 = 0; add2 = 0; add3 = 0; add4 = 0; add5 = 0; % address buffers

s_data1 = 0; s_data2 =0; % shadow data buffers

s_add1 = 0; s_add2 = 0; % shadow address buffers

out_data = [];

p_address = 0;

fid = fopen('down_data.m','r');

A1 = fread(fid);

o2 = hex2dec(A1);

st = fclose(fid);

fid= fopen('strip_data.m','r');

C=fread(fid);

m = hex2dec(C);

st = fclose(fid);

datasi=size(m);

datasize=datasi(1);

for k=1:23:datasize

data_type1=m(k)*2+ m(k+1);

data_type = [data_type data_type1];

new_data1 = m(k+2);

new_data = [new_data new_data1];

eof_data1 = m(k+3);

eof_data = [eof_data eof_data1];

data1 = m(k+4)*128 + m(k+5)*64 + m(k+6)*32 + m(k+7)*16 + m(k+8)*8 + m(k+9)*4 + m(k+10)*2 + m(k+11);

data =[data data1];

address1 = m(k+12)*1024 + m(k+13)*512 + m(k+14)*256 + m(k+15)*128 + m(k+16)*64 + m(k+17)*32 + m(k+18)*16 + m(k+19)*8 + m(k+20)*4 + m(k+21)*2 + m(k+22);

address=[address address1];

end

matsize = size (data);

matrixsize = matsize(2);

state = rotate(1);

j = 1;

current_data_type = 0;

while (j <= matrixsize)

add_threshold = (4*data_type(j)) + 13;

threshold1 = o2(add_threshold)*16 + o2(add_threshold + 1);

threshold2 = o2(add_threshold + 2)*16 + o2(add_threshold + 3);

neof_data = eof_data(j);

if data_type(j) == current_data_type

end_cluster_1 = 0;

else

end_cluster_1 = 1;

end

if address(j) == (p_address + 1)

end_cluster_2 = 0;

else

end_cluster_2 = 1;

end

if data(j) < threshold1

end_cluster_3 = 1;

else

end_cluster_3 = 0;

end

end_cluster_12 = or (end_cluster_1, end_cluster_2);

end_cluster_123 = or (end_cluster_12, end_cluster_3);

condition = or(neof_data,end_cluster_123);

switch lower (state)

case 'f'

current_data_type = data_type(j);

d1 = 0;

d2 = 0;

d3 = data(j);

d4 = 0;

d5 = 0;

add1 = 0;

add2 = 0;

add3 = address(j);

add4 = 0;

add5 = 0;

s_data1 = 0;

s_data2 = 0;

s_add1 = 0;

s_add2 = 0;

p_address = address(j);

j= j + 1;

state = rotate(2);

case '1'

current_data_type = data_type(j);

if condition == 1

state = rotate(5);

j = j -1 ;

else

if data(j) < d3

d4 = data(j);

add4 = address(j);

p_address = address(j);

j= j + 1;

state = rotate(3); % the next state is case '2' = rotate (3)

else

d1 = d2;

d2 = d3;

d3 = data(j);

add1 = add2;

add2 = add3;

add3 = address(j);

p_address = address(j);

j= j+ 1;

state = rotate(2);  % this state is case '1' = rotate (2)

end

end

case '2'

current_data_type = data_type(j);

if condition == 1

state = rotate (5);

j = j -1 ;

else

if data(j) < d3

d5= data(j);

add5 = address(j);

s_data1= d4;

s_data2 = data(j);

s_add1 = add4;

s_add2 = address(j);

p_address = address(j);

state = rotate (4); % next state is case 'p' = rotate (4)

j= j + 1;

else

d1 = d3;

d2 = d4;

d3 = data(j);

d4 = 0;

d5 = 0;

add1 = add3;

add2= add4;

add3 = address(j);

add4 = 0;

add5 = 0;

p_address = address(j);

j = j + 1;

state = rotate (2);

end

end

case 'p'

current_data_type = data_type(j);

if condition == 1

state = rotate (5);

j = j -1 ;

else

if data(j) > d3

d1 = s_data1;

d2 = s_data2;

d3 = data(j);

d4 = 0;

d5 = 0;

add1 = s_add1;

add2 = s_add2;

add3 = address(j);

add4 = 0;

add5 = 0;

p_address = address(j);

state = rotate(2);

j= j + 1;

else

s_data1 = s_data2;

s_data2 = data (j);

s_add1 = s_add2;

s_add2 = address(j);

p_address = address(j);

state = rotate(4);

j = j + 1;

end

end

case 't'

if  d3 > threshold2

d1_b = dec2bin(d1,8);

d2_b = dec2bin(d2,8);

d3_b = dec2bin(d3,8);

d4_b = dec2bin(d4,8);

d5_b = dec2bin(d5,8);

if add2 == 0

temp = add3 - 1;

add2_b = dec2bin (temp,11);

else

add2_b = dec2bin(add2,11);

end

address_passed_on =add2_b;

c_data_type_b = dec2bin(current_data_type,2);

out_data1 = [ c_data_type_b d1_b d2_b d3_b d4_b d5_b add2_b];

out_data = [out_data out_data1];

j = j + 1;

state = rotate(1);

else

state = rotate(1);

end

end

end

fid = fopen('cluster_data.m','w');

cluster = fwrite( fid,out_data);

st = fclose('all');

disp ('The clusters are found!!!!!');

D.6 Centroid Calculator

% VHDL design and FPLD implementation of the Silicon Track Card

% Shweta Lolage

% this the code to realise the Centroid Calculation

clear all

% declarations

d1 = []; d2 = [];d3 = []; d4 = []; d5 = [];

add2 = [];

data_type = [];

ctype = [];

out_to_hit = [];

num = 0; den = 0;

quo = 0.0; quo_i = 0; address = 0.0; centroid = 0.0;

fid = fopen('down_data.m','r');

A1 = fread(fid);

o=hex2dec(A1);

st = fclose(fid);

fid= fopen('cluster_data.m','r');

D=fread(fid);

m = hex2dec(D);

st = fclose(fid);

datasi=size(m);

datasize=datasi(1);

for k=1:53:datasize

data_type1 = m(k)*2 + m(k +1);

data_type = [data_type data_type1];

d_1= m(k+2)*128+ m(k+3)*64 +m(k+4)*32 + m(k+5)*16 + m(k+6)*8 + m(k+7)*4 + m(k+8)*2 +m(k+9);

d1 = [d1 d_1];

d_2= m(k+10)*128+ m(k+11)*64 +m(k+12)*32 + m(k+13)*16 + m(k+14)*8 + m(k+15)*4 + m(k+16)*2 +m(k+17);

d2 = [d2 d_2];

d_3= m(k+18)*128+ m(k+19)*64 +m(k+20)*32 + m(k+21)*16 + m(k+22)*8 + m(k+23)*4 + m(k+24)*2 +m(k+25);

d3 = [d3 d_3];

d_4= m(k+26)*128+ m(k+27)*64 +m(k+28)*32 + m(k+29)*16 + m(k+30)*8 + m(k+31)*4 + m(k+32)*2 +m(k+33);

d4 = [d4 d_4];

d_5= m(k+34)*128+ m(k+35)*64 +m(k+36)*32 + m(k+37)*16 + m(k+38)*8 + m(k+39)*4 + m(k+40)*2 +m(k+41);

d5 = [d5 d_5];

add_1 = m(k+42)*1024 + m(k+43)*512 + m(k+44)*256 + m(k+45)*128 + m(k+46)*64 + m(k+47)*32 + m(k+48)*16 + m(k+49)*8 + m(k+50)*4 + m(k+51)*2 + m(k+52);

add2=[add2 add_1];

end

clusters = size (d1);

clusterno = clusters(2);

cluster_type = o(65);

j= 1;

while ( j <= clusterno)

add_pulse_threshold = data_type(j)*9 + 29;

threshold1 = o(add_pulse_threshold)*256 + o(add_pulse_threshold +1)*16 + o(add_pulse_threshold+2);

threshold2 = o(add_pulse_threshold+ 3)*256 + o(add_pulse_threshold +4)*16 + o(add_pulse_threshold+5);

threshold3 = o(add_pulse_threshold+6)*256 + o(add_pulse_threshold +7)*16 + o(add_pulse_threshold+8);

if cluster_type ==0

num = d4(j)-d2(j);

den = d2(j) + d3(j) + d4(j);

else

num = d3(j) - d1(j) + (2*d4(j)) + (3*d5(j));

den = d1(j) + d2(j) + d3(j) + d4(j) + d5(j);

end

total_area = d1(j) + d2(j) + d3(j) + d4(j) + d5(j);

if (total_area < threshold1)

pulse_area_d = 0;

elseif (total_area >= threshold1 & total_area<= threshold2)

pulse_area_d = 1;

elseif (total_area >= threshold2 & total_area<= threshold3)

pulse_area_d = 2;

elseif (total_area >= threshold3)

pulse_area_d = 3;

end

quo = num / den;

quo_i = quo * 4;

address = add2(j)*4;

centroid = address + quo_i;

centroid_b = dec2bin(centroid,13);

pulse_area_b = dec2bin(pulse_area_d,2);

ctype = dec2bin(data_type(j),2);

out_to_hit1 = [ctype pulse_area_b centroid_b];

out_to_hit = [out_to_hit out_to_hit1];

j= j + 1;

end

fid = fopen('centriods_data.m','w');

centroids = fwrite( fid,out_to_hit);

st = fclose('all');

disp('The centroids are ready');

D.7 Hit Filter

% VHDL design and FPLD implementation of the Silicon Track Card

% Shweta Lolage

% this the code to realise the hit filter

clear all

centroid_type = []; centroid = []; c_precision_d = [];

pulse_area = [];

upper_road = []; lower_road = [];

hit_out = []; hits = [];

fid= fopen('centriods_data.m','r');

E=fread(fid);

n = hex2dec(E);

cen_size=size(n);

c_size=cen_size(1);

for k=1:17:c_size

centroid_type1 = n(k)*2 + n(k +1);

centroid_type = [centroid_type centroid_type1];

pulse_area1 = n(k+2)*2 + n(k+3);

pulse_area = [pulse_area pulse_area1];

centroid_1= n(k+4)*1024+ n(k+5)*512+ n(k+6)*256 +n(k+7)*128 + n(k+8)*64 + n(k+9)*32 + n(k+10)*16 + n(k+11)*8 +n(k+12)*4 +n(k+13)*2 +n(k+14);

centroid = [centroid centroid_1];

c_precision_d_1 = n(k+15)*2 +n(k+16);

c_precision_d = [c_precision_d c_precision_d_1];

end

centroids = size (centroid);

centroidno = centroids(2);

fid= fopen('roads_data.m','r');

F=fread(fid);

d = hex2dec(F);

road_size=size(d);

road_no =road_size(1);

for l=1:22:road_no

upper_road1= d(l)*1024 + d(l+1)*512 + d(l+2)*256 + d(l+3)*128 + d(l+4)*64 + d(l+5)*32 +d(l+6)*16 +d(l+7)*8 +d(l+8)*4 +d(l+9)*2 +d(l+10);

upper_road = [upper_road upper_road1];

lower_road1= d(l+11)*1024 +d(l+12)*512 + d(l+13)*256 + d(l+14)*128 + d(l+15)*64 + d(l+16)*32 +d(l+17)*16 +d(l+18)*8 +d(l+19)*4 +d(l+20)*2 +d(l+21);

lower_road = [lower_road lower_road1];

end

roads = size (upper_road);

roadno = roads(2);

x= 1;

while ( x <= centroidno)

y =1;

while (y <= roadno)

if upper_road(y) >= centroid(x)

c1 = 1;

else

c1 = 0;

end

if lower_road(y) <= centroid(x)

c2 = 1;

else

c2 = 0;

end

hit = and(c1,c2);

if hit == 1

road_b = dec2bin(y,6);

pulse_area_b = dec2bin(pulse_area(x),2);

centroid_b = dec2bin(centroid(x),11);

c_precision_b = dec2bin(c_precision_d(x),2);

hit_out1 = [road_b pulse_area_b centroid_b c_precision_b];

hit_out= [hit_out hit_out1];

end

y = y+1;

end

x= x+1;

end

fid = fopen('hits.m','w');

hitbits = fwrite( fid,hit_out);

st = fclose('all');

disp('The hits are done........');