#### .  APPENDIX – C:  VHDL CODE FOR THE MAIN DATA PATH AND DETAILED MEMORY MAP FOR THE DOWNLOADED PARAMETERS

# .1: MAIN MEMORY DECODER

-----------------------------------------------------------------------

--  This block decodes the main memory space

-----------------------------------------------------------------------

-- The initialization of the standard library files

-----------------------------------------------------------------------

--This revision 2B to modify the test lut data to 18 bits

--Shweta Lolage: 10/05/2000

-----------------------------------------------------------------------

library altera;

use altera.maxplus2.all;

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

-----------------------------------------------------------------------entity main_memory_decoder is

port (clock,reset: in std_logic;

main_address:in std_logic_vector(14 downto 0);

main_data_in: in std_logic_vector(31 downto 0);

main_select,main_read,main_write: in std_logic;

monitor_data_in,misc_parameters_data_in:in std_logic_vector(31 downto 0);

gainoffset_data_in: in std_logic_vector(7 downto 0);

roadlut_data_in : in std_logic_vector (21 downto 0);

testlut_data_in: in std_logic_vector(17 downto 0);

registered_data: out std_logic_vector(31 downto 0);

registered_address: out std_logic_vector(14 downto 0);

monitor_enable,misc_parameters_enable,

gainoffset_enable,testlut_enable,roadlut_enable : out std_logic;

global_write_not_read,global_dataoutput_enable: out std_logic;

main_data_out: out std_logic_vector(31 downto 0)

);

end entity main_memory_decoder;

--------------------------------------------------------------------

--Architecture body

architecture logic of main_memory_decoder  is

--

-- define memory map for address bits 14 downto 12

--

constant monitor_address_space:std_logic_vector(2 downto 0) := "0000";

constant misc_address_space: std_logic_vector(2 downto 0) := "0001";

constant gainoffset_address_space0: std_logic_vector(2 downto 0) := "0010";

constant gainoffset_address_space1: std_logic_vector(2 downto 0) := "0011";

constant gainoffset_address_space2: std_logic_vector(2 downto 0) := "0100";

constant gainoffset_address_space3: std_logic_vector(2 downto 0) := "0101";

constant test_address_space: std_logic_vector(2 downto 0) := "0110";

--- define constants for zero's to increase bit width of above memory space

constant zero8: std_logic_vector(7 downto 0) := "00000000";

constant zero10: std_logic_vector(9 downto 0) := "0000000000";

constant zero14: std_logic_vector(13 downto 0) := "00000000000000";

constant zero15: std_logic_vector(14 downto 0) := "000000000000000";

constant zero24: std_logic_vector(23 downto 0) := "000000000000000000000000";

constant zero23: std_logic_vector(22 downto 0) := "00000000000000000000000";

constant zero32 : std_logic_vector(31 downto 0) := "00000000000000000000000000000000";

--

--   Define constants to determine if this is a read or write mode

--

constant read_mode: std_logic := '0';

constant write_mode: std_logic := '1';

constant disabled: std_logic := '0';

constant enabled: std_logic := '1';

---  define signals to latch all external inputs

signal nmain_address,pmain_address: std_logic_vector(14 downto 0);

signal nmain_data_in,pmain_data_in: std_logic_vector(31 downto 0);

signal nmain_read,pmain_read,nmain_write,pmain_write,nmain_select,pmain_select: std_logic;

begin

--  This process block sets the enables for each address space if this smt channel has been selected

process(main_address,pmain_select)

variable address: std_logic_vector(3 downto 0);

variable vmonitor,vmisc,vgainoffset,vtestlut,vroadlut: std_logic;

variable vdata_out: std_logic_vector(31 downto 0);

variable memory_area_select : std_logic;

-- this signal is to differentiate between the memory for memories on the chip

-- and the road data memory outside the chips

begin

-- set default outputs

vmonitor := '0';

vmisc    := '0';

vgainoffset := '0';

vtestlut := '0';

vroadlut := '0';

vdata_out := zero32;

address := main_address(12 downto 10);

-- only need top 4 bits for address decoding

-- Dr. Perry I have made this change according to the memory map we discussed

memory_area_select := main_address(13);

--  if select line was high during last cycle then immediately enable appropriate address space

if(pmain_select = '1') then

if (memory_area_select = '0') then

case address is

when monitor_address_space =>

vmonitor := '1';

vdata_out := monitor_data_in;

when misc_address_space =>

vmisc := '1';

vdata_out := misc_parameters_data_in;

when gainoffset_address_space0|gainoffset_address_space1 =>

vgainoffset := '1';

vdata_out := zero24&gainoffset_data_in;

when gainoffset_address_space2|gainoffset_address_space3 =>

vgainoffset := '1';

vdata_out := zero24&gainoffset_data_in;

when test_address_space =>

vtestlut := '1';

vdata_out := zero14&testlut_data_in;

when others =>

null;

end case;

else

vroadlut := '1';

vdata_out := zero10&roadlut_data_in;

end if;

end if;

monitor_enable <= vmonitor;

misc_parameters_enable  <= vmisc;

gainoffset_enable    <= vgainoffset;

testlut_enable   <= vtestlut;

roadlut_enable  <= vroadlut;

main_data_out <= vdata_out;

end process;

---  This process block is used to prepare data internally for read or write

process(main_address,main_data_in,main_select,main_read,main_write,

pmain_select,pmain_write)

variable vaccess_mode,vgoe: std_logic;

begin

-- set default outputs

nmain_address <= main_address;

nmain_data_in <= main_data_in;

nmain_select <= main_select;

nmain_read <= main_read;

nmain_write <= main_write;

vaccess_mode := write_mode;  -- default access mode to write mode

vgoe := disabled;   -- default global oe to disabled

--   if not selected then we are in read mode

--

if(pmain_select = '0') then

vaccess_mode := read_mode;

vgoe := disabled;  ---   we are not selected turn off global output enable

elsif(pmain_write = '1') then

vaccess_mode := write_mode;

vgoe := disabled;

-- we are selected but writing, turn off global output enable

else

vaccess_mode := read_mode;

vgoe := enabled;

-- we are selected and reading, turn on global output enable

end if;

global_dataoutput_enable <= vgoe;

global_write_not_read <= vaccess_mode;

end process;

---

---  This is the register block

---

process(clock,reset,nmain_select,nmain_address,nmain_data_in,nmain_read,

nmain_write)

begin

if(reset = '0') then

pmain_select <= '0';

pmain_address <= zero15;

pmain_data_in <= zero32;

pmain_read <= '0';

pmain_write <= '0';

elsif(clock='1' and clock'event) then

pmain_select <= nmain_select;

pmain_address <= nmain_address;

pmain_data_in <= nmain_data_in;

pmain_read <= nmain_read;

pmain_write <= nmain_write;

end if;

end process;

registered_data <= pmain_data_in;

registered_address <= pmain_address;

end architecture logic;

## C.2: MISCELLANEOUS MEMORY DECODER

------------------------------------------------------------------------------

--  This block decodes the misc memory space

------------------------------------------------------------------------------

-- The initialization of the standard library files

------------------------------------------------------------------------------

library altera;

use altera.maxplus2.all;

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

-------------------------------------------------------------------------------

entity misc_memory_decoder is

port  (main_address:in std_logic_vector(10 downto 0);

bad_channel_data,misc_parameters_data:in std_logic_vector(31 downto 0);

chip_range_data,dedx_data: in std_logic_vector(23 downto 0);

cluster_data:in std_logic_vector(16 downto 0);     bad_channel_enable,chip_range_enable,dedx_enable,misc_parameters_enable,

cluster_enable: out std_logic;

misc_data_out: out std_logic_vector(31 downto 0)

);

end entity misc_memory_decoder;

--------------------------------------------------------------------

--Architecture body

architecture logic of misc_memory_decoder  is

--

-- define memory map for address bits 11 downto 8

--

constant bad_channel_address_space:std_logic_vector(2 downto 0) := "000";

constant chip_range_address_space: std_logic_vector(2 downto 0) := "001";

constant dedx_address_space: std_logic_vector(2 downto 0) := "010";

constant misc_parameters_address_space: std_logic_vector(2 downto 0) := "011";

constant cluster_address_space: std_logic_vector(2 downto 0) := "100";

constant zero8: std_logic_vector(7 downto 0):= "00000000";

constant zero15: std_logic_vector(14 downto 0):= "000000000000000";

constant zero32 : std_logic_vector(31 downto 0):= "00000000000000000000000000000000";

begin

--  This process block sets the enables for each address space

process(main_address)

variable address: std_logic_vector(2 downto 0);

variable vbc,vcr,vdedx,vmp,vce: std_logic;

begin

-- set default outputs

vbc := '0';

vcr := '0';

vdedx := '0';

vmp := '0';

vce := '0';

address := main_address(9 downto 7);

-- only need top 3 bits for address decoding

case address is

when bad_channel_address_space =>

vbc := '1';

when chip_range_address_space =>

vcr := '1';

when dedx_address_space =>

vdedx := '1';

when misc_parameters_address_space =>

vmp := '1';

when cluster_address_space =>

vce := '1';

when others =>

null;

end case;

bad_channel_enable <= vbc;

chip_range_enable   <= vcr;

dedx_enable     <= vdedx;

misc_parameters_enable   <= vmp;

cluster_enable    <= vce;

end process;

---  use top three bits of address to mux correct data word out

with main_address(10 downto 8) select

misc_data_out <=  bad_channel_data         when "000",

zero8&chip_range_data    when "001",

zero8&dedx_data          when "010",

misc_parameters_data     when "011",

zero15&cluster_data      when "100",

zero32                   when others;

end architecture logic;

## C.3: PULSE THRESHOLD CONTROL

------------------------------------------------------------------------------

--  This block decodes the memory addresses for the 8 de/dx parameters

------------------------------------------------------------------------------

-- Initial Design: Reginald Perry  (07/23/2000)

------------------------------------------------------------------------------

-- The initialization of the standard library files

------------------------------------------------------------------------------

library altera;

use altera.maxplus2.all;

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

-------------------------------------------------------------------------------

entity dedx_threshold_control is

port (control_address: in std_logic_vector(5 downto 0);

-- address from the control block

logic_address  : in std_logic_vector(1 downto 0);

-- for chiprange design the logic address is always 0

write_enable, memory_enable   : in std_logic;

-- read request signal from the control block

block0,block1,block2,block3: in std_logic_vector(23 downto 0);

-- this is the block input data

read_address : out std_logic_vector(5 downto 0);

-- the mux control or logic read_address to the FIFO

write_not_read0,write_not_read1,write_not_read2,

write_not_read3 : out std_logic;

block_out: out std_logic_vector(23 downto 0)

-- this is the block data out

);

end entity dedx_threshold_control;

--------------------------------------------------------------------

--Architecture body

architecture logic of  dedx_threshold_control is

signal control_memory_select: std_logic_vector(1 downto 0);

begin

-- let's use a with select to mux read address line

-- use a separate mux for each memory block

-- This is block 0 addresses 0-4

-- need two bits for address

with memory_enable select

--- use global_memory_enable to pick read_address

read_address  <= logic_address when '0',

control_address(1 downto 0) when '1',

logic_address when others;

-- Define control memory  select value

control_memory_select <= control_address(3)&control_address(2);

-- use a process block to determine write_not_read lines

process(memory_enable,write_enable,control_memory_select,block0,block1,block2,

block3)

variable block_data : std_logic_vector(23 downto 0);

variable  write_access_required: std_logic;

begin

--- use another variable to decide if write access is required

write_access_required :=  memory_enable and write_enable;

if(write_access_required = '1') then

--- need to determine which block wants access

case control_memory_select is

when "00" =>

write_not_read0 <='1';

write_not_read1 <='0';

write_not_read2 <='0';

write_not_read3 <='0';

when "01" =>

write_not_read0 <='0';

write_not_read1 <='1';

write_not_read2 <='0';

write_not_read3 <='0';

when "10" =>

write_not_read0 <='0';

write_not_read1 <='0';

write_not_read2 <='1';

write_not_read3 <='0';

when "11" =>

write_not_read0 <='0';

write_not_read1 <='0';

write_not_read2 <='0';

write_not_read3 <='1';

when others =>  -- should never come here

write_not_read0 <='0';

write_not_read1 <='0';

write_not_read2 <='0';

write_not_read3 <='0';

end case;

else  --  this a read cyle, set all write_not_read lines low;

write_not_read0 <='0';

write_not_read1 <='0';

write_not_read2 <='0';

write_not_read3 <='0';

end if;

--  use another case statement as a mux to set block_data out

case control_memory_select is

when "00" =>

block_data := block0;

when "01" =>

block_data := block1;

when "10" =>

block_data := block2;

when "11" =>

block_data := block3;

when others =>

block_data := block0;

end case;

block_out <= block_data;

end process;

end architecture logic;

## C.4: PULSE MEMORY CONCATENATION

------------------------------------------------------------------------------

--   This block concatinates dedx values into one 36 bit word

--   Include the possiblility for a three bit de/dx but don't do anything with those values now

------------------------------------------------------------------------------

-- Initial Design: Reginald Perry   (07/03/2000)

------------------------------------------------------------------------------

--

------------------------------------------------------------------------------

library altera;

use altera.maxplus2.all;

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

-------------------------------------------------------------------------------

entity dedx_memory_concatination is

port (block0,block1,block2,block3 : in std_logic_vector(23 downto 0);

dedx_threshold_coarse: out std_logic_vector(35 downto 0);

dedx_threshold_fine: out std_logic_vector(47 downto 0)

);

end entity dedx_memory_concatination;

--------------------------------------------------------------------

--Architecture body

--  memory map

--  dedx_threshold_coarse has coarse level approximations (2 bits)

--  dedx_threshold_fine gives one additional bit between coarse levels

--   Example: note high is (23 downto 12) while low is (11 downto 0)

--         coarse      fine

--   max   FFF

--                     block3_high

--         block1_low

--                     block3_low

--         block0_high

--                     block2_high

--         block0_low

--                     block2_low

--   min   000

architecture logic of dedx_memory_concatination  is

begin

dedx_threshold_coarse(35 downto 24) <= block1(11 downto 0);

dedx_threshold_coarse(23 downto 0) <= block0;

dedx_threshold_fine                <= block3&block2;

end architecture logic;

## C.5: CLUSTER THRESHOLD CONTROL

------------------------------------------------------------------------------

-- Version 0 This block is used to decide between the read address from the

-- control module or from the logic

------------------------------------------------------------------------------

-- Initial Design: Shweta Lolage   (07/03/2000)

------------------------------------------------------------------------------

--

-- Modified 7/20/2000   Simplify logic  RJP

-- The initialization of the standard library files

------------------------------------------------------------------------------

library altera;

use altera.maxplus2.all;

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

-------------------------------------------------------------------------------

entity cluster_threshold_control is

port (control_address: in unsigned(1 downto 0);

-- address from the control block

logic_address  : in unsigned(1 downto 0);

-- for chiprange design the logic address is always 0

write_enable, memory_enable   : in std_logic;

-- read request signal from the control block

read_address : out unsigned(1 downto 0);

-- the mux output read_address to the FIFO

write_not_read : out std_logic

);

end entity cluster_threshold_control;

--------------------------------------------------------------------

--Architecture body

architecture logic of  cluster_threshold_control is

begin

-- let's use a with select to mux read address line

-- we only have 3 types so set top three bits of address space equal to zero

with memory_enable select   --- use test_enable to pick read_address

read_address  <= logic_address when '0',

control_address when '1',

logic_address when others;

--   use simple data_path statement for write_not_read

---  write_not_read is write_enable and test_enable

write_not_read <= memory_enable and write_enable;

end architecture logic;

## C.6: CHIP RANGE CONTROL

------------------------------------------------------------------------------

-- Version 0 This block is used to decide between the read address from the

-- control module or from the logic

------------------------------------------------------------------------------

-- Initial Design: Shweta Lolage   (07/03/2000)

------------------------------------------------------------------------------

--

-- Modified 7/20/2000   Simplify logic  RJP

-- The initialization of the standard library files

------------------------------------------------------------------------------

library altera;

use altera.maxplus2.all;

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

-------------------------------------------------------------------------------

entity chiprange_control is

port (control_address: in unsigned(5 downto 0);

-- address from the control block

write_enable, memory_enable   : in std_logic;

-- read request signal from the control block

read_address : out unsigned(1 downto 0);

-- the mux output read_address to the FIFO

write_not_read : out std_logic

);

end entity chiprange_control;

--------------------------------------------------------------------

--Architecture body

architecture logic of chiprange_control  is

constant logic_address: unsigned(5 downto 0) := "000000";

begin

-- let's use a with select to mux read address line

with memory_enable select   --- use test_enable to pick read_address

read_address  <= logic_address(1 downto 0) when '0',

control_address(1 downto 0) when '1',

logic_address(1 downto 0) when others;

--   use simple data_path statement for write_not_read

---  write_not_read is write_enable and test_enable

write_not_read <= memory_enable and write_enable;

end architecture logic;

## C.7: MISCELLANEOUS DOWNLOADED PARAMETER SPLITTER

------------------------------------------------------------------------------

--   This block splits the 32 bit long misc downloaded parameter word into

--   individual blocks

------------------------------------------------------------------------------

-- Initial Design: Reginald Perry   (07/03/2000)

------------------------------------------------------------------------------

--

------------------------------------------------------------------------------

library altera;

use altera.maxplus2.all;

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

-------------------------------------------------------------------------------

entity misc_downloaded_parameter_splitter is

port (data_in : in std_logic_vector(31 downto 0);

dp_test_bit,dp_disable_bit: out std_logic;

dp_smt_id: out std_logic_vector(2 downto 0);

dp_delay_count,dp_seq_id: out std_logic_vector(7 downto 0);

dp_hdi_id: out std_logic_vector(2 downto 0)

);

end entity misc_downloaded_parameter_splitter;

--------------------------------------------------------------------

--Architecture body

architecture logic of misc_downloaded_parameter_splitter  is

begin

dp_test_bit        <= data_in(28);

dp_disable_bit     <= data_in(27);

dp_smt_id          <= data_in(26 downto 24);

dp_delay_count     <= data_in(23 downto 16);

dp_seq_id          <= data_in(15 downto 8);

dp_hdi_id          <= data_in(2 downto 0);

end architecture logic;

## C.8: TEST DATA CONTROL

------------------------------------------------------------------------------

-- Version 0 This block is used to decide between the read address from the

-- control module or from the logic

------------------------------------------------------------------------------

-- Initial Design: Shweta Lolage   (07/03/2000)

------------------------------------------------------------------------------

--

-- Modified 7/20/2000   Simplify logic  RJP

-- The initialization of the standard library files

------------------------------------------------------------------------------

library altera;

use altera.maxplus2.all;

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

-------------------------------------------------------------------------------

entity testdata_control is

port (control_address: in unsigned(7 downto 0);

-- address from the control block

logic_address  : in unsigned(7 downto 0);

-- address form the logic for the bad channels

write_enable, test_enable   : in std_logic;

-- read request signal from the control block

read_address : out unsigned(7 downto 0);

-- the mux output read_address to the FIFO

write_not_read : out std_logic

);

end entity testdata_control;

--------------------------------------------------------------------

--Architecture body

architecture logic of testdata_control  is

constant zero11 : unsigned (10 downto 0):= "00000000000";

begin

-- let's use a with select to mux read address line

with test_enable select   --- use test_enable to pick read_address

read_address  <= logic_address when '0',

control_address when '1',

logic_address when others;

--   use simple data_path statement for write_not_read

---  write_not_read is write_enable and test_enable

write_not_read <= test_enable and write_enable;

end architecture logic;

## C.9: BAD CHANNEL CONTROL

------------------------------------------------------------------------------

-- Version 0 This block is used to decide between the read address from the

-- control module or from the logic

------------------------------------------------------------------------------

-- Initial Design: Shweta Lolage   (07/03/2000)

------------------------------------------------------------------------------

-- Modified 7/20/2000   Simplify logic  RJP

-- The initialization of the standard library files

------------------------------------------------------------------------------

library altera;

use altera.maxplus2.all;

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

-------------------------------------------------------------------------------

entity badchannel_control is

port (control_address: in unsigned(10 downto 0);

-- address from the control block

logic_address  : in unsigned(10 downto 0);

-- address form the logic for the bad channels

write_enable, memory_enable   : in std_logic;

-- read request signal from the control block

read_address : out unsigned(10 downto 0);

-- the mux output read_address to the FIFO

write_not_read : out std_logic

);

end entity badchannel_control;

--------------------------------------------------------------------

--Architecture body

architecture logic of badchannel_control  is

begin

-- let's use a with select to mux read address line

with memory_enable select   --- use test_enable to pick read_address

read_address  <= logic_address when '0',

control_address when '1',

logic_address when others;

--   use simple data_path statement for write_not_read

---  write_not_read is write_enable and test_enable

write_not_read <= memory_enable and write_enable;

end architecture logic;

## C.10: BAD CHANNEL MULTIPLEXER

------------------------------------------------------------------------------

-- Version 1 This code decodes the data for the bad channels as the data

-- from the control is down loaded in set of 32 words, with each word representing

-- the status of 32 channels. This logic finds out the individual channel status.

------------------------------------------------------------------------------

-- Initial Design: Shweta Lolage   (6/27/2000)

-- Simplify Logic: RJP (7/21/2000)

------------------------------------------------------------------------------

-- Initializations to the standard library files

------------------------------------------------------------------------------

library altera;

use altera.maxplus2.all;

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

-------------------------------------------------------------------------------

entity bad_channel_mux is

port  (data_word      : in unsigned (31 downto 0);

address        : in integer range 31 downto 0;

bad_channel_status : out std_logic

);

end entity bad_channel_mux;

----------------------------------------------------------------------------------

--Architectural body of the design

architecture logic of bad_channel_mux is

begin

-- use with select to mux

with address select

bad_channel_status <= data_word(0) when 0,

data_word(1) when 1,

data_word(2) when 2,

data_word(3) when 3,

data_word(4) when 4,

data_word(5) when 5,

data_word(6) when 6,

data_word(7) when 7,

data_word(8) when 8,

data_word(9) when 9,

data_word(10) when 10,

data_word(11) when 11,

data_word(12) when 12,

data_word(13) when 13,

data_word(14) when 14,

data_word(15) when 15,

data_word(16) when 16,

data_word(17) when 17,

data_word(18) when 18,

data_word(19) when 19,

data_word(20) when 20,

data_word(21) when 21,

data_word(22) when 22,

data_word(23) when 23,

data_word(24) when 24,

data_word(25) when 25,

data_word(26) when 26,

data_word(27) when 27,

data_word(28) when 28,

data_word(29) when 29,

data_word(30) when 30,

data_word(31) when 31,

data_word(0) when others;

end architecture logic;

## C.11: GAIN OFFSET CONTROL

------------------------------------------------------------------------------

-- Version 0 This block is used to decide between the read address from the

-- control module or from the logic

------------------------------------------------------------------------------

-- Initial Design: Shweta Lolage   (07/03/2000)

------------------------------------------------------------------------------

--

-- Modified 7/20/2000   Simplify logic  RJP

-- The initialization of the standard library files

------------------------------------------------------------------------------

library altera;

use altera.maxplus2.all;

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

-------------------------------------------------------------------------------

entity gainoffset_control is

port (control_address: in unsigned(11 downto 0);

-- address from the control block

logic_address  : in unsigned(11 downto 0);

-- address form the logic for the bad channels

write_enable, memory_enable   : in std_logic;

-- read request signal from the control block

read_address : out unsigned(11 downto 0);

-- the mux output read_address to the FIFO

write_not_read : out std_logic

);

end entity gainoffset_control;

--------------------------------------------------------------------

--Architecture body

architecture logic of gainoffset_control  is

begin

-- let's use a with select to mux read address line

with memory_enable select   --- use test_enable to pick read_address

read_address  <= logic_address when '0',

control_address when '1',

logic_address when others;

--   use simple data_path statement for write_not_read

---  write_not_read is write_enable and test_enable

write_not_read <= memory_enable and write_enable;

end architecture logic;

## C.11: SMT DATA FILTER

-----------------------------------------------------------------------------

-- Revision 2.1 SMT Data Filter

-- Based on 5/26/2000 Document -- Earle, Specifications for One Channel of the STC Logic

-----------------------------------------------------------------------------

-- Initial Design: Reginald J. Perry   (6/4/2000)

-----------------------------------------------------------------------------

--  Modified 6/22/00 based on new specs:  Reginald J. Perry

--     Add 18 bit output (double byte wide)

--  Modified 7/14/00  RJP

--    Add delay counter for event number

-- Modified 8/02/00 Shweta Lolage

-- to change the format of the smt_out to fit the specifications

-- and minor changes to filter the excess C0's

-- Modified 10/05/00 Shweta Lolage

-- to remove the test inputs as they will be mux-ed with the FIFO output

-- to go directly to the Strip reader control

---------------------------------------------------------------------------

-- This block filters raw SMT data before it is stored in the input FIFO

-- Complex delay counter currently does not fit.  Implement simple delay counter for now

--

--

library altera;

use altera.maxplus2.all;

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

---------------------------------------------------------------------------

entity smt_data_filter is

port (delay_in: in unsigned(7 downto 0);

-- this is the delay in FRC start signal

frc_start: in std_logic;

-- this indicates that FRC data has arrived

disable: in std_logic;

--- this bit is used to turn off the channel

event_in: in unsigned(7 downto 0); -- this is the event number

reset,clock,cav,dav,lnkrdy,smt_error_in: in std_logic;

smt_in: in unsigned(7 downto 0);

end_of_event: out std_logic;  -- specifies the end of event

smt_out: out unsigned(17 downto 0);

wrreq: out std_logic

);

end entity smt_data_filter;

----------------------------Architecture ---------------------------

architecture logic_design of smt_data_filter is

---------------------Signal and constant Declaration--------------------------

signal delay_eq_zero,tdelay: std_logic;

signal nserr,pserr: std_logic;  --  this bit is used to record the serr

signal nsmt_out,psmt_out: unsigned(17 downto 0);

-- this register is used to hold smt data

signal ndelay,pdelay: unsigned(7 downto 0);

-- this is the internal delay counter

constant zero18: unsigned(17 downto 0) := "000000000000000000";

constant zero8: unsigned(7 downto 0) := "00000000";

constant zero_address: unsigned(7 downto 0) := "00000000";

constant zC0: unsigned(7 downto 0) := "11000000";

type mystates is (s0,s1,s2,s3,s4);

signal ns,ps,ndcs,pdcs: mystates;

begin

--  need two states to write data

process(ps, psmt_out,smt_error_in,cav,dav,lnkrdy,event_in,pdelay,pserr)

variable vsmt_enabled: std_logic;

variable vsmt_in,vupper,vlower: unsigned(7 downto 0);

begin

--   set default outputs

ns <= ps;

nserr <= pserr;

end_of_event <= '0';

nsmt_out <= psmt_out;

vupper := psmt_out(15 downto 8);   --  this is the last data byte

vsmt_in := smt_in;  -- use variable to hold the input stream data.

-- this is the real data

---  SMT data is valid if

--- dav = lnkrdy=0 and cav=1 and enable = 1 and disable = 0

---  use a simple logic expression for this.  Save in a variable

vsmt_enabled := (not dav) and (not lnkrdy) and cav  and (not disable);

--  if channel is active then access the state machine

if (vsmt_enabled= '1') then

case ps is

when s0 =>

--- this state is also used to initialize the delay counter.  See process delay_counter

end_of_event <= '0';

wrreq <= '0';

nsmt_out(17) <= smt_error_in;

nsmt_out(15 downto 8) <= vsmt_in;

-- this reads the upper byte of the word

--

--  we were told that you can't start up with C0s.

-- So no need to filter them here

--  go immediately to s1

--

ns <= s1;

when s1 =>

--   this is the lower byte

--   If upper byte is C0 then go to s2 to wait for delay counter to be zero to write event number

--   into trailer

end_of_event <= '0';

if(vupper = zC0) then

ns <= s2;

wrreq <= '0';

else

--    we have not seen trailer C0 in upper byte, use smt_in for lower byte.

wrreq <= '0';   --- we will enable output FIFO in next state

--- use this state to load register

vlower := vsmt_in;

end_of_event <= '0';   -- we are not at end of event

ns <= s3;

--  use s3 to write current word to FIFO and read next upper byte

nsmt_out(7 downto 0) <= vlower;

nsmt_out(16) <= smt_error_in;

end if;

--  this is the end of the current channel,

-- must wait for delay_eq_zero to assert before writing trailer

when s2 =>

end_of_event <= '0';

wrreq <= '0';

---  need to wait in here for delay to go to zero

nsmt_out <= '1'&pserr&zC0&event_in;

-- need to use dummy state to actually write FIFO.  See s3 below

if(delay_eq_zero = '1') then

-- write out the trailer if delay is zero

-- for the trailer, use msb for eof and use the next bit for serr

ns <= s4;   --  go back to s0 and start over

else

ns <= s2;

end if;

when s3 =>

-- use this state to write last word and read next upper byte

-- for non end of event

end_of_event <= '0';

--  set serr based on two error bits and current value of serr. ---- Use boolean equation

nserr <= psmt_out(17) or psmt_out(8) or pserr;

wrreq <= '1';

-- wrreq is not latched so it goes high immediately when we enter

-- s3.  Data word will be latched on the next clock edge. Timing

-- is a bit problematic here because new

-- dataword is being written at the same time.  Therefore, --

-- output FIFO must latch on

--  rising edge of clock when wrreq is enabled.

nsmt_out(17) <= smt_error_in;

nsmt_out(15 downto 8) <= vsmt_in;

-- this reads the next upper byte of the word

ns <= s1;  -- go to s1 to see if this is a C0

when s4 =>   -- this is the write for an end of event

end_of_event <= '1';

wrreq <= '1';

if( vsmt_in = zC0) then

-- if the next byte after the end of event is still C0 then wait in this state

-- till you get anything other than C0.

ns <= s4;

else

ns <= s0;  -- go to s0 to reset delay counter

end if;

when others =>   -- should never come here

ns <= s0;

end case;

else

--  if channel goes off or we are disabled, reset to s0

wrreq <= '0';

end_of_event <= '0';

ns <= s0;

end if;

end process;

--   This process block implements the simple "delay" counter.  Not exactly sure what it is used for RJP

delay_counter: process(pdelay, ps, delay_in,frc_start,pdcs)

variable temp_delay: std_logic;

variable i: integer;

begin

ndelay <= pdelay;

ndcs <= pdcs;

--  use a variable to determine if current count is zero

temp_delay := '0';

for i in 7 downto 0 loop

temp_delay := temp_delay or pdelay(i);

-- OR all bits to determine if we are at zero

end loop;

tdelay <= temp_delay;

-- this is the main state machine for delay counter

case pdcs is

when s0 =>

---   we are waiting to start the counter in this state

delay_eq_zero <= '0';

ndelay <= delay_in;

ndcs <= s0;  --  come back here if channel is inactive

if(ps=s1) then

ndcs <= s1;     --  smt data has started counter

end if;

if(frc_start = '1') then

ndcs <= s4;     --- frc_start has started counter

end if;

--  For simple delay counter, we will only have three states.

when s1 =>

delay_eq_zero <= '0';

if(tdelay = '0')  then

ndcs <= s3;   -- go to s3 to set delay_eq_zero = 1

else

ndelay <= pdelay - 1;    -- decrement ndelay

ndcs <= s1;  -- come back to this state

end if;

if(frc_start = '1') then   --- should be able to combine with above but synth tool choked on this

ndcs <= s3;

end if;

--  in this state we set delay_eq_zero = 1 and hold it until main FSM resets by -- going back to s0

when s3 =>

delay_eq_zero <= '1';

-- check if main FSM is in s0, if so reset this FSM to S0 otherwise stay here

if(ps = s0) then

ndcs <= s0;

else

ndcs <= s3;

end if;

--

-- use smt data to stop counter

--

when s4 =>

delay_eq_zero <= '0';

if(tdelay = '0')  then

ndcs <= s3;   -- go to s3 to set delay_eq_zero = 1

else

ndelay <= pdelay - 1;    -- decrement ndelay

ndcs <= s4;  -- come back to this state

end if;

if(ps=s1) then   --- should be able to combine with above but synth tool choked on this

ndcs <= s3;

end if;

when others =>   -- should never come here

ndcs <= s0;

delay_eq_zero <= '0';

end case;

end process;

--  register block

reg: process(ns,ndcs,nsmt_out,clock,reset,ndelay,nserr)

begin

if(reset = '0') then

ps <= s0;

pdcs <= s0;

psmt_out <= zero18;

pdelay <= zero8;

pserr <= '0';

elsif (clock'event and clock='1') then

ps <= ns;

pdcs <= ndcs;

psmt_out <= nsmt_out;

pdelay <= ndelay;

pserr <= nserr;

end if;

end process reg;

smt_out <= psmt_out; -- this goes to the output fifo

end architecture logic_design;   --End of architecture block

## C.12: SMT TEST SELECT

------------------------------------------------------------------------------

-- Version 0 This block is used to decide between the read address from the

-- control module or from the logic

------------------------------------------------------------------------------

-- Initial Design: Shweta Lolage   (10/05/2000)

------------------------------------------------------------------------------

-- The initialization of the standard library files

------------------------------------------------------------------------------

library altera;

use altera.maxplus2.all;

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

-------------------------------------------------------------------------------

entity smt_test_select is

port  (clock,reset : in std_logic;

test_data, fifo_data : in unsigned (17 downto 0);

l3_raw_data_eof : in std_logic;

-- this signal is taken from the strip reader control

event_start, test , read_data: in std_logic;

-- event start is the main signal which indicates start of an --

-- event for the channel

-- test input this indicates whether the strip reader is going to

-- process real data

-- or this event is a test run

-- read data is the signal from the strip reader

-- to read the next 18 bit word

fifo_empty : in std_logic; -- signal from the smt fifo;

logic_address  : out unsigned(7 downto 0); -- for test lut

strip_reader_data : out unsigned (17 downto 0);

read_fifo, memory_fifo_empty : out std_logic

-- read_req signal to the FIFO

-- fifo or memory empty signal for strip reader

);

end entity smt_test_select;

--------------------------------------------------------------------

--Architecture body

architecture logic of smt_test_select  is

type mystates is (stest,sstart,sfirst,saddress);

signal ps, ns : mystates;

signal naddress, paddress : unsigned (7 downto 0);

signal read_test : std_logic;

-- signal indicating to increment the address for next word

signal int_test : std_logic;

-- this signal is used to latch the test signa

-- I am not sure whether this signal

-- will remain high all the time test data is running

-- or will be a pulse indicating test mode

signal test_lut_empty : std_logic;

constant zero8 : unsigned(7 downto 0) := "00000000";

constant zero18 : unsigned (17 downto 0) := "000000000000000000";

constant max_count : unsigned (7 downto 0) := "11111111";

begin

main: process (ps,test,event_start,paddress)

begin

naddress <= paddress;

case ps is

when stest =>

int_test <= '0';

-- check for the test input to go to the next state

if (test = '1') then

int_test <= '1';

ns <= sstart;

else

int_test <= '0';

ns <= stest;

end if;

when sstart =>

-- initialize the adddress and the testlut empty signal

naddress <= zero8;

test_lut_empty <= '0';

if(event_start = '1') then

ns <= sfirst;

else

ns <= sstart;

end if;

when sfirst =>

-- this the word read out of the test lut

if(read_test = '1') then

naddress <= zero8; -- this just to make sure that the first word is read out

ns <= saddress;

else

ns <= sfirst;

end if;

when saddress =>

-- the machine will remain in this state till the reset signal comes in

if(read_test = '1') then

naddress <= paddress + 1;

else

naddress <= paddress;

end if;

if (l3_raw_data_eof = '0') then

ns <= saddress;

else

ns <= stest;

end if;

when others =>

ns <= stest;

end case;

end process main;

process(int_test,read_data)

variable vread_test, vread_fifo : std_logic;

begin

if (ps = sfirst or ps = saddress) then

vread_test := read_data;

vread_fifo := '0';

else

vread_test:= '0';

vread_fifo:= read_data;

end if;

read_test <= vread_test;

read_fifo <= vread_fifo;

end process;

reg: process (clock,reset,ps, paddress)

begin

if (reset = '0') then

ps <= stest;

paddress <= zero8;

elsif( clock'event and clock ='1') then

ps <= ns;

paddress <= naddress;

end if;

end process;

-- with select to mux read data

with int_test select   --- use test_enable to pick read_address

strip_reader_data  <= fifo_data when '0',

test_data when '1',

zero18 when others;

with int_test select   --- use test_enable to pick read_address

memory_fifo_empty  <= fifo_empty when '0',

test_lut_empty when '1', -- this just a dummy signal

'0' when others;

logic_address <= paddress;

end architecture logic;

## C.13: STRIP READER CONTROL

-----------------------------------------------------------------------------

-- Revision 2.1 Strip Reader Control

-- Based on 6-22-00 Document

-----------------------------------------------------------------------------

-- Modified by Shweta Lolage(04-10-2000)

-- Modified by Reginald Perry (6-4-2000)

--     Add comments and fix up notation

--     Add some monitoring counters

-- Modified by Reginald Perry (6-5-2000)

--     Add additional monitoring counters

-- Modified by Reginald Perry (6-22-2000)

--  Add 18 bit SMT input

-- Modified by Reginald Perry (7-12-2000)

--  Add monitor clear input

--  Add chip monitor counters

--- Modified by RJP (7/21/2000)

--  Need to add rawdata l3 output

-- Modified to add the initial state by Shweta Lolage (09-07-2000)

-----------------------------------------------------------------------------

library altera;

use altera.maxplus2.all;

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

library cluster_package;

use cluster_package.cluster_package.all;

-----------------------------------------------------------------------------

--    This block defines the inputs and outputs of the counter              -

-----------------------------------------------------------------------------

entity strip_reader_control is

port(reset,clock,input_fifo_empty,output_fifo_full,monitor_clear,

event_start : in  std_logic;

smt_in: in unsigned(2 downto 0);

bad_channel_data :in std_logic;

-- bad channel data is read from a rom

stream_in: in unsigned(17 downto 0);

-- MSB of stream_in is Error bit from SMT channel

seqid: in unsigned(7 downto 0);

hdiid: in unsigned(2 downto 0);

corrected_data:in unsigned (7 downto 0);

---  this is input data_type parameteric data in the format --- of axial, stereo, and ninety_degrees

--- (low followed by high)

data_range_in: in unsigned(23 downto 0);--  start of output data

oread,owrite:out std_logic;

--  signals used to enable input and output fifo

ostate :out unsigned(3 downto 0);

bad_channel_address:out unsigned(10 downto 0);

chip_data : out unsigned (11 downto 0);

output:out unsigned (22 downto 0);

--  monitoring data

l3_correcteddata: out unsigned(24 downto 0);

-- note l3_out(22) is the end of event bit

chip_monitor: out chip_array;

smt_monitor: out smt_array;

l3_rawdata: out unsigned(17 downto 0);

l3_rawdata_eof,l3_rawdata_enable: out std_logic

);

end entity strip_reader_control;

----------------------------Architecture Begining---------------------------

architecture logic_design of strip_reader_control is

---------------------Signal and constant Declaration--------------------------

signal neof,peof,nmismatchbit,pmismatchbit,pbc,nbc,nserr,pserr: std_logic;

signal pnew_event,nnew_event: std_logic;

signal n_new_data_flag,p_new_data_flag: std_logic;

signal nseq_id,pseq_id: unsigned(7 downto 0);

signal nhdi_id,phdi_id: unsigned(2 downto 0);

signal noutput,poutput    : unsigned (22 downto 0);

signal nchip_id,pchip_id  : unsigned (3 downto 0);

signal nchan_id,pchan_id  : unsigned (6 downto 0) ;

signal ndata_type, pdata_type: unsigned(1 downto 0);

signal nstate, pstate   : unsigned(3 downto 0);

signal nbad_channel_address, pbad_channel_address: unsigned(10 downto 0);

signal nchip_data,pchip_data : unsigned (11 downto 0);

-----

---  These signals are for monitoring information

---

signal nsmt_mismatch_count,psmt_mismatch_count,nsmt_error_count,

psmt_error_count: unsigned(23 downto 0);

signal nsmt_zero_error,psmt_zero_error,nl3_trailer,

pl3_trailer: unsigned(23 downto 0);

--  These signals are used in the clear operation for the monitor counters.

signal nchip,pchip: chip_array;  -- chip array is defined in cluster_package

constant zero9  : unsigned (8 downto 0)  := "000000000";

constant zero8  : unsigned (7 downto 0)  := "00000000";

constant zero7  : unsigned (6 downto 0)  := "0000000";

constant zero4  : unsigned (3 downto 0)  := "0000";

constant zero3  : unsigned (2 downto 0)  := "000";

constant C0     : unsigned (7 downto 0)  := "11000000";

constant zero2  : unsigned (1 downto 0)  := "00";

constant initial_state 	  	 : unsigned (3 downto 0):= "0000";

constant first_read_state 	 : unsigned (3 downto 0):= "0001";

constant seq_hdi_state   	 : unsigned (3 downto 0):= "0010";

constant chip_zero_state  	 : unsigned (3 downto 0):= "0011";-- State constants

constant chan_value_state 	 : unsigned (3 downto 0):= "0100";

constant wait_for_data_state : unsigned (3 downto 0):= "0101";

constant correctvalue_state  : unsigned (3 downto 0):= "0110";

constant write_fifo_state    : unsigned (3 downto 0):= "0111";

constant read_fifo_state     : unsigned (3 downto 0):= "1000";

constant read_wait_state 	 : unsigned (3 downto 0):= "1001";

constant zero23 : unsigned (22 downto 0) := "00000000000000000000000";

constant zero24 : unsigned (23 downto 0) := "000000000000000000000000";

constant high   : std_logic := '1';

constant low    : std_logic := '0';

-- constant

constant chip0 : unsigned(3 downto 0) := "0000";

constant chip1 : unsigned(3 downto 0) := "0001";

constant chip2 : unsigned(3 downto 0) := "0010";

constant chip3 : unsigned(3 downto 0) := "0011";

constant chip4 : unsigned(3 downto 0) := "0100";

constant chip5 : unsigned(3 downto 0) := "0101";

constant chip6 : unsigned(3 downto 0) := "0110";

constant chip7 : unsigned(3 downto 0) := "0111";

constant chip8 : unsigned(3 downto 0) := "1000";

constant chip9 : unsigned(3 downto 0) := "1001";

--  valid data types are now defined in cluster package

--constant undefined_type: unsigned(1 downto 0) := "00";

--constant stereo_type : unsigned(1 downto 0)  := "01";

--constant axial_type: unsigned(1 downto 0) := "10";

--constant ninety_degrees_type: unsigned(1 downto 0) := "11";

-- to define the states of the decoder --

type mystates is (sinit,sfirst_word,seq_hdi,chip_zero,chan_value,wait_for_data,fill_output_fifo,

write_fifo,read_fifo,read_wait);

signal ns,ps,nrs,prs : mystates;

type event_states is (sinit,snew_event,swait1,swait2,sevent);

signal nevent_state,pevent_state: event_states;

-----------------------------Block Begins-----------------

begin

process(pstate,ps,prs,poutput,pchip_id,pchan_id,p_new_data_flag,pdata_type,

corrected_data,peof,pseq_id,phdi_id,bad_channel_data,seqid,hdiid,

input_fifo_empty,psmt_mismatch_count,psmt_zero_error,pmismatchbit,

output_fifo_full,pchip,monitor_clear,pbc,pserr,pnew_event)

---  Variable declaration ---

---  Use variables to much of the state information.  Need to include a definition table (RJP 6/4/00)

variable voutput : unsigned ( 22 downto 0);

variable vnew_data_flag,vend_of_event: std_logic;

variable verror_high,verror_low: unsigned(0 downto 0);

variable vchip_id : unsigned(3 downto 0);

variable vchan_id: unsigned(6 downto 0);

variable vstream_high_byte,vstream_low_byte,vseq_id: unsigned(7 downto 0);

variable vhdi_id: unsigned(2 downto 0);

variable vdata_type:unsigned(1 downto 0);

variable next_state,return_state: mystates;

variable vstate : unsigned(3 downto 0);

variable veof,vbc,vserr: std_logic;

begin

--

--  Default Outputs

--

--

--  Default variable values

--

vstate := pstate;

vnew_data_flag := p_new_data_flag;

next_state := ps;

voutput := poutput;

veof := peof;

vdata_type := pdata_type;

vchip_id := pchip_id;

vchan_id := pchan_id;

vseq_id := pseq_id;

vhdi_id := phdi_id;

vbc := pbc;

vserr := pserr;

verror_high(0) := stream_in(17);

verror_low(0)  := stream_in(16);

vstream_high_byte := stream_in(15 downto 8); -- MSB of stream data

vstream_low_byte := stream_in(7 downto 0);  -- LSB of stream data

return_state := prs;

nchip_data <= pchip_data;

---  smt counters

nsmt_mismatch_count <= psmt_mismatch_count;

nsmt_zero_error <= psmt_zero_error;

nmismatchbit <= pmismatchbit;

nbad_channel_address <= pchip_id&pchan_id;

--  this is used to address bad channel prom

--

--  Chip counters -- default outputs

--

nchip <= pchip;

--

--  Check for serr

--

vserr := verror_high(0) or verror_low(0) or pserr;

--  set l3_rawdata to defaults.

l3_rawdata <= stream_in;

l3_rawdata_eof <= '0';

l3_rawdata_enable <= '0';

---------------------

----CASE STATEMENT --

---------------------

-- The following case statment will look at the incoming stream of data and --will proceed to

-- compare and check for the validity of the same.  For each case different actions will occur.

-- Note1: Every state requires to have a read fifo before it can proceed to check the information.

-- Note2: After the channel state and 11-bit word composed of the chip and channel id are sent to

-- to an lpm rom, which will send an 9 bit word back into the decoder.  From this 9 bit word we

-- will look at the MSB and if it is "0" then we know that the channel is ok.

-- Note3: Only the channel and Value state will have the command to enable writing to the FIFO.

CASE ps IS

-- the initial state in which the FSM is waiting for the event start

When sinit =>

vstate := initial_state;

if (event_start = '1') then

next_state := sfirst_word;

else

next_state := sinit;

end if;

-- the initial state in which the first word is read outof the fifo

when sfirst_word =>

vstate := first_read_state;

if (pevent_state = sevent) then

next_state := read_fifo;

return_state := seq_hdi;

else

next_state := sfirst_word;

end if;

-- Sequence State

-- Check for correct sequence ID and hdi_id  --

WHEN seq_hdi =>

vstate := seq_hdi_state;

-- variable used to check current state this is for debugging only

next_state := read_fifo;

return_state := chip_zero;

vnew_data_flag := high;

-- set new data high when looking for seqid and hdiid

vseq_id := vstream_high_byte;    --  save the sequence id

vhdi_id := vstream_low_byte(2 downto 0);

--- save the hdi id.  It is bits 2,1,0

nmismatchbit <= low;   -- reset mismatch bit

if((vstream_high_byte /= seqid) or (vstream_low_byte /= hdiid)) then

--when stream does not equal the preset value

--  need to count the number of errors in seq_Id mismatch

nsmt_mismatch_count <= psmt_mismatch_count + 1;

nmismatchbit <= high;   -- set the mismatch bit high

end if;

--  need to enable l3_rawdata so that l3 will latch when we change states

l3_rawdata_eof <= '0';

l3_rawdata_enable <= '1';

-- CHIP State

-- Check for correct chip ID and zero_byte

WHEN chip_zero =>

vstate := chip_zero_state;

next_state := read_fifo;

if(vstream_low_byte /= zero7) then

-- check that low byte is zero

nsmt_zero_error <= psmt_zero_error + 1;

end if;

if (vstream_high_byte(7) = high) then

---  if MSB = 1 this is a new chip.  Save chip ID

vchip_id    := vstream_high_byte(3 downto 0);

--  and return to channel value

return_state := chan_value;

else

return_state := chip_zero;

---  If MSB is not 1 then error come back to this state

end if;                     ---  and look for valid chip id

--  need to save l3 raw

l3_rawdata_eof <= '0';

l3_rawdata_enable <= '1';

-- CHANNEL State --

-- Get Channel ID and Value from Stream

WHEN chan_value =>            -- The channel ID is read from stream

vstate := chan_value_state; --

next_state := read_fifo;

return_state := chan_value;

veof := '0';

if (vstream_high_byte = C0) then

-- if channel is C0 then we are at the end of event

veof     := high;

-- event number is in low_byte so save it in voutput

voutput(18 downto 11) :=  vstream_low_byte;

next_state := fill_output_fifo;

--  jump to fill_output_fifo

return_state := sinit;

-- return to next sequence ID after write fifo

l3_rawdata_enable <= '1';

l3_rawdata_eof <= '1';      -- set end of event bit

elsif (vstream_high_byte(7) = '1') then

-- This means this is a new chip ID

next_state := chip_zero;

--  Check for a new type of chip. Do NOT read fifo.

l3_rawdata_enable <= '0';      --  do not write into l3

l3_rawdata_eof <= '0';

else

next_state := wait_for_data;

--  this is a regular channel/data pair

vchan_id := vstream_high_byte(6 downto 0);

-- need to goto a wait state to give LUT enough

nchip_data <= pchip_id&vstream_low_byte;

-- time to access corrected data

l3_rawdata_eof <= '0';

l3_rawdata_enable <= '1';   -- enable l3 rawdata

end if;                         -- come back here after write

-- wait for data State --

-- This is a dummy state to wait for the corrected data to come from the LUT.

WHEN wait_for_data =>

vstate := wait_for_data_state;

next_state := fill_output_fifo;

-- go to fill output fifo state

---

---   Data is valid, Increment chip monitor counters here

--

--  Chip counters -- default outputs

--

nchip <= pchip;

case vchip_id is

when chip0 =>

nchip(0) <= add_one(pchip(0));

when chip1 =>

nchip(1) <= add_one(pchip(1));

when chip2 =>

nchip(2) <= add_one(pchip(2));

when chip3 =>

nchip(3) <= add_one(pchip(3));

when chip4 =>

nchip(4) <= add_one(pchip(4));

when chip5 =>

nchip(5) <= add_one(pchip(5));

when chip6 =>

nchip(6) <= add_one(pchip(6));

when chip7 =>

nchip(7) <= add_one(pchip(7));

when chip8 =>

nchip(8) <= add_one(pchip(8));

when others =>

nchip <= pchip;   --- should never come here;

end case;

--  DO NOT enable l3 raw here

l3_rawdata_eof <= '0';

l3_rawdata_enable <= '0';

WHEN fill_output_fifo =>

vstate := correctvalue_state;

--  This is a bit backward but if veof leave voutput(18 downto 11) only.  This was set in chan_value state

--  if not end of event that check for a bad channel and replace with zero if true

--  otherwise use corrected data in output.  Hopefully, the synthesis tool can clean this up

if(peof = '1')  then

voutput(18 downto 11) := voutput(18 downto 11);

-- if at end of event, leave voutput only but reset vserr;

elsif(bad_channel_data = '1') then

-- bad channel LUT is access using pchip_id and pchannel_id

voutput(18 downto 11)   :=  zero8;

--  this channel is bad, replace data with zero

else

voutput(18 downto 11)  := corrected_data;

-- not eof and channel good, replace with

end if;                                    -- corrected data

voutput(22 downto 21)   :=  pdata_type;

voutput(20)             :=  p_new_data_flag;

voutput(19)             :=  peof;

voutput(10 downto 7)    :=  pchip_id;

voutput(6 downto 0)     :=  pchan_id;

vnew_data_flag   := low;

--

-- save bad channel data for l3

--

vbc := bad_channel_data;

next_state := write_fifo;

--  DO NOT enable l3 raw here

l3_rawdata_eof <= '0';

l3_rawdata_enable <= '0';

-- write_fifo State --

WHEN write_fifo =>

vstate := write_fifo_state;   --- this is for debugging

vnew_data_flag := low;

-- reset new data flag after first write.

if(output_fifo_full = '1') then

next_state := write_fifo;

else

next_state := read_fifo;

-- fifo is free to write into. Next state should be calling state

if(peof = '1') then     --  if end of event, reset vserr

vserr := low;     --

end if;

end if;

--  bring l3_rawdata low in here

l3_rawdata_eof <= '0';

l3_rawdata_enable <= '0';

--read_fifo state

when read_fifo =>

vstate := read_fifo_state;

if(input_fifo_empty = '1') then

next_state := read_fifo;  --  fifo is empty.  need to wait

else

next_state := return_state;

end if;

--  bring l3_rawdata low in here

l3_rawdata_eof <= '0';

l3_rawdata_enable <= '0';

-- OTHERS --   -- should never come here

WHEN OTHERS =>

next_state := sinit;

return_state := sinit;

l3_rawdata_eof <= '0';

l3_rawdata_enable <= '0';

END CASE;

--  if monitor_clear, need to reset monitor counters)

if(monitor_clear = '1') then

for i in 8 downto 0 loop

nchip(i) <= zero_monitor_word;

end loop;

nsmt_mismatch_count <= zero24;

nsmt_zero_error <= zero24;

end if;

--    set the next state of signal to their respective variable signals

nstate <= vstate;

n_new_data_flag <= vnew_data_flag;

nrs <= return_state;

ns <= next_state;

noutput <= voutput;

neof <= veof;

nchip_id <= vchip_id;

nchan_id <= vchan_id;

nseq_id <= vseq_id;

nhdi_id <= vhdi_id;

nbc <= vbc;

nserr <= vserr;

end process;

-- The following two process block work concurrently with the above process block.  One

-- block reads raw data from the input FIFO process block and the other write corrected data

-- to the write FIFO process block

--  Read fifo block

--  This process block reads raw data from the input fifo

process(ps,input_fifo_empty)

--  This process is used to read the input fifo

variable vread: std_logic;

begin

vread := '0';

if(input_fifo_empty = '1') then

--  Buffer is empty, therefore no read signal

vread := '0';

else

--Write and read fifo process

case ps is

when read_fifo =>  -- For any read_fifo state set read flag high

vread := '1';

when others =>

vread := '0';

end case;

end if;

oread <= vread;  	-- output read signal is set to read variable

end process;

process(ps)        --  This process is used to write to the output fifo

variable vwrite: std_logic;

begin

vwrite := '0';

if(output_fifo_full = '1') then      --  If buffer is full, we must wait

vwrite := '0';

else

case ps is         --when we have a write enable signal, we will proceed

when write_fifo=> -- to write to the ouput fifo the 23-bit word

vwrite := '1';

when others =>

vwrite := '0';

end case;

end if;

owrite <= vwrite;

end process;

---------------------------------------------------------------------------------

---   Use this process block to determine the type of data

---   Input parameters must come from another block

---------------------------------------------------------------------------------

process (data_range_in,pchip_id)

--  use variables to break up data_range data

variable axial_low,axial_high,stereo_low,stereo_high,ninety_low,ninety_high: unsigned(3 downto 0);

begin

axial_low := data_range_in(3 downto 0);

axial_high := data_range_in(7 downto 4);

stereo_low := data_range_in(11 downto 8);

stereo_high := data_range_in(15 downto 12);

ninety_low := data_range_in(19 downto 16);

ninety_high := data_range_in(23 downto 20);

ndata_type <= undefined_type;

if(pchip_id >= axial_low and pchip_id <= axial_high) then

ndata_type <= axial_type;

end if;

if(pchip_id >= stereo_low and pchip_id <= stereo_high) then

ndata_type <= stereo_type;

end if;

if(pchip_id >= ninety_low and pchip_id <= ninety_high) then

ndata_type <= ninety_degrees_type;

end if;

end process;

---------------------------------------------------------------------------------

--- This block monitors the number of times ERROR is high in the smt data

---------------------------------------------------------------------------------

process(stream_in,monitor_clear)

begin

nsmt_error_count <= psmt_error_count;

if(monitor_clear = '1') then

nsmt_error_count <= zero24;

elsif((stream_in(16) = high) or (stream_in(17) = high)) then

nsmt_error_count <= psmt_error_count + 1;

end if;

end process;

---

--- This process block is used to start a new event

---

process(pevent_state,peof,event_start)

begin

nnew_event <= pnew_event;

case pevent_state is

when sinit =>

nnew_event <= '0';

if(event_start = '1') then

nevent_state <= snew_event;

else

nevent_state <= sinit;

end if;

when snew_event =>   -- this is a new event

nnew_event <= '0';

-- enter three wait states before reading first word

nevent_state <= swait1;  -- this gives L3 time to write headers

when swait1 =>

nnew_event <= '0';

nevent_state <= swait2;

when swait2 =>

nnew_event <= '0';

nevent_state <= sevent;

when sevent =>

nnew_event <= '1';

if(peof = '1') then

nevent_state <= sinit;

else

nevent_state <= sevent;

end if;

when others =>    ---  never should come here

null;

end case;

end process;

---   Use this process block to clear monitor signals

------------------------------------------------------------------------------

------------          REGISTER BLOCK      ---------------------

---------------------------------------------------------------------------------

reg:process(clock,reset, ns, nrs, noutput,nchip_id,nchan_id,n_new_data_flag,

ndata_type,neof,nbad_channel_address,nstate,nchip_data,nmismatchbit,

nsmt_error_count,nsmt_mismatch_count,nsmt_zero_error,nbc,nnew_event)

begin

if (reset = '0') then  --Sets all outputs and counters to zero

pstate	       <= initial_state;

ps             <= sinit;

prs            <= sinit;

poutput        <= zero23;

pchip_id       <= zero4;

pchan_id       <= zero7;

p_new_data_flag<= '0';

pseq_id        <= zero8;

phdi_id        <= zero3;

pdata_type     <= zero2;

peof           <= '0';

pbc            <= '0';

pserr          <= '0';

pbad_channel_address   <= zero4&zero7;

pchip_data      <="000000000000";

psmt_error_count <= zero24;

psmt_mismatch_count <= zero24;

psmt_zero_error <= zero24;

pl3_trailer <= zero24;

pmismatchbit <= '0';

pnew_event <= '0';

pevent_state <= sinit;

elsif (clock'event and clock = '1') then

pstate 		<= nstate;

ps             <= ns;

--at every clock edge there will be an output

prs            <= nrs;

poutput        <= noutput;

pchip_id       <= nchip_id;

pchan_id       <= nchan_id;

p_new_data_flag<= n_new_data_flag;

pseq_id        <= nseq_id;

phdi_id        <= nhdi_id;

pdata_type     <= ndata_type;

peof           <= neof;

pbc            <= nbc;

pserr          <= nserr;

pbad_channel_address   <= nbad_channel_address;

pchip_data     <= nchip_data;

psmt_error_count <= nsmt_error_count;

psmt_mismatch_count <= nsmt_mismatch_count;

psmt_zero_error <= nsmt_zero_error;

pmismatchbit <= nmismatchbit;

pl3_trailer <= nl3_trailer;

pnew_event <= nnew_event;

pevent_state <= nevent_state;

end if;

end process reg;                --End of register block

--

-- register block for chip counters

--

process(clock,reset,nchip)

begin

if(reset = '0') then

for i in 8 downto 0 loop

pchip(i) <= zero_monitor_word;

end loop;

elsif(clock'event and clock='1') then

pchip <= nchip;

end if;

end process;

--

-- This process block is used to form l3 word

--

process(peof,pmismatchbit,pdata_type,poutput,pseq_id,phdi_id,pbc,smt_in,pserr,pl3_trailer)

variable vnew_data_flag,vend_of_event: std_logic;

variable vchip_id : unsigned(3 downto 0);

variable vchan_id: unsigned(6 downto 0);

variable vdata_event, vseq_id: unsigned(7 downto 0);

variable vhdi_id: unsigned(2 downto 0);

variable vdata_type:unsigned(1 downto 0);

variable veof,vbc,vmismatch,vserr: std_logic;

variable vsmt_in: unsigned(2 downto 0);

variable vl3_out: unsigned(24 downto 0);

begin

--

--  Use variables to help form data word

--

vdata_type := poutput(22 downto 21);

vnew_data_flag :=  poutput(20);

veof := poutput(19);

vdata_event := poutput(18 downto 11);

vchip_id := poutput(10 downto 7);

vchan_id := poutput(6 downto 0);

veof := peof;

vmismatch := pmismatchbit;

vseq_id := pseq_id;

vhdi_id := phdi_id;

vsmt_in := smt_in;

vserr := pserr;

vbc := pbc;

nl3_trailer <= vserr&vmismatch&vsmt_in&vseq_id&vhdi_id&vdata_event;

--  if end of event we must output trailer l3 word  --- event number should be in trailer

if(veof = '1') then

--                  1  +  1  + 1       + 3  +  8 + 3  + 8  = 25

vl3_out := veof&vserr&vmismatch&vsmt_in&vseq_id&vhdi_id&vdata_event;

--                       1  +    1    +  3

else

--                   1+ 1 + 2         + 8   + 4 +      7      + 2 = 25

vl3_out := veof&vbc&vdata_type&vdata_event&vchip_id&vchan_id&"00";

--        nl3_trailer <= pl3_trailer;

end if;

l3_correcteddata <= vl3_out;

end process;

ostate <= pstate;           -- Ensures that we keep the outputs constant at every cycle

output <= poutput;		  -- until a condition is met and we update the values.

bad_channel_address <= pbad_channel_address;

chip_data <= pchip_data;

chip_monitor <= pchip;

smt_monitor(0) <= to_monitorword(pl3_trailer);

smt_monitor(1) <= to_monitorword(psmt_error_count);

smt_monitor(2) <= to_monitorword(psmt_mismatch_count);

smt_monitor(3) <= to_monitorword(psmt_zero_error);

end architecture logic_design;   --End of architecture block

## C.14: MONITOR ADDRESS SPACE

--

--  This is the monitor address space

--  It is 32x24

--

library altera;

use altera.maxplus2.all;

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

library cluster_package;

use cluster_package.cluster_package.all;

--  This package contains the following definitions

--  smt_array is 4 x 24

--  chip_array is  9 x 24

--  centroid_array is 3 x 24

entity monitor_spacea is

port(address: in unsigned(9 downto 0);

monitor_latch: in std_logic;

smt_counter: in smt_array;

chipcounter: chip_array;

axial:in centroid_array;

clk,reset: in std_logic;

l3_trailer: out fifoword;

word_out: out fifoword;

monitor_clear: out std_logic

);

end entity monitor_spacea;

architecture behavior of monitor_spacea is

--

--  Define the following general memory map for the 1k monitor space

--

--  smt counters                     000-00F

--  centroid counters                010-01F

--  chip counters                    020-02F

--  empty space                      030-3FF

constant smt_address_space :unsigned(5 downto 0) := "000000";

constant centroid_address_space :unsigned(5 downto 0) := "000001";

constant chip_address_space :unsigned(5 downto 0) := "000010";

signal nsmt_registers,psmt_registers: smt_array;

signal nchip_registers,pchip_registers: chip_array;

signal ncentroid_registers,pcentroid_registers: centroid_array;

constant true: std_logic := '1';

constant false: std_logic := '0';

signal nmonitor_clear,pmonitor_clear: std_logic;

type smt_states is (s0,s1,s2);

signal psmt,nsmt: smt_states;

signal nword_out, pword_out: fifoword;

--signal nbyte_out, pbyte_out: fifobyte;

begin

--

-- This process block attaches the monitor counters to the register array

-- It is enabled on the rising edge of monitor_latch

--

process(psmt,smt_counter,chipcounter,axial,monitor_latch)

begin

--

--  Default outputs

--

--   smt_registers

--

for i in 3 downto 0 loop

nsmt_registers(i) <= psmt_registers(i);

end loop;

--

--  chip registers

--

for i in 8 downto 0 loop

nchip_registers(i) <= pchip_registers(i);

end loop;

--

-- centroid registers

--

for i in 2 downto 0 loop

ncentroid_registers(i) <= pcentroid_registers(i);

end loop;

--

--  look for the rising edge of monitor_latch

--

case psmt is

when s0 =>

nsmt <= s0;

nmonitor_clear <= '0';

if(monitor_latch = '1') then

nsmt <= s1;

end if;

when s1 =>

nsmt <= s2;

nmonitor_clear <= '1';

for i in 3 downto 0 loop

nsmt_registers(i) <= smt_counter(i);

end loop;

for i in 8 downto 0 loop

nchip_registers(i) <= chipcounter(i);

end loop;

for i in 2 downto 0 loop

ncentroid_registers(i) <= axial(i);

end loop;

when s2 =>

nmonitor_clear <= '1';

if(monitor_latch = '1') then

nsmt <= s2;

else

nsmt <= s0;

end if;

when others =>

nsmt <= s0;

nmonitor_clear <= '0';

end case;

end process;

--

--    This process block sets the data output to the addressed monitor space

--

process(address,psmt_registers,pchip_registers,pcentroid_registers)

variable vaddress_high: unsigned(5 downto 0);

variable vaddress_low: unsigned(3 downto 0);

variable vword_out: monitor_word;

begin

vaddress_high := address(9 downto 4);

vaddress_low := address(3 downto 0);

vword_out := zero_monitor_word;

--

--  use address_high to select memory space

--

--  use address_low to select specific word

--

case vaddress_high is

when smt_address_space =>

case vaddress_low is

when address0 =>

vword_out := psmt_registers(0);

when address1 =>

vword_out := psmt_registers(1);

when address2 =>

vword_out := psmt_registers(2);

when address3 =>

vword_out := psmt_registers(3);

when others =>

vword_out := zero_monitor_word;

end case;

when chip_address_space =>

case vaddress_low is

when address0 =>

vword_out := pchip_registers(0);

when address1 =>

vword_out := pchip_registers(1);

when address2 =>

vword_out := pchip_registers(2);

when address3 =>

vword_out := pchip_registers(3);

when address4 =>

vword_out := pchip_registers(4);

when address5 =>

vword_out := pchip_registers(5);

when address6 =>

vword_out := pchip_registers(6);

when address7 =>

vword_out := pchip_registers(7);

when address8 =>

vword_out := pchip_registers(8);

when others =>

vword_out := zero_monitor_word;

end case;

when centroid_address_space =>

case vaddress_low is

when address0 =>

vword_out := pcentroid_registers(0);

when address1 =>

vword_out := pcentroid_registers(1);

when address2 =>

vword_out := pcentroid_registers(2);

when others =>

vword_out := zero_monitor_word;

end case;

when others =>

vword_out := zero_monitor_word;

end case;

nword_out  <= to_fifoword(vword_out);

end process;

--

--  This process block implements the registers for the monitor space

--

process(nsmt_registers,nchip_registers,ncentroid_registers,clk,reset)

begin

if(reset='0') then

for i in 3 downto 0 loop

psmt_registers(i) <= zero_monitor_word;

end loop;

for i in 8 downto 0 loop

pchip_registers(i) <= zero_monitor_word;

end loop;

for i in 4 downto 0 loop

pcentroid_registers(i) <= zero_monitor_word;

end loop;

elsif(clk='1' and clk'event) then

for i in 3 downto 0 loop

psmt_registers(i) <= nsmt_registers(i);

end loop;

for i in 8 downto 0 loop

pchip_registers(i) <= nchip_registers(i);

end loop;

for i in 2 downto 0 loop

pcentroid_registers(i) <= ncentroid_registers(i);

end loop;

end if;

end process;

--

--   This process block implements registers

--

process(nsmt, nmonitor_clear, clk,reset)

begin

if(reset = '0') then

psmt <= s0;

pmonitor_clear <= '0';

pword_out <= zero_fifoword;

elsif(clk = '1' and clk'event) then

psmt <= nsmt;

pmonitor_clear <= nmonitor_clear;

pword_out <= nword_out;

end if;

end process;

--

--  This process block is used to construct l3 trailer

--

process(smt_counter(0))

variable vl3temp: fifoword;

begin

vl3temp := to_fifoword(smt_counter(0));

---                5   +      2              +  3      +  22 =

l3_trailer <= "11110"&vl3temp(23 downto 22)&"000"&vl3temp(21 downto 0);

end process;

monitor_clear <= pmonitor_clear;

word_out <= pword_out;

end architecture;

## C.15: CLUSTER PACKAGE

-- This code is for the package- for clustering the data for the counters

-- this is just a package declaration common for all the other files

library altera;

use altera.maxplus2.all;

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

package cluster_package is

type  monitor_word is array(23 downto 0) of std_logic;

type  fifoword is array(31 downto 0) of std_logic;

type  fifobyte is array(7 downto 0) of std_logic;

type  smt_array is array(3 downto 0) of monitor_word;

type  chip_array is array(8 downto 0) of monitor_word;

type  centroid_array is array(2 downto 0) of monitor_word;

constant zero_monitor_word: monitor_word := "000000000000000000000000";

constant zero_fifoword: fifoword := "00000000000000000000000000000000";

constant zero_fifobyte: fifobyte := "00000000";

constant address0: unsigned(3 downto 0) := "0000";

constant address1: unsigned(3 downto 0) := "0001";

constant address2: unsigned(3 downto 0) := "0010";

constant address3: unsigned(3 downto 0) := "0011";

constant address4: unsigned(3 downto 0) := "0100";

constant address5: unsigned(3 downto 0) := "0101";

constant address6: unsigned(3 downto 0) := "0110";

constant address7: unsigned(3 downto 0) := "0111";

constant address8: unsigned(3 downto 0) := "1000";

constant address9: unsigned(3 downto 0) := "1001";

constant addressa: unsigned(3 downto 0) := "1010";

constant addressb: unsigned(3 downto 0) := "1011";

constant addressc: unsigned(3 downto 0) := "1100";

constant addressd: unsigned(3 downto 0) := "1101";

constant addresse: unsigned(3 downto 0) := "1110";

constant addressf: unsigned(3 downto 0) := "1111";

--  define data_types

constant undefined_type: unsigned(1 downto 0) := "00";

constant stereo_type : unsigned(1 downto 0)  := "01";

constant axial_type: unsigned(1 downto 0) := "10";

constant ninety_degrees_type: unsigned(1 downto 0) := "11";

function to_fifoword(a: fifobyte) return fifoword;

function to_fifoword(a: monitor_word) return fifoword;

function to_monitorword(a: unsigned(23 downto 0)) return monitor_word;

function add_one(a: monitor_word) return monitor_word;

end package;

package body cluster_package is

function to_fifoword(a:fifobyte) return fifoword is

variable result: fifoword;

begin

for i in 31 downto 8 loop

result(i) := '0';

end loop;

for i in 7 downto 0 loop

result(i) := a(i);

end loop;

return result;

end function; --

function to_fifoword(a:monitor_word) return fifoword is

variable result: fifoword;

begin

for i in 31 downto 24 loop

result(i) := '0';

end loop;

for i in 23 downto 0 loop

result(i) := a(i);

end loop;

return result;

end function; --

function to_monitorword(a:unsigned(23 downto 0)) return monitor_word is

variable result: monitor_word;

begin

for i in 23 downto 0 loop

result(i) := a(i);

end loop;

return result;

end function; --

function add_one(a:monitor_word) return monitor_word is

variable result: monitor_word;

variable temp: unsigned(23 downto 0);

variable i: integer;

begin

for i in 23 downto 0 loop

temp(i) := a(i);

end loop;

temp := temp + 1;

for i in 23 downto 0 loop

result(i) := temp(i);

end loop;

return result;

end function; --

end;

## C.16: CLUSTER FINDER

------------------------------------------------------------------------------

-- Revision 2 Cluster Finder Algorithm

-- Based on 2-15-2000 Specification Document

-----------------------------------------------------------------------------

-- Initial Design : Reginald Perry (6-12-2000)

-----------------------------------------------------------------------------

--  Add monitor counter  - 7/17/2000 RJP

--  Add end of event bit - 7/17/2000 RJP

library altera;

use altera.maxplus2.all;

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

library cluster_package;

use cluster_package.cluster_package.all;

entity cluster_finder is

port(data_in: in unsigned(22 downto 0);

--  input data word from fifo

data_threshold_1,data_threshold_2: in unsigned(7 downto 0);

-- read as inputs

cluster_type: in unsigned(0 downto 0);

-- use 3 or 5 strips in scalculation

--

--   altera compiler does not like unsigned without vector.

--   Need to have two bits until

--   we can check this out

input_fifo_empty,output_axialfifo_full,clk, reset : in std_logic;

input_fifo_read,output_axialfifo_write: out std_logic;

ad2: out unsigned(10 downto 0); -- address bus

d1,d2,d3,d4,d5: out unsigned(7 downto 0);

current_type: out unsigned(1 downto 0);  -- current data type

end_of_event_out: out std_logic;  -- this is the end of event bit

calculation_type: out unsigned(0 downto 0);

-- this is the type of calculation

l3_enable: out std_logic; -- this is used to enable l3

centroid_counters: out centroid_array;

l3out:out unsigned(31 downto 0)

);

--   inputs for data_threshold

end entity cluster_finder;

architecture behavior of cluster_finder is

--  define major states

type mystates is (sreset, sread,sread_fifo,sload_fifo,scheck_for_eof,

sinit,smain,snext,soutpeak,scalculate,swrite,swrite_fifo,

sl3_wait);

type l3states is (s0,s1,s2,s3,s4,s5,s6);

--  data types are now defined in cluster_package

--    constant axial: unsigned(1 downto 0) := "01";

--    constant stereo: unsigned(1 downto 0) := "10";

--    constant z_axis: unsigned(1 downto 0) := "11";

--    constant undefined: unsigned(1 downto 0) := "00";

signal ns, ps,nreturn_state,preturn_state: mystates;

signal nl3s,pl3s: l3states;

--  counter used to keep track of number of strips read

signal nstrip_count, pstrip_count: unsigned(7 downto 0);

--  signals used for data and address registers

signal nd1,nd2,nd3,nd4,nd5,ndata_shadow1,

ndata_shadow2: unsigned(7 downto 0);

signal pd1,pd2,pd3,pd4,pd5,pdata_shadow1,

pdata_shadow2: unsigned(7 downto 0);

signal nad1,nad2,nad3,nad4,nad5,naddress_shadow1,

naddress_shadow2: unsigned(10 downto 0);

signal pad1,pad2,pad3,pad4,pad5,paddress_shadow1,

paddress_shadow2: unsigned(10 downto 0);

--  these signals are used to keep track of the current data type

signal nnext_address,pnext_address: unsigned(10 downto 0);

signal ncurrent_type,pcurrent_type: unsigned(1 downto 0);

signal nfirst_read,pfirst_read,data_compare,valid_cluster_peak: std_logic;

signal pdata,ndata: unsigned(22 downto 0);

--   these signals are for the end of event bit

signal neof,peof,nl3_wait,pl3_wait: std_logic;

---  these signals are used for monitoring

signal naxial_counter,paxial_counter,nstereo_counter,

pstereo_counter: unsigned(23 downto 0);

signal nninety_counter, pninety_counter: unsigned(23 downto 0);

--  l3buffer

signal pl3out,nl3out: unsigned(31 downto 0);

--  these constants are used with the data registers

constant zero_data_reg: unsigned(7 downto 0) := "00000000";

constant zero_address_reg: unsigned(10 downto 0) := "00000000000";

constant zero23:unsigned(22 downto 0) := "00000000000000000000000";

constant zero24:unsigned(23 downto 0) := "000000000000000000000000";

constant zero32: unsigned(31 downto 0) := "00000000000000000000000000000000";

constant true: std_logic := '1';

constant false: std_logic := '0';

--

--   l3stuff

begin

process(ps,preturn_state,pd1,pd2,pd3,pd4,pd5,pad1,pad2,pad3,pad4,pad5,

paddress_shadow1,paddress_shadow2,pdata_shadow1,pdata_shadow2,

pnext_address,pcurrent_type,pdata,preturn_state,pfirst_read,pdata,

pstrip_count,data_compare,valid_cluster_peak,data_threshold_1,

data_threshold_2,input_fifo_empty,cluster_type,paxial_counter,

pstereo_counter,pninety_counter,pl3_wait)

variable vtype: unsigned(1 downto 0);

variable veof,vnew_data_flag,end_of_cluster: std_logic;

variable vdata: unsigned(7 downto 0);

variable vaddress: unsigned(10 downto 0);

variable vl3out:unsigned(31 downto 0);

begin

--   set default outputs

--   state information

ns <= ps;

nreturn_state <= preturn_state;   --  hold onto return state

--   save data buffers

nd1 <= pd1;

nd2 <= pd2;

nd3 <= pd3;

nd4 <= pd4;

nd5 <= pd5;

ndata_shadow1 <= pdata_shadow1;

ndata_shadow2 <= pdata_shadow2;

--  save address buffers

nad1 <= pad1;

nad2 <= pad2;

nad3 <= pad3;

nad4 <= pad4;

nad5 <= pad5;

naddress_shadow1 <= paddress_shadow1;

naddress_shadow2 <= paddress_shadow2;

--   parse current input word in parts. Use variables for this.

vtype := pdata(22 downto 21);

vnew_data_flag := pdata(20); -- variable defined as std_logic

veof := pdata(19);           -- variable defined as std_logic

vdata := pdata(18 downto 11);

vaddress := pdata(10 downto 0);

--  save eof for next block

neof <= veof;

--

--   set defaults for these signals

--

ncurrent_type <= pcurrent_type;    --  current data type

nnext_address <= pnext_address;

nfirst_read <= pfirst_read;

--  flag to indicate we are coming out of a reset state

--  this prevents a centriod calculation before the first read.

ndata <= pdata;    -- save data

--    This implements the main FSM

case ps is

--

--   Reset state

--   This is the first state AFTER the initial reset

--

when sreset =>

--

--    Need to load data parameters here eventually.

--    For now, just go to the first read

--

ns <= sread;

ncurrent_type <= axial_type;

-- default to axial type to give memory something to access

nreturn_state <= sinit;

-- this statement is really not needed since this is the first read

nfirst_read <= true;     -- but let's help the synthesis tool anyway.

--  Read state

when sread =>

if(input_fifo_empty = true) then

ns <= sread;

--  fifo is empty. Need to wait for some data

else

ns <= sread_fifo;

--  data is ready  Go to next state to read_fifo

end if;

--   Read fifo state

--   add this "wait" state to allow data to be read from input fifo

when sread_fifo =>

ns <= sload_fifo;

--

--    Data should be available at fifo.  Load into an internal register data

--

when sload_fifo =>

ndata <= data_in;

ns <=  scheck_for_eof;  -- let's go check for the eof

--

--     probably we can combine sread_fifo and sload_fifo.  Look at this later

--

--  This state checks for an eof

--

when scheck_for_eof =>

-- if veof=1 then this is the end of a cluster

-- if veof=0 then end of cluster is initial reset to 0

end_of_cluster :=veof;

--

-- Check for a change of type

if(vtype /= pcurrent_type) then

end_of_cluster := true;

ncurrent_type <= vtype;   -- save new data type as next current_type

end if;

-- if data is less than threshold then end of cluster.  Data_threshold_1 is an input

if(vdata < data_threshold_1) then

end_of_cluster := true;

end if;

-- if current_address /= next_address then end of cluster

-- need to set next_address based on this outcome

if(vaddress /= pnext_address) then

end_of_cluster := true;

nnext_address <= vaddress + 1;

else

nnext_address <= pnext_address + 1;

end if;

--

--   if this is the first read then we don't want to store

--   a hit in the output fifo

--

if(pfirst_read = true) then

nfirst_read <= false;

ns<= sinit;

elsif(end_of_cluster = '1') then

--

--      This is the start of a new cluster.

--      Need to calculate centroid on old cluster.

--      The current data word will be used in the new cluster

--

ns <= scalculate;

nreturn_state <= sinit;

-- after calculate need to initialize current data word

else

ns <= preturn_state;

--  this is not the end of a cluster.  continue cluster finding algorithm

end if;

--  Initialization State.  We are starting the search for a new cluster

when sinit =>

-- init data registers

nd1 <= zero_data_reg;

nd2 <= zero_data_reg;

nd3 <= vdata;

nd4 <= zero_data_reg;

nd5 <= zero_data_reg;

-- init address registers

nad1 <= zero_address_reg;

nad2 <= zero_address_reg;

nad3 <= vaddress;

nad4 <= zero_address_reg;

nad5 <= zero_address_reg;

-- init shadow registers

ndata_shadow2 <= zero_data_reg;

ndata_shadow1 <= zero_data_reg;

naddress_shadow2 <= zero_address_reg;

naddress_shadow1 <= zero_address_reg;

--  go read the next word and return to smaim

ns <= sread;

nreturn_state <= smain;

--   this is the main state.  Compare the next data word to the current cluster center

when smain =>

--   next state will be sread

ns <= sread;

if(data_compare = '1') then

--

--  if data  is greater than or equal to current peak then move data as cluster peak

--  need to shift all other data words to the left by one slot

nd1 <= pd2;

nd2 <= pd3;

nd3 <= vdata;

--  need to also shift addresses

nad1 <= pad2;

nad2 <= pad3;

nad3 <= vaddress;

nreturn_state <= smain;    --  come back to this state

else

--   Data is less than current cluster peak.  Store this value in d4.

nd4 <= vdata;

nad4 <= vaddress;

nreturn_state <= snext;  ---  need to go to snext state

end if;

--

--   this is the next state.  This looks at the D5 register

--

when snext =>

--  next state will be sread

ns <= sread;

if(data_compare = '1') then

--  if data is greater than or equal to current cluster peak

--  shift d3 to d1 and d4 to d2 and new peak (nd3) gets vdata

--  must also remember to zero out nd4 and nd5

nd1 <= pd3;

nd2 <= pd4;

nd3 <= vdata;

nd4 <= zero_data_reg;

nd5 <= zero_data_reg;

--   also need to change the addresses

nad1 <= pad3;

nad2 <= pad4;

nad3 <= vaddress;

nad4 <= zero_address_reg;

nad5 <= zero_address_reg;

nreturn_state <= smain;

else

--   vdata  is less than current cluster peak

--   save vdata in nd5 also need to set up "shawdow" registers

nd5 <= vdata;

nad5 <= vaddress;

--   need to setup shadow registers for "out-peak" case

ndata_shadow1 <= pd4;

ndata_shadow2 <= vdata;

naddress_shadow1 <= pad4;

naddress_shadow2 <= vaddress;

--   return state should be out peak

nreturn_state <= soutpeak;

end if;

--  This is the outpeak case.

--  That is, the current cluster address is out of the range of the

--  cluster peak.  However, we need to save the last two values in case

--  we come across a case where

--  vdata > pd3.

when soutpeak =>

--  next state will be sread

ns <= sread;

--

-- This is where we check the data against the current cluster peak

--

if(data_compare = '1') then

-- data is greater than current cluster peak.

-- Need to move shadow registers into real registers

-- don't forget to zero d4 and d5

nd1 <= pdata_shadow1;

nd2 <= pdata_shadow2;

nd3 <= vdata;

nd4 <= zero_data_reg;

nd5 <= zero_data_reg;

--   same goes for the addresses

nad1 <= paddress_shadow1;

nad2 <= paddress_shadow2;

nad3 <= vaddress;

nad4 <= zero_address_reg;

nad5 <= zero_address_reg;

nreturn_state <= smain;   --- return now goes to smaim

else

--

--  din is not greater than current peak.

--  shift shadow registers and return to this state

--

ndata_shadow1 <= pdata_shadow2;

ndata_shadow2 <= vdata;

naddress_shadow1 <= paddress_shadow2;

naddress_shadow2 <= vaddress;

nreturn_state <= soutpeak;

end if;

--

--  This is the centriod calculate state

--  Actually centroid is always being calculated

--  we just need to determine if we send it on to the

--  hit filter.

--

when scalculate =>

--

--

--

if(valid_cluster_peak = true) then   --- this signal is generated by another process block

--

--          need to and increment counters

case vtype is

when axial_type =>

naxial_counter <= paxial_counter + 1;

when stereo_type =>

nstereo_counter <= pstereo_counter + 1;

when ninety_degrees_type =>

nninety_counter <= pninety_counter + 1;

when others =>

null;

end case;

ns <= swrite;

else

--

--   this is not a valid peak.  Throw away this cluster and start over

--

ns <= sinit;

end if;

when swrite =>

--   dummy state to see if output fifo is not empty

if(output_axialfifo_full = true) then

ns <= swrite;

else

ns <= swrite_fifo;

end if;

when swrite_fifo =>

-- output fifo is written during this cycle

ns <= sl3_wait;   --  let's wait for l3 to finish

when sl3_wait =>

if(pl3_wait = '1') then

ns <= sl3_wait;

else

ns <= sinit;

end if;

when others =>

-- we should never come here.  Therefore go to sreset

ns <= sreset;

end case;

end process;

--

--  this state machine is used to read a word from the input fifo

--

process(ps)

variable vread: std_logic;

begin

vread := false;

case ps is

when sread_fifo =>

vread := true;

when others =>

vread := false;

end case;

input_fifo_read <= vread;

end process;

--

--  this state machine is used to write a word to the output fifo

--

process(ps)

variable vwrite: std_logic;

begin

vwrite := false;

case ps is

when swrite_fifo =>

vwrite := true;

when others =>

vwrite := false;

end case;

output_axialfifo_write<= vwrite;

-- do this as a mealy machine to save a cycle

end process;

--

--   This process block counts the number of strips in the current cluster (-1)

--

process(ps,pstrip_count)

variable vcnt : unsigned(7 downto 0);

begin

vcnt := pstrip_count;

case ps is

when sinit =>

vcnt := zero_data_reg;

when smain|snext|soutpeak =>

-- only increment this counter when we have new data

vcnt := vcnt + 1;

when others =>

null;

end case;

nstrip_count <= vcnt;

end process;

--

--   This process block compares the data against the current peak

--

process(pdata,pd3)

variable vdata:unsigned(7 downto 0);

begin

vdata := pdata(18 downto 11);

--

--  compare data word to current cluster center

--

if(vdata < pd3) then

data_compare <= '0';

else

data_compare <= '1';

end if;

end process;

--

--   This process block compares pd3 against data_threshold_2

--

process(pd3,data_threshold_2)

variable vdata:unsigned(7 downto 0);

begin

vdata := pd3;

--

--  compare data word to current cluster center

--

if(vdata < data_threshold_2) then

valid_cluster_peak <= false;

-- current peak is less than data_threshold_2 -- invalid peak.

else

valid_cluster_peak <= true;   -- we have a valid peak here.

end if;

end process;

--

--  Register process block for state information

--

process (ns,nreturn_state,nl3s,clk, reset)

begin

if( reset = '0') then

ps <= sreset;

preturn_state <= sreset;

pl3s <= s0;

elsif(clk = '1' and clk'event) then

ps <= ns;

preturn_state <= nreturn_state;

pl3s <= nl3s;

end if;

end process;

--

--  Register process block for misc signals

--

process (ncurrent_type,nnext_address, pfirst_read, clk, reset)

begin

if( reset = '0') then

pcurrent_type <= undefined_type;

pnext_address <= zero_address_reg;

pfirst_read <= '0';

elsif(clk = '1' and clk'event) then

pcurrent_type <= ncurrent_type;

pnext_address <= nnext_address;

pfirst_read <= nfirst_read;

end if;

end process;

--

--  Register process block to shadow address and data registers also the data is saved here

--

process(nd1,nd2,nd3,nd4,nd5,nad1,nad2,nad3,nad4,nad5,nstrip_count,

nl3out,nl3_wait,naddress_shadow1,naddress_shadow2,

ndata_shadow1,ndata_shadow2,ndata, clk, reset)

begin

if( reset = '0') then

pd1 <= zero_data_reg;

pd2 <= zero_data_reg;

pd3 <= zero_data_reg;

pd4 <= zero_data_reg;

pd5 <= zero_data_reg;

pad1 <= zero_address_reg;

pad2 <= zero_address_reg;

pad3 <= zero_address_reg;

pad4 <= zero_address_reg;

pad5 <= zero_address_reg;

pdata_shadow1 <= zero_data_reg;

pdata_shadow2 <= zero_data_reg;

paddress_shadow1 <= zero_address_reg;

paddress_shadow2 <= zero_address_reg;

pdata <= zero23;

pstrip_count <= zero_data_reg;

pl3out <= zero32;

peof <= '0';

pl3_wait <= '0';

elsif( clk'event and clk = '1') then

pd1 <= nd1;

pd2 <= nd2;

pd3 <= nd3;

pd4 <= nd4;

pd5 <= nd5;

pad1 <= nad1;

pad2 <= nad2;

pad3 <= nad3;

pad4 <= nad4;

pad5 <= nad5;

pdata_shadow1 <= ndata_shadow1;

pdata_shadow2 <= ndata_shadow1;

paddress_shadow1 <= naddress_shadow1;

paddress_shadow2 <= naddress_shadow1;

pdata <= ndata;

pstrip_count <= nstrip_count;

pl3out <= nl3out;

peof <= neof;

pl3_wait <= nl3_wait;

end if;

end process;

--  Registers for counters

process(clk,reset,naxial_counter,nstereo_counter,nninety_counter)

begin

if(reset = '0') then

paxial_counter <= zero24;

pstereo_counter <= zero24;

pninety_counter <= zero24;

elsif(clk'event and clk='1') then

paxial_counter <= naxial_counter;

pstereo_counter <= nstereo_counter;

pninety_counter <= nninety_counter;

end if;

end process;

--  this process block takes care of l3, we will need multiple cycles to write all the data

process(ps, pl3s,pd1,pd2,pd3,pd4,pd5,pad1,pad2,pad3,pad4,pad5,data_threshold_1,data_threshold_2,pstrip_count,

valid_cluster_peak, pcurrent_type, cluster_type)

variable vl3out: unsigned(31 downto 0);

begin

-- default outputs

nl3_wait <= '0';

vl3out := zero32;

nl3s <= pl3s;

case pl3s is

when s0 =>

l3_enable <= '0';

if(ps = scalculate) then

nl3_wait <= '1';  --- tell main fsm to hold

nl3s <= s1;

--  need to set up first l3 output word

vl3out(31 downto 28) := "0000";

vl3out(27 downto 20) := data_threshold_2;

vl3out(19 downto 18) := pcurrent_type;

vl3out(17) := cluster_type(0);     --- only a single bit

vl3out(16 downto 9) := data_threshold_1;

vl3out(8 downto 1) := pstrip_count;

vl3out(0) := '1';

else

nl3_wait <= '0';

nl3s <= s0;

end if;

when s1  =>

nl3_wait <= '1';

l3_enable <= '1';  -- write the previous word into l3 using l3_enable --do not register

vl3out(31 downto 23) := "000000000";

vl3out(22 downto 21) := pcurrent_type;

vl3out(20 downto 13) := pd2;

vl3out(12 downto 2) := pad2;

vl3out(1 downto 0) := "00";

nl3s <= s2;

when s2 =>

nl3_wait <='1';

l3_enable <= '1';

---  write the remaining data out

vl3out(31 downto 23) := "000000000";

vl3out(22 downto 21) := pcurrent_type;

vl3out(20 downto 13) := pd3;

vl3out(12 downto 2) := pad3;

vl3out(1 downto 0) := "00";

nl3s <= s3;

when s3 =>

nl3_wait <='1';

l3_enable <= '1';

---  write the remaining data out

vl3out(31 downto 23) := "000000000";

vl3out(22 downto 21) := pcurrent_type;

vl3out(20 downto 13) := pd4;

vl3out(12 downto 2) := pad4;

vl3out(1 downto 0) := "00";

nl3s <= s4;

when s4 =>

nl3_wait <='1';

l3_enable <= '1';

---  write the remaining data out

vl3out(31 downto 23) := "000000000";

vl3out(22 downto 21) := pcurrent_type;

vl3out(20 downto 13) := pd5;

vl3out(12 downto 2) := pad5;

vl3out(1 downto 0) := "00";

nl3s <= s5;

when s5 =>

nl3_wait <='0';      --  we can let main FSM restart now

l3_enable <= '1';

---  write the remaining data out

vl3out(31 downto 23) := "000000000";

vl3out(22 downto 21) := pcurrent_type;

vl3out(20 downto 13) := pd1;

vl3out(12 downto 2) := pad1;

vl3out(1 downto 0) := "00";

nl3s <= s6;    --  need one more state to write out last word

when s6 =>

nl3_wait <='0';

l3_enable <= '1';       --- write out last word

nl3s <= s0;

when others =>

null;

end case;

nl3out <= vl3out;  -- this will save l3out for writing into l3

end process;

--   attach address and data registers to output buffers

d1 <= pd1;

d2 <= pd2;

d3 <= pd3;

d4 <= pd4;

d5 <= pd5;

ad2 <= pad2;

l3out <= pl3out;

current_type <= pcurrent_type;

end_of_event_out <= peof;

calculation_type <= cluster_type;

--  attach counters

centroid_counters(0) <= to_monitorword(paxial_counter);

centroid_counters(1) <= to_monitorword(pstereo_counter);

centroid_counters(2) <= to_monitorword(pninety_counter);

end behavior;

## C.17: CENTROID CALCULATOR

----------------------------------------------------------------

--  File: centroid_calculator.vhd

--  Description:  VHDL code to calculate the three or five strip centroid.

--  For the five strip case we are given five (5) eight bit values -

-- d1,d2,d3,d4,d5

--  The centroid is usually given by  (d1+2d2+3d3+4d4+5d5)/(d1+d2+d3+d4+d5)

--  in this case we are shifting the result so it is centered around d2

--  so the numerator becomes   -d1+0d2+d3+2d4+3d5

--  For the three strip case: the equation is -d1+0d2+d3/d1+d2+d3

--

--  Originally Created:   6/4/00

-- (Reginald J. Perry - FAMU-FSU College of Engineering)

-- Add l3 logic:  7/17/2000   RJP

-- Modified : by Shweta Lolage

-- Changes in the 3 cluster centroid calculations

------------------------------------------------------------------

--- normal initialization stuff

library altera;

use altera.maxplus2.all;

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

--

--  define entity

--

entity centroid_calculator is

--

--  The maximum value is given by 5*255 which will need 8+3+1=12 bits to save --- (need  one bit for the sign).  We'll assume the d's are unsigned values.

--  Therefore, we will need N+4 bits for the num and den results.

port(d1,d2,d3,d4,d5: in signed(7 downto 0);

threshold_range_in:in unsigned(35 downto 0); -- this is threshold data

address_d2: in signed(10 downto 0);

-- this is the original address of d2

calculation_type: in std_logic;

centroid_type: in signed(1 downto 0);

end_of_event_bit: in signed(0 downto 0);

l3_trailer: in signed(31 downto 0);

l3_out: out signed(31 downto 0);

centroid_out: out signed(17 downto 0)

);

end entity centroid_calculator;

architecture behavior of centroid_calculator is

signal  num,den: signed(11 downto 0);

constant three_strip_centroid: std_logic := '0';

constant five_strip_centroid: std_logic := '1';

constant high: std_logic := '1';

constant low: std_logic := '0';

constant zero9 : signed(8 downto 0) := "000000000";

-- these signals are needed by the divider circuit

--

signal q,qint: signed(3 downto 0);

signal y3,y3m,y2,y2m,y1,y1m: signed(11 downto 0);

signal centroid,sum3,sum2,sum1,sum0: signed(12 downto 0);

--  These signals and constants are used to calculate the quantized pulse areas

signal centroid_pulse_area: signed(1 downto 0);

signal level1_check,level2_check,level3_check: std_logic;

signal level_check: std_logic_vector(2 downto 0);

constant level0 : signed(1 downto 0) := "00";

constant level1 : signed(1 downto 0) := "01";

constant level2 : signed(1 downto 0) := "10";

constant level3 : signed(1 downto 0) := "11";

begin

--   This process block provides the sums for the numerator and denominator

summer: process(d1,d2,d3,d4,d5,calculation_type)

--  Create variables to temporarily save results

variable s1,s2,s3,s4,s5           : signed(8 downto 0);

variable temp1,temp1a,temp2,temp3 : signed(9 downto 0);

variable temp32,temp5             : signed(10 downto 0);

variable parea,temp4,temp6,temp7  : signed(11 downto 0);

variable temp1b,temp2b            : signed (9 downto 0);

variable temp3b	                : signed (10 downto 0);

begin

--

--  s's are used to add a sign bit to the d's

--

s1 := "0"&d1;

s2 := "0"&d2;

s3 := "0"&d3;

s4 := "0"&d4;

s5 := "0"&d5;

--  define temporary variables needed by both the three and five strip calculator

temp1 := s1+s2;          -- this sum will be 10 bits (d1+d2)

temp2 := s3-s1;          -- this sum will be 10 bits (d3-d1)

--  Use two separate adders for three and five strip centroid calculation

--  this may save some time in the calculation

if(calculation_type = five_strip_centroid) then

temp3 := s4+s5;          -- this sum will be 10 bits (d4+d5)

temp32 := temp3&"0";     -- this is 2xtemp3 = 2(d4+d5); it will be 11 bits

temp4 := ("00"&s5)+temp32;    -- this is 2d4 + 3d5: sum will be 12 bits

temp5 := temp1 + temp3;       -- this is d1+d2+d4+d5: sum will be 11 bits

den <= ("000"&s3)+temp5;     -- this is d1+d2+d3+d4+d5: need 12 bits

temp7 := "00"&temp2;         -- bit extend temp2 from 10 to 12 bits

num <=  temp7+temp4;         --  this is -d1+d3+2d4+3d5:  need 12 bits

else

-- this is the three strip case. num is -d2+d4 and den = d2+d3+d4

temp1b := s4 - s2;  -- this will be num = -d2+d4

num <= "000"&temp1b;    -- sign extend to 12 bits

temp2b := s2 + s4;  -- the sum of d2 + d4 , for the den

temp1a := "00"&d3;   --  need to sign extend d3 to add it to temp1

den <= "0"&(temp2b+temp1a);

end if;

end process summer;

--   These process blocks implements the divider circuit.

--   The design consists of four identical process blocks with a 2x1 mux

--   Each process calculates   yx - b  where yx the result from the previous

--   process block.  For the first process block, yx=a.  If the result of yx-a

-- is greater than 0 (i.e. positive)

-- then this result is given to the next process

--   block otherwise  yx is propagated.

-- A mux is used to select between the two possiblities.

--

--

p3:process(num,den)

begin

sum3 <= num - den;

--

--  the "sign bit" is stored in sum(12) and saved as qint(3)

--

qint(3) <= sum3(12);

end process;

--

--  if qint(3) is 1, then the result is negative and we DONT want to keep the subtraction result

--  in this case we'll send yx (num in this case) to the next process block.  If qint = 0 , then

--   yx >= b and we want to keep the subtraction.

--

with qint(3) select

y3m   <=  num                  when '1',

sum3(11 downto 0)    when '0',

num                    when others;

--

--  We need to shift the result from above to the left by one bit so we can calculate the next

--  bit of the division.  We could also accomplish this by shifting den one bit to the right.

--

y3 <= y3m(10 downto 0) & '0';

--

--  This process block is identical to the above except num has been

--  replaced with y3 and

--  the appropriate substitutions have been made for sum, y, and qint.

--

p2:process(y3,den)

begin

sum2 <= y3 - den;

qint(2) <= sum2(12);

end process;

with qint(2) select

y2m  <=  y3                when '1',

sum2(11 downto 0) when '0',

y3                 when others;

y2 <= y2m(10 downto 0) & '0';

--

--  This process block is identical to the above except y3

--  has been replaced with y2 and

--  the appropriate substitutions have been made for sum, y, and qint.

--

p1:process(y2,den)

begin

sum1 <= y2 - den;

qint(1) <= sum1(12);

end process;

with qint(1) select

y1m  <=  y2                when '1',

sum1(11 downto 0) when '0',

y2                 when others;

y1 <= y1m(10 downto 0) & '0';

--

--  This process block is identical to the above except

--  y2 has been replaced with y1 and

--  the appropriate substitutions have been made for sum, y, and qint.

--

p0:process(y1,den)

begin

sum0 <= y1 - den;

qint(0) <= sum0(12);

end process;

inv:for I in 3 downto 0 generate

q(i) <= not qint(i);

end generate inv;

--  q's now contain the result of the division.

--  Need to add this result to the original address of d2

process(address_d2,q)

variable tempa, tempb: signed(12 downto 0);

constant zero10: signed(9 downto 0) := "0000000000";

begin

-- the result should be 13 bits

-- original 11 bits plus two extra bits

-- let's use a couple of variables so we don't get lost

tempa := address_d2&"00";

tempb := zero10&q(3 downto 1);

centroid <= tempa + tempb;

end process;

--  This process block will calculate the two bit pulse area threshold values

---------------------------------------------------------------------------------

---------------------------------------------------------------------------------

process (threshold_range_in,den)

--  use variables to break up data_range data

variable level1_low,level2_low,level3_low:unsigned(11 downto 0);

variable parea:signed(1 downto  0);

constant code_level0:std_logic_vector(2 downto 0)  := "000";

constant code_level1: std_logic_vector(2 downto 0) := "001";

constant code_level2: std_logic_vector(2 downto 0) := "011";

constant code_level3 : std_logic_vector(2 downto 0) := "111";

begin

--  implement this like a coarse approximation ADC

--  Check input data against threshold levels and then use an encoder to

--  convert thermometer code to binary code

level1_low := threshold_range_in(11 downto 0);

level2_low := threshold_range_in(23 downto 12);

level3_low := threshold_range_in(35 downto 24);

-- make the default level 3

level1_check <= high;

level2_check <= high;

level3_check <= high;

if(den < level1_low) then

level1_check <= low;

end if;

if(den < level2_low) then

level2_check <= low;

end if;

if(den < level3_low) then

level3_check <= low;

end if;

level_check <= level3_check&level2_check&level1_check;

case level_check is

when code_level0 =>

parea := level0;

when code_level1 =>

parea := level1;

when code_level2 =>

parea := level2;

when code_level3 =>     -- this is the default condition

parea := level3;

when others =>

parea := level3;

end case;

centroid_pulse_area <= parea;

end process;

-- this process block is for l3 for the centroids

process(l3_trailer,address_d2,centroid_pulse_area,centroid_type,end_of_event_bit)

variable vsmt,vhdi_id: signed(2 downto 0);

variable vseq_id: signed(7 downto 0);

begin

vsmt := l3_trailer(21 downto 19);

vhdi_id := l3_trailer(10 downto 8);

vseq_id := l3_trailer(18 downto 11);

if(end_of_event_bit(0) = '1') then

l3_out <= l3_trailer;

else

--              1+  3  +  2          +     2             +  8    + 3     +  13   =

l3_out <= "0"&vsmt&centroid_type&centroid_pulse_area&vseq_id&vhdi_id&centroid;

end if;

end process;

--  centroid out is the end_of_event, datatype, centroid_pulse_area and centroid

--                        1         +     2       +     2             +  13

centroid_out <= end_of_event_bit(0)&centroid_type&centroid_pulse_area&centroid;

end architecture behavior;

## C.18: HIT CONTROL BLOCK

-------------------------------------------------------------------

--File:hit_control_block.vhd

-- To implement the control logic for the hit filter.

-- Originally created by : Shweta Lolage date - 05/15/2000

--Modifications by Shweta : - 06/02/2000

-- modified by Shweta lolage on 06/15/2000 to remove all unnecessary registered signals

-- modified to include one state to accommodate a control signal to read the masked register

-- by Shweta Lolage - 09/04/2000

--------------------------------------------------------------------

library altera;

use altera.maxplus2.all;

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

--  Entity Declaration

entity hit_control_block is

port (road_write,done,event_start : in std_logic;

-- road_write signal from the road data handler

-- DONE signal is from the pointer block indicating

--that it is through with the hits for that particular centroid.

-- event _start is a global signal marking the start of an event

clock,reset : in std_logic;

last_road : in std_logic; --signal indicating end of road data

fifoempty : in std_logic;   -- active low signal

datatype  : in std_logic_vector (2 downto 0);

-- the end of event tagged with the datatype of the centroid

-- coming from the centroid finder

centroid  : in unsigned (12 downto 0);

-- centroids from the centroid finder

road,mask : out unsigned (5 downto 0);

-- the signals to the ROMs to get the corrected load road signal

-- and final mask register

read_comp,read_hitreg,input_fifo_enable,hits_busy : out std_logic;

--the read_comp signal is to read the output from the comparators

-- this read_hitreg signal is to read the masked hit register

-- the input_fifo_enable signal is a read fifo signal

-- to the centroid's fifo

-- the hits_busy signal is to indicate that the

-- hitfilter has started pulling out

-- the centroids and is busy processing hits

z_write    : out std_logic; -- fifo enable for the z-axis fifo

a_centroid_comp  : out unsigned (13 downto 0);

-- the extra bit is for the last centroid or end of event bit for the L3

z_centroid_fifo  : out unsigned (13 downto 0);

road_event_count : out unsigned (7 downto 0);

-- this is the internal event counter for the hitfilter

count            : out unsigned(5 downto 0)

--this output indicates the number of comparators loaded with road_data

);

end entity hit_control_block;

--  Architecture Body

architecture behavior of hit_control_block is

type states is (sevent,sfirstload,snext,sload,slast_road,sdecide,

scentroid_write,scomp_read,shitreg_read,swait,sdata_wait);

signal ncount,pcount,vcount  : unsigned (5 downto 0);

signal nstate, pstate : states;

signal nroad,proad,nmask,pmask: unsigned (5 downto 0);

signal nread_comp,pread_comp: std_logic;

signal nread_hitreg,pread_hitreg: std_logic;

signal ndone,pdone : std_logic;

signal nhits_busy,phits_busy : std_logic;

signal ninput_fifo_enable,pinput_fifo_enable,nfifoempty,pfifoempty,

nlast_road,plast_road : std_logic;

signal nroad_write,proad_write : std_logic;

signal loadin: std_logic; --signals to load the road_data

signal nroad_event_count,proad_event_count : unsigned (7 downto 0);

signal vroad_event_count : unsigned (7 downto 0);

signal nmux_select,pmux_select : std_logic;

signal mux_select : std_logic;

signal ncentroid, pcentroid : unsigned (12 downto 0);

signal vz_centroid_fifo,va_centroid_comp : unsigned (13 downto 0);

-- this for temporary assignment of z-centroids and axial centroids

signal nz_write,pz_write : std_logic;

signal nend_of_event, pend_of_event : std_logic;

signal nevent_end,pevent_end : std_logic;

signal nrerr,prerr : std_logic;

constant zero6  : unsigned (5 downto 0)  :="000000";

constant zero13 : unsigned (12 downto 0) := "0000000000000";

constant zero14 : unsigned (13 downto 0) := "00000000000000";

constant zero46 : unsigned (45 downto 0) := "0000000000000000000000000000000000000000000000";

begin

ncentroid <= centroid;

nend_of_event <= datatype(2);

-- the end of event bit is tagged along with the datatype

-- and the centroids in the input FIFO

-- this process block is used to keep assign a track number

-- to the incoming road data

process (road_write,plast_road,pcount,pstate,pread_comp,done,pdone,pfifoempty,

phits_busy,pz_write)

begin

-- initialization of the signals

ncount             <= pcount;

nread_comp         <= pread_comp;

nread_hitreg       <= pread_hitreg;

nstate             <= pstate;

ndone              <= done;

ninput_fifo_enable <= pinput_fifo_enable;

nfifoempty         <= fifoempty;

nroad_write        <= road_write;

nlast_road         <= last_road;

nhits_busy         <= phits_busy;

nroad_event_count  <= proad_event_count;

vroad_event_count  <= "00000000";

nz_write           <= pz_write;

nevent_end         <= pevent_end;

case pstate is

when sevent =>

ncount <= zero6;

vcount <= zero6;

nhits_busy <= '0';

loadin <= '0';

ninput_fifo_enable <= '0';

nz_write     <= '0';

nevent_end <= '0';

if(event_start = '1') then

-- yes, we have a new event so go to the next state

nstate <= sfirstload;

nroad_event_count <= proad_event_count + 1;

-- the new internal event count

else

nstate <= sevent;

end if;

when sfirstload =>  -- will load the first road into the comparator

if (proad_write = '1') then

-- do we have a road_write signal only

-- then this the first road and there are more to come

loadin <= '1';

vcount<= zero6;

nstate <= snext;

elsif(plast_road = '1') then

-- do we have no roads and this is the signal of end of roads

nstate <= slast_road;

else

nstate <= sfirstload;

-- otherwise wait in this state for a road_write signal

end if;

when snext =>

-- checking whether the road_write signal has gone low,

-- only then can we wait for the next road_write high

if (proad_write ='0') then

nstate <= sload;

loadin <= '0';

else

nstate <= snext;

end if;

when sload =>  --loading the next road_data into next comaparator

if (proad_write = '1') then

ncount <= pcount + 1;

vcount <= pcount + 1;

loadin <= '1';

nstate <= snext;

elsif (plast_road = '1') then

nstate <= slast_road;

else

nstate <= sload;

end if;

when slast_road =>		-- the previous road was the last raod

-- so now load the centroid

if (pfifoempty = '0') then

ninput_fifo_enable <= '1';

nhits_busy <= '1';

vroad_event_count <= proad_event_count - 1;

-- storing the current event value

nstate <= sdecide;

else

nstate <= slast_road;

end if;

when sdecide =>

ninput_fifo_enable <= '0';

if (datatype(1 downto 0) = "10") then -- if the centroid is axial

mux_select <= '0';

nstate <= scomp_read;

elsif(datatype(1 downto 0) = "11") then

-- if the centroid is z-axis

mux_select <= '1';

nstate <= scentroid_write;

nz_write <= '1';

else

nstate <= slast_road;

end if;

when scentroid_write =>

-- this state writes the z-centroid in the block to be formatted

-- and written to the fifo

nz_write <= '1';

nstate <= slast_road;

when scomp_read =>

-- now the comaparator output is read to be read in the

-- hit register to get valid hits

nread_comp <= '1';	-- active high signal

nstate <= shitreg_read;

when shitreg_read =>

nread_hitreg <= '1';

nread_comp <= '0';

nstate <= swait;

when swait =>

-- wait till the hits for the centroid are found and put in the data format

nread_hitreg <= '0';

if (pdone ='1') then

-- active high signal from the pointer block

if (pfifoempty = '0'and pend_of_event = '1') then

-- indicating it is through with the centroids

nstate <= sevent;

-- yes wait for the new event start

nhits_busy <= '0';

-- the hit filter is not busy, the calculation is done

elsif (pfifoempty = '1' and pend_of_event = '0') then

-- no not yet done, but fifo is still being loaded

nstate <= sdata_wait;

else

nstate <= slast_road;          -- go pull another centroid

end if;

else

nstate <= swait;

end if;

when sdata_wait =>   -- wait for the fifo to go down again

if (pfifoempty = '0') then

nstate <= slast_road;

else

nstate <= sdata_wait;

end if;

when others =>

-- if any other state other than the above defined go to the initial state

nstate <= sevent;

end case;

end process;

-- this process block is to decide whether axial centroids should go to the comparator

-- or the zaxis-degree centroids to the fifo

process(pmux_select,pcentroid)

begin

va_centroid_comp <= zero14;

vz_centroid_fifo <= zero14;

case mux_select is

when '0' =>

va_centroid_comp <= pend_of_event & pcentroid;

when '1' =>

vz_centroid_fifo <= pend_of_event & pcentroid;

when others =>

va_centroid_comp <= zero14;

vz_centroid_fifo <= zero14;

end case;

end process;

--this block assigns the comparator in which road-data will be loaded

process(loadin,pcount,proad)

variable tcount : unsigned (5 downto 0);

begin

nroad <= proad;

tcount := vcount + 1;

if(loadin = '1') then   -- active low signal

road <= tcount;

else

road <= zero6;

end if;

end process;

-- this block give	is out the final mask to be used to determine the hits

process(pread_comp,pmask)

variable tmask : unsigned (5 downto 0);

begin

tmask := pcount + 1;

nmask <= pmask;

if(pread_comp = '1') then

mask <=tmask;

else

mask <= zero6;

end if;

end process;

-- process block to register the signals

process(clock,reset,nstate,ncount,nread_comp,ndone,nfifoempty,nroad_write,

nlast_road,nhits_busy,nroad,nmask,nroad_event_count,ncentroid,nevent_end)

begin

if (reset ='0') then

pstate             <= sevent;

pcount             <= zero6;

pmask              <= zero6;

proad              <= zero6;

pread_comp         <= '0';

pread_hitreg       <= '0';

pdone              <= '0';

pfifoempty         <= '0';

proad_write        <= '0';

plast_road         <= '0';

phits_busy         <= '0';

proad_event_count  <= "00000000";

pcentroid          <= zero13;

pevent_end         <= '0';

pend_of_event      <= '0';

elsif( clock'event and clock='1') then

pstate             <= nstate;

pcount             <= ncount;

pmask              <= nmask;

proad              <= nroad;

pread_comp         <= nread_comp;

pread_hitreg       <= nread_hitreg;

pdone              <= ndone;

pfifoempty         <= nfifoempty;

proad_write        <= nroad_write;

plast_road         <= nlast_road;

phits_busy         <= nhits_busy;

proad_event_count  <= nroad_event_count;

pcentroid          <= ncentroid;

pevent_end         <= nevent_end;

pend_of_event      <= nend_of_event;

end if;

end process;

-- block to register the centroid fifo enable signal

process( clock,reset,ninput_fifo_enable,nz_write)

begin

if (reset ='0') then

pinput_fifo_enable <= '0';

pz_write <= '0';

elsif( clock'event and clock='0') then

pinput_fifo_enable <= ninput_fifo_enable;

pz_write <= nz_write;

end if;

end process;

-- assignment of all output signals

read_comp         <= pread_comp;

read_hitreg       <= pread_hitreg;

count             <= pcount;

input_fifo_enable <= pinput_fifo_enable;

hits_busy         <= phits_busy;

road_event_count  <= vroad_event_count;

a_centroid_comp   <=  va_centroid_comp;

z_centroid_fifo   <=  vz_centroid_fifo;

z_write           <= pz_write;

end architecture behavior;

## C.19: Z-CENTROID

-------------------------------------------------------------------

--File:z_centroid.vhd

-- To put the z centroids in the right format and then give them to L3

-- and also store them in a FIFO within the design

-- Originally created by : Shweta Lolage : Date 06/29/2000

--------------------------------------------------------------------

-- The initialization of the standard libraries

library altera;

use altera.maxplus2.all;

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

-----------------------------------------------------------------------

--  Entity Declaration

entity z_centroid is

port (clock,reset : in std_logic;

zvc_enable    : in std_logic;

-- this signal indicates whether the data should be written to the fifo's

z_centroid  : in unsigned (13 downto 0);

-- the z- axis centroid  tagged with the end of event on the MSB

z_write     : in std_logic;

-- write signal from the control indicating the centroid coming in

-- is z axis-type

l3_trailer  : in unsigned (31 downto 0);

-- this trailer is passed on from the centroid finder block

pulse_area  : in unsigned (1 downto 0);

-- from the centroid finder output FIFO

z_l3_output : out unsigned (31 downto 0);

-- the output which is passed on to the L3 buffer

z_enable : out std_logic

);

end entity z_centroid;

--  Architecture Body

architecture behavior of z_centroid is

-- internal signals for the process blocks

signal nz_l3_output,pz_l3_output : unsigned (31 downto 0);

signal nz_enable,pz_enable : std_logic;

signal nl3_trailer,pl3_trailer : unsigned (31 downto 0);

signal pevent_status,nevent_status : std_logic;

signal pcentroid,ncentroid     : unsigned(12 downto 0);

signal ppulse_area, npulse_area  :  unsigned (1 downto 0);

constant c_type     : unsigned (1 downto 0)  := "11";

constant zero2      : unsigned (1 downto 0)  := "00";

constant zero4      : unsigned (3 downto 0)  := "0000";

constant zero5      : unsigned (4 downto 0)  := "00000";

constant zero11     : unsigned (10 downto 0) := "00000000000";

constant zero13     : unsigned (12 downto 0) := "0000000000000";

constant zero14     : unsigned (13 downto 0) := "00000000000000";

constant zero17     : unsigned (16 downto 0) := "00000000000000000";

constant zero32     : unsigned (31 downto 0) := "00000000000000000000000000000000";

begin

nl3_trailer   <= l3_trailer;

nevent_status <= z_centroid(13);

ncentroid     <= z_centroid (12 downto 0);

npulse_area   <= pulse_area;

process(zvc_enable,z_write)

begin

nz_l3_output   <= pz_l3_output;

if (zvc_enable = '1' and z_write = '1') then

nz_enable <= '1';

if (pevent_status = '0') then

nz_l3_output <= "0" & pl3_trailer(21 downto 19) & c_type & ppulse_area & pl3_trailer(18 downto 8) & pcentroid;

else

nz_l3_output <= pl3_trailer;

end if;

else

nz_enable <= '0';

end if;

end process;

-- this process block is to register the signals

process( reset,clock,nz_l3_output,nz_enable)

begin

if(reset ='0') then

pz_l3_output  <= zero32;

pl3_trailer   <= zero32;

pcentroid     <= zero13;

ppulse_area   <= zero2;

pevent_status <= '0';

pz_enable <= '0';

elsif (clock'event and clock ='1') then

pz_l3_output   <= nz_l3_output;

pl3_trailer    <= nl3_trailer;

pcentroid      <= ncentroid;

ppulse_area    <= npulse_area;

pevent_status  <= nevent_status;

pz_enable      <= nz_enable;

end if;

end process;

-- assignment of the output signals

z_l3_output   <= pz_l3_output;

z_enable      <= pz_enable;

end architecture behavior;

## C.20: COMPARATOR

-------------------------------------------------------------------

--File:comparator.vhd

-- To compare the strip pointer with the data value from the

-- road data

-- Originally created by : Shweta Lolage

--Modified by Shweta Lolage on 05/15/2000 : Changes made according to the new approach

--------------------------------------------------------------------

library altera;

use altera.maxplus2.all;

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

--  Entity Declaration

entity comparator is

port (clock,reset : in std_logic;

centroid : in unsigned (10 downto 0);

-- the centroid value of out of the centroid finder

uroaddata : in unsigned (10 downto 0); -- the upper road_data address

lroaddata : in unsigned (10 downto 0); -- the lower road)data address

loadroad: in std_logic;

-- load the road data at the inputs into the comparator

hit : out std_logic      -- to give outthe comparator output

);

end entity comparator;

--  Architecture Body

architecture behavior of comparator is

-- internal signals for the process blocks

signal ndata,pdata, compare1, compare2 : std_logic;

signal nroaddatau,proaddatau,nroaddatal,proaddatal : unsigned (10 downto 0);

constant zero11 : unsigned ( 10 downto 0) := "00000000000";

begin

-- this process block loads in the upper and lower road-data in the comparator

process(loadroad,uroaddata,lroaddata)

begin

if (loadroad = '1') then

nroaddatal <= lroaddata;

nroaddatau <= uroaddata;

else

nroaddatal <= proaddatal;

nroaddatau <= proaddatau;

end if;

end process;

-- this block is used for the comparison of the road-data

-- with the given centroid

process(proaddatau,proaddatal,centroid)

begin

compare1 <= '0';

compare2 <= '0';

if( proaddatau /= zero11 and centroid /= zero11 ) then

if (proaddatau >= centroid ) then

compare1 <= '1';

else

compare1 <= '0';

end if;

else

compare1 <= '0';

end if;

if(proaddatal /= zero11 and centroid /= zero11) then

if (proaddatal <= centroid) then

compare2 <= '1';

else

compare2 <= '0';

end if;

else

compare2 <= '0';

end if;

end process;

-- this block gives the result of the comparison

process (compare1,compare2)

begin

ndata <= pdata;

if (compare1 ='1' and compare2 ='1') then

ndata <= '1';

else

ndata <= '0';

end if;

end process;

-- the process block to register the road-data signals

process( nroaddatau,nroaddatal,ndata,clock,reset)

begin

if ( reset ='0') then

proaddatau <= zero11;

proaddatal <= zero11;

pdata <= '0';

elsif ( clock'event and clock='1') then

proaddatau <= nroaddatau;

proaddatal <= nroaddatal;

pdata <= ndata;

end if;

end process;

-- final out from the comparator

hit <= pdata;

end architecture behavior;

## C.21: HIT REGISTER

-------------------------------------------------------------------

-- File:r2_hitreg_v2.vhd

-- this process block finds the valid hits

-- Originally created by : Shweta Lolage

-- By Shweta Lolage on 05/17/2000 : Changes made according to the new approach

--modified on  06/04 /2000 : by Shweta Lolage

-- modified to remove the delays

-- The control output line removed from this block and put in the control logic as an additional state

-- date - 09/04/2000 - Shweta Lolage

--------------------------------------------------------------------

library altera;

use altera.maxplus2.all;

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

--  Entity Declaration

entity hitreg is

port (clock, reset : in std_logic;

read_comp : in std_logic;  -- read signal from the control block

mask,hitreg : in unsigned (45 downto 0);

-- the mask register and the hit register used to find the valid hits

hitbit : out std_logic_vector (45 downto 0) -- valid hits

);

end entity hitreg;

--  Architecture Body

architecture behavior of hitreg is

signal nhitbit, phitbit : std_logic_vector (45 downto 0);

constant zero46 : std_logic_vector (45 downto 0) := "0000000000000000000000000000000000000000000000";

begin

-- process block to register in the mask and the hits register

process (mask,hitreg)

begin

if (read_comp = '1') then

nhitbit(0)  <= mask(0)  and hitreg(0);

nhitbit(1)  <= mask(1)  and hitreg(1);

nhitbit(2)  <= mask(2)  and hitreg(2);

nhitbit(3)  <= mask(3)  and hitreg(3);

nhitbit(4)  <= mask(4)  and hitreg(4);

nhitbit(5)  <= mask(5)  and hitreg(5);

nhitbit(6)  <= mask(6)  and hitreg(6);

nhitbit(7)  <= mask(7)  and hitreg(7);

nhitbit(8)  <= mask(8)  and hitreg(8);

nhitbit(9)  <= mask(9)  and hitreg(9);

nhitbit(10) <= mask(10) and hitreg(10);

nhitbit(11) <= mask(11) and hitreg(11);

nhitbit(12) <= mask(12) and hitreg(12);

nhitbit(13) <= mask(13) and hitreg(13);

nhitbit(14) <= mask(14) and hitreg(14);

nhitbit(15) <= mask(15) and hitreg(15);

nhitbit(16) <= mask(16) and hitreg(16);

nhitbit(17) <= mask(17) and hitreg(17);

nhitbit(18) <= mask(18) and hitreg(18);

nhitbit(19) <= mask(19) and hitreg(19);

nhitbit(20) <= mask(20) and hitreg(20);

nhitbit(21) <= mask(21) and hitreg(21);

nhitbit(22) <= mask(22) and hitreg(22);

nhitbit(23) <= mask(23) and hitreg(23);

nhitbit(24) <= mask(24) and hitreg(24);

nhitbit(25) <= mask(25) and hitreg(25);

nhitbit(26) <= mask(26) and hitreg(26);

nhitbit(27) <= mask(27) and hitreg(27);

nhitbit(28) <= mask(28) and hitreg(28);

nhitbit(29) <= mask(29) and hitreg(29);

nhitbit(30) <= mask(30) and hitreg(30);

nhitbit(31) <= mask(31) and hitreg(31);

nhitbit(32) <= mask(32) and hitreg(32);

nhitbit(33) <= mask(33) and hitreg(33);

nhitbit(34) <= mask(34) and hitreg(34);

nhitbit(35) <= mask(35) and hitreg(35);

nhitbit(36) <= mask(36) and hitreg(36);

nhitbit(37) <= mask(37) and hitreg(37);

nhitbit(38) <= mask(38) and hitreg(38);

nhitbit(39) <= mask(39) and hitreg(39);

nhitbit(40) <= mask(40) and hitreg(40);

nhitbit(41) <= mask(41) and hitreg(41);

nhitbit(42) <= mask(42) and hitreg(42);

nhitbit(43) <= mask(43) and hitreg(43);

nhitbit(44) <= mask(44) and hitreg(44);

nhitbit(45) <= mask(45) and hitreg(45);

else

nhitbit <= zero46;

end if;

end process;

-- process block to register the signals

process(clock,reset,nhitbit)

begin

if ( reset = '0') then

phitbit <= zero46;

elsif ( clock'event and clock ='1') then

phitbit <= nhitbit;

end if;

end process;

-- assigning the outputs

hitbit <= phitbit;

end architecture;

## C.22: HIT FORMAT

-------------------------------------------------------------------

-- File:hit_format.vhd

-- Process block to locate the hits from the valid hits register

-- and give them to the format block to put them in the data format

-- Originally created by : Shweta Lolage

-- By Shweta Lolage on 05/17/2000 : Changes made according to the new approach

-- Changes are made to the design to combine the hit block and the format block

-- date : 09/04/2000 - Shweta Lolage

--------------------------------------------------------------------

library altera;

use altera.maxplus2.all;

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

--  Entity Declaration

entity hit_format is

port (clock,reset : in std_logic;

count : in unsigned (5 downto 0);

-- upper limit for the hit search as this is the number of comparators

-- loaded with valid road_data

hitreg: in std_logic_vector (45 downto 0);

-- valid hits register

l3_trailer : in unsigned (31 downto 0) ;

-- this trailer is passed on from the centroid finder block

pulsearea : in unsigned (1 downto 0);	-- pulse_area for the centroid

centroid : in unsigned (13 downto 0);	-- centroid which is compared

hitreg_valid : in std_logic;		-- read valid hit register signal

cen_done,hits_done : out std_logic;

--cen_done signal for the channel control block

output : out unsigned (32 downto 0);

-- the final output to the hits FIFO

fifo_enable,end_of_event  : out std_logic

-- signal to the main control block indicating that the --

-- channel has valid hits

);

end entity hit_format;

--  Architecture Body

architecture behavior of hit_format is

type hitstates is (sinit,spoint_select,sready,sread_hits,sread_bit,swrite_hit,

soutput_hit,snext_hit,strailer);

signal pstate,nstate : hitstates;

signal or1,or2,or3,or4,or5 : std_logic;

signal nor1,nor2,nor3,nor4,nor5 : std_logic;

signal por1,por2,por3,por4,por5 : std_logic;

signal nhits_present,phits_present,pcen_done,ncen_done : std_logic;

signal npointer,ppointer: unsigned (5 downto 0);

signal ncounter,pcounter: unsigned (5 downto 0);

signal nhitreg_valid,phitreg_valid : std_logic;

signal nhitreg, phitreg : std_logic_vector(45 downto 0);

signal nhits_done,phits_done,nbit,pbit : std_logic;

constant zero46 : std_logic_vector (45 downto 0) := "0000000000000000000000000000000000000000000000";

signal ndata,pdata: unsigned (32 downto 0);

signal ncount,pcount : unsigned (5 downto 0);

signal noutput,poutput : unsigned (32 downto 0);

signal nfifoenable,pfifoenable : std_logic;

signal npulsearea,ppulsearea : unsigned (1 downto 0);

signal pcentroid, ncentroid : unsigned (12 downto 0);

signal nevent_status, pevent_status : std_logic;

signal nl3_trailer,pl3_trailer : unsigned (31 downto 0);

constant zero33 : unsigned (32 downto 0) := "000000000000000000000000000000000";

constant zero32 : unsigned (31 downto 0) := "00000000000000000000000000000000";

constant zero22 : unsigned (21 downto 0) := "0000000000000000000000";

constant zero17 : unsigned (16 downto 0) := "00000000000000000";

constant zero16 : unsigned (15 downto 0) := "0000000000000000";

constant zero11 : unsigned (10 downto 0) := "00000000000";

constant zero8  : unsigned (7 downto 0)  := "00000000";

constant zero6  : unsigned (5 downto 0)  := "000000";

constant zero3  : unsigned (2 downto 0)  := "000";

constant two    : unsigned (1 downto 0)  := "10";

constant three  : unsigned (1 downto 0)  := "11";

begin

-- the initialization of the signals

npulsearea <= pulsearea;

ncentroid <= centroid(12 downto 0);

nevent_status <= centroid(13);

-- the bit tagged along with centroid indicating the end of event

nl3_trailer <= l3_trailer;

nhitreg_valid <= hitreg_valid;

-- this process block groups the comparators to reduce the search

process(hitreg)

variable tor1,tor2,tor3,tor4,tor5 : std_logic;

begin

tor1 := '0';

tor2 := '0';

tor3 := '0';

tor4 := '0';

tor5 := '0';

for i in 0 to 9 loop

tor1 := tor1 or hitreg(i);

end loop;

or1 <= tor1;

for i in 10 to 19 loop

tor2 := tor2 or hitreg(i);

end loop;

or2 <= tor2;

for i in 20 to 29 loop

tor3 := tor3 or hitreg(i);

end loop;

or3 <= tor3;

for i in 30 to 39 loop

tor4 := tor4 or hitreg(i);

end loop;

or4 <= tor4;

for i in 40 to 45 loop

tor5 := tor5 or hitreg(i);

end loop;

or5 <= tor5;

end process;

-- this process block is to find the hits and send them to the format block

process(ppointer,hitreg_valid,count,pcen_done,pcen_done,pcounter)

begin

ncen_done <= pcen_done;

ncounter  <= pcounter;

nor1  	  <= por1;

nor2  	  <= por2;

nor3  	  <= por3;

nor4  	  <= por4;

nor5  	  <= por5;

nhits_present <= phits_present;

npointer <= ppointer;

nhitreg <= phitreg;

nfifoenable <= pfifoenable;

noutput <= poutput;

nhits_done <= phits_done;

case pstate is

when sinit =>		-- initial state,

ncen_done <= '0';

nhits_present <= '0';

nfifoenable <= '0';

nhits_done <= '0';

if (hitreg_valid ='1') then	-- is valid register ready to be read

nstate <= spoint_select;

-- yes, go ahead and select the point of start

-- after determining whether we have any hits

nhitreg <= hitreg;

nor1  <= or1;

nor2  <= or2;

nor3  <= or3;

nor4  <= or4;

nor5  <= or5;

else

nstate <= sinit;

-- no be in this state and wait for it

nhitreg <= zero46;

nor1  <='0';

nor2  <='0';

nor3  <='0';

nor4  <='0';

nor5  <='0';

end if;

when spoint_select =>

nfifoenable <= '0';

nhits_present <= por1 or por2 or por3 or por4 or por5;

if (por1 ='1') then

npointer <= "000000";

else

if (por2 ='1') then

npointer <= "001010";

else

if (por3 ='1') then

npointer <= "010100";

else

if (por4='1') then

npointer <= "011110";

else

if (por5 = '1') then

npointer <= "101000";

else

npointer <= "000000";

end if;

end if;

end if;

end if;

end if;

nstate <= sready;

when sready =>   -- read the valid hits

nfifoenable <= '0';

if (phits_present = '1') then  -- are there any hits in this register

nstate <= sread_hits;		-- yes there are

ncen_done <= '0';

ncounter <= ppointer;

else

ncen_done <= '1';

nstate <= sinit;

end if;

when sread_hits =>

nstate     <= sread_bit;

nfifoenable <= '0';

case pcounter is

when "000000" => nbit <= phitreg(0);

when "000001" => nbit <= phitreg(1);

when "000010" => nbit <= phitreg(2);

when "000011" => nbit <= phitreg(3);

when "000100" => nbit <= phitreg(4);

when "000101" => nbit <= phitreg(5);

when "000110" => nbit <= phitreg(6);

when "000111" => nbit <= phitreg(7);

when "001000" => nbit <= phitreg(8);

when "001001" => nbit <= phitreg(9);

when "001010" => nbit <= phitreg(10);

when "001011" => nbit <= phitreg(11);

when "001100" => nbit <= phitreg(12);

when "001101" => nbit <= phitreg(13);

when "001110" => nbit <= phitreg(14);

when "001111" => nbit <= phitreg(15);

when "010000" => nbit <= phitreg(16);

when "010001" => nbit <= phitreg(17);

when "010010" => nbit <= phitreg(18);

when "010011" => nbit <= phitreg(19);

when "010100" => nbit <= phitreg(20);

when "010101" => nbit <= phitreg(21);

when "010110" => nbit <= phitreg(22);

when "010111" => nbit <= phitreg(23);

when "011000" => nbit <= phitreg(24);

when "011001" => nbit <= phitreg(25);

when "011010" => nbit <= phitreg(26);

when "011011" => nbit <= phitreg(27);

when "011100" => nbit <= phitreg(28);

when "011101" => nbit <= phitreg(29);

when "011110" => nbit <= phitreg(30);

when "011111" => nbit <= phitreg(31);

when "100000" => nbit <= phitreg(32);

when "100001" => nbit <= phitreg(33);

when "100010" => nbit <= phitreg(34);

when "100011" => nbit <= phitreg(35);

when "100100" => nbit <= phitreg(36);

when "100101" => nbit <= phitreg(37);

when "100110" => nbit <= phitreg(38);

when "100111" => nbit <= phitreg(39);

when "101000" => nbit <= phitreg(40);

when "101001" => nbit <= phitreg(41);

when "101010" => nbit <= phitreg(42);

when "101011" => nbit <= phitreg(43);

when "101100" => nbit <= phitreg(44);

when "101101" => nbit <= phitreg(45);

when others   => nbit <= '0';

end case;

when sread_bit =>

nfifoenable <= '0';

if (pbit = '1') then  -- no there are more

nstate <= swrite_hit;

ncount <= pcounter;

else

nstate <= snext_hit;

ncount <= pcount;

end if;

when swrite_hit =>

nstate <= soutput_hit;

nfifoenable <= '0';

ndata(32)           <= '0';

ndata(31 downto 26) <= pcount;

ndata(25 downto 24) <= ppulsearea;

ndata(23 downto 13) <= pl3_trailer(18 downto 8);

ndata(12 downto 0)  <= pcentroid;

when soutput_hit =>

nstate <= snext_hit;

noutput<= pdata;

nfifoenable <= '1';

when snext_hit =>

-- state to decide whether we should go ahead for the next hit

nfifoenable <= '0';

if (pcounter <= count) then

-- have we reached the end of the comparators

ncen_done <='0';

ncounter <= pcounter + 1;		-- No , the end has not yet come

nstate <= sread_hits;

else

ncen_done  <= '1';

ncounter <= "000000";

if (pevent_status = '0')then

nstate <= sinit;

-- yes go to start and wait for the next lot

else

nstate <= strailer;

nhits_done <= '1';

ndata (32)          <= '1';

ndata(31 downto 0)  <= pl3_trailer;

end if;

end if;

when strailer =>

nstate <= sinit;

nfifoenable <= '1';

noutput <= pdata;

nhits_done <= '0';

when others =>

-- any other state other than the once defined above, go to the initial state

nstate <= sinit;

ncen_done <= '0';

nhits_done <= '0';

end case;

end process;

--this process block registers the signals

process(clock,reset)

begin

if (reset ='0') then

ppointer  <="000000";

pcounter <="000000";

phits_present <= '0';

pstate <= sinit;

pcen_done <= '0';

pbit <= '0';

phitreg_valid <= '0';

por1  <= '0';

por2  <= '0';

por3  <= '0';

por4  <= '0';

por5  <= '0';

phitreg <= zero46;

elsif (clock'event and clock ='1') then

ppointer      <= npointer;

pcounter      <= ncounter;

phits_present <= nhits_present;

pstate        <= nstate;

pcen_done     <= ncen_done;

pbit          <= nbit;

phitreg_valid <= nhitreg_valid;

por1          <= nor1;

por2   		  <= nor2;

por3  		  <= nor3;

por4  	      <= nor4;

por5  		  <= nor5;

phitreg       <= nhitreg;

end if;

end process;

-- the process block to register the signals

process(clock,reset)

begin

if(reset ='0') then

poutput <= zero33;

pcount <= zero6;

pdata <= zero33;

ppulsearea <= "00";

pcentroid <= "0000000000000";

pevent_status <= '0';

pl3_trailer <= zero32;

pfifoenable <= '0';

phits_done <= '0';

elsif( clock'event and clock='1') then

poutput <= noutput;

pcount <= ncount;

pdata <= ndata;

ppulsearea <= npulsearea;

pcentroid <= ncentroid;

pl3_trailer <= nl3_trailer;

pevent_status <= nevent_status;

pfifoenable <= nfifoenable;

phits_done <= nhits_done;

end if;

end process;

-- final out to the fifo, this block is not totally complete as will have to see the handshaking with the fifo

output <= poutput;

fifo_enable <= pfifoenable;

end_of_event <= poutput(32);

cen_done  <= pcen_done;

hits_done <= phits_done;

end architecture;

## C.23: HIT READ OUT

-------------------------------------------------------------------

-- File:hit_readout.vhd

-- Process block to read the hits from the FIFO by the control block

-- Originally created by : Shweta Lolage :08/01/2000

--------------------------------------------------------------------

library altera;

use altera.maxplus2.all;

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

--  Entity Declaration

entity hit_readout is

port (clock, reset : in std_logic;

hits_enable,hit_fifo_empty, hits_busy : in std_logic;

read_hit_fifo,hits_available : out std_logic

);

end entity hit_readout;

--  Architecture Body

architecture behavior of hit_readout is

signal nread_hit_fifo,pread_hit_fifo : std_logic;

begin

process( hits_enable, hit_fifo_empty)

begin

if(hits_enable = '1' and hit_fifo_empty = '0') then

nread_hit_fifo <= '1';  -- this is  read_req signal to the fifo

else

nread_hit_fifo <= '0';

end if;

end process;

hits_available <=  hits_busy;

-- this is an internal signal to the read_out control to indicate that hits are being processed

reg : process ( clock, reset)

begin

if(reset = '0') then

pread_hit_fifo <= '0';

elsif (clock'event and clock ='1') then

pread_hit_fifo <= nread_hit_fifo;

end if;

end process reg;

-- assigning the outputs

read_hit_fifo <= pread_hit_fifo;

end architecture;

## C.24: TRAILER PATTERN RECOGINSER

-------------------------------------------------------------------

-- File:trailer_pat.vhd

-- Process block to check for the trailer

-- Originally created by : Shweta Lolage :08/01/2000

--------------------------------------------------------------------

library altera;

use altera.maxplus2.all;

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

--  Entity Declaration

entity trailer_pat is

port (stream : in unsigned(4 downto 0);

trailer_signal : out std_logic

);

end entity trailer_pat;

--  Architecture Body

architecture behavior of trailer_pat is

constant pattern : unsigned (4 downto 0):= "11110";

begin

with stream select

trailer_signal <= '1' when pattern,

'0' when others;

end architecture;

C.25 DETAILED MEMORY ALLOCATION OF THE DOWNLOADED PARAMETERS

- Monitor space: 1K memory: 0000 – 03FF.

- Miscellaneous: 1K memory: 0400 – 07FF.

- Gain Offset LUT: 4K X 8:    0800 – 17FF

- Test data LUT: 1K (default) X 18: 1800 – 1BFF.

- Empty Space: 1C00 – 3FFF.

- Road data LUT: 16 K X 22: 4000 – 7FFF.

### C.25.1 The monitoring data memory allocation

- SERR,mismatch,SMT_ID,SEQ_ID,HDI_ID,data (last data word read):

address 0000.

- SERR error count : address 0001.

- Mismatch counter: mismatch in the SEQ-ID or the HDI-ID: address 0002.

- Zero error count: Zero byte not found: address 0003.

- Number of undefined data_type clusters: address 0010.

- Number of axial data_type clusters: address 0011.

- Number of stereo data_type clusters: address 0012.

- Number of Z-axis data_type clusters: address 0013.

- Data activity on Chip(I): address 0020-0028.

### C.25.2 The miscellaneous memory space memory allocation

- Bad channel: 32 X 64: address: 0400 – 047F.

- Chip ranges: 24 X 1: address: 0480.

- Pulse area thresholds: 24 X 4 X4: address: 0500 – 0503.

- Miscellaneous parameters: 32 X 1: address: 0580.

- Cluster threshold values: 17 X 4: address: 0600 – 0603.