# APPENDIX C

VHDL CODE OF THE STC MODULES

Hit-Filter Interface

-- Version 0 This block is used to interface to the Hit Filter

------------------------------------------------------------------------------

-- Initial Design: Reginald Perry  (12/15/2000)

------------------------------------------------------------------------------

--  Modified

-- 7/28/2001  Fix start_centroids

-- 7/30/2001 Arvindh Lalam	Making changes to get proper Data_valid waveform.

-- 6/10/2002 Arvindh Lalam	Forcing this module to check for bus availability before

--							reading out each word.

------------------------------------------------------------------------------

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

--------------------------------------------------------------------------------------------

-- Entity Declaration : Defines Inputs and Outputs of the device.

--------------------------------------------------------------------------------------------

entity hit_filter_interface is

port (clk,reset,hits_available,zvcs_available:in std_logic;

end_of_hits,end_of_zvcs,end_of_l3_event: in std_logic;

hits_in,centroids_in:in std_logic_vector(31 downto 0);

start_hits,start_centroids: in std_logic;

hc_inh: in std_logic_vector(6 downto 0);

event_start_int: in std_logic;

end_of_hit_event: in std_logic;

--

--       Hits busy is actually hits bus request

--

hits_read_req,hc_busy,hits_output_enablen: out std_logic;

hc_data_out: out std_logic_vector(31 downto 0);

hc_wr, hdone, cdone,dv,zvcs_read_req,event_busy: out std_logic;

hit_filter_ostate: out std_logic_vector(3 downto 0)

);

end entity hit_filter_interface;

----------------------------------------------------------------------------------------------

--Architecture body

-----------------------------------------------------------------------------------------------

architecture logic of hit_filter_interface is

type mystates is (sreset,swait_for_start,swait_for_hits,swait_for_centroids,swait_for_bus,

swrite_hits,swrite,sdummy_wait);

signal ndv,pdv,nhdone,phdone,ncdone,pcdone: std_logic;

signal nhc_busy,phc_busy: std_logic;

signal pevent_busy, nevent_busy: std_logic;

signal nhits_output_enablen, phits_output_enablen,nhwr,phwr: std_logic;

signal bus_available,nctype,pctype: std_logic;

signal ns,ps: mystates;

constant hits_type: std_logic := '0';

constant zvcs_type: std_logic := '1';

begin

----------------------------------------------------------------------------------

-- This WITH SELECT is used to extract the current state of the Finite State Machine

-----------------------------------------------------------------------------------

with ps select

hit_filter_ostate <= "0000" when sreset,

"0001" when swait_for_start,

"0010" when swait_for_hits,

"0011" when swait_for_centroids,

"0100" when swait_for_bus,

"0101" when swrite_hits,

"0110" when swrite,

"0111" when sdummy_wait,

"1111" when others;

------------------------------------------------------------------------------------

--   This process block ORs all the "inhibit"s to determine if the bus is

---  available

------------------------------------------------------------------------------------

process(hc_inh)

variable i: integer;

variable temp: std_logic;

begin

temp := '0';

for i in 0 to 6 loop

temp := temp or hc_inh(i);

end loop;

bus_available <= not temp;

end process;

-------------------------------------------------------------------------------------

--  This0 process block controls the interface lines

---------------------------------------------------------------------------------------

process(ps,start_hits,start_centroids,bus_available,pctype,zvcs_available,hits_available,

phdone,pcdone,pdv,phwr,phc_busy,phits_output_enablen,end_of_hits,end_of_zvcs)

begin

nhdone <= phdone;

ncdone <= pcdone;

ndv <= pdv;

nhwr <= phwr;

nhc_busy <= phc_busy;

hits_read_req <= '0';

zvcs_read_req <= '0';

nhits_output_enablen <=phits_output_enablen;

nctype <= pctype;

----------------------------------------------------

--      This is the main FSM

----------------------------------------------------

case ps is

----------------------------------------------------

--  SRESET : This state sets the correct values for all outputs after the initial reset

----------------------------------------------------

when sreset =>

ns <= swait_for_start;

nhdone <= '1';

ncdone <= '1';

ndv <= '1';

nhwr <= '0';

nhc_busy <= '0';

nhits_output_enablen <= '1';

nctype <= pctype;

---------------------------------------------------

--      Wait for hits or centroids to be available

----------------------------------------------------

when swait_for_start =>

ns <= swait_for_start;

nhdone <= phdone;

ncdone <= pcdone;

ndv <= pdv;

nhwr <= phwr;

nhc_busy <= phc_busy;

hits_read_req <= '0';

zvcs_read_req <= '0';

nhits_output_enablen <=phits_output_enablen;

-----------------------------------------------------

-- Looking for start_hits or start_centroids.  Assuming that can't occur

-- simulataneously

-----------------------------------------------------

if(start_hits = '1') then

nhdone <= '0';

ncdone <= pcdone;

ndv <= '0';

ns <= swait_for_hits;

nctype <= hits_type;

elsif(start_centroids = '1') then

nhdone <= phdone;

ncdone <= '0';

ndv <= '0';

ns <= swait_for_centroids;

nctype <= zvcs_type;

else

nhdone <= '1';

ncdone <= '1';

ndv <= '1';

ns <= swait_for_start;

end if;

----------------------------------------------------

--  When Control logic wants to read hits,

--  This state Checks if hits are available

----------------------------------------------------

when swait_for_hits =>

nhits_output_enablen <= '1';

nhwr <= '0';

nhc_busy <= '0';

nhdone <= '0';

ncdone <= pcdone;

hits_read_req <= '0';

zvcs_read_req <= '0';

nctype <= hits_type;

----------------------------------------------------

-- IF hits are available -THEN request the bus

-- 		  ELSE wait here until we have hits

----------------------------------------------------

if(hits_available = '1') then

nhc_busy <= '1';

ns <= swait_for_bus;

else

nhc_busy <= '0';

ns <= swait_for_hits;

end if;

----------------------------------------------------

--    Check if z centroids are available

----------------------------------------------------

when swait_for_centroids  =>

nhits_output_enablen <= '1';

nhwr <= '0';

nhc_busy <= '0';

ncdone <= '0';

nhdone <= phdone;

zvcs_read_req <= '0';

hits_read_req <= '0';

nctype <= zvcs_type;

----------------------------------------------------

-- IF zvcs_centroids  are available THEN request the bus

--			ELSE wait in this state

----------------------------------------------------

if(zvcs_available = '1') then

nhc_busy <= '1';

ns <= swait_for_bus;

else

nhc_busy <= '0';

ns <= swait_for_centroids;

end if;

----------------------------------------------------

--   We have hits or zvcs, now we are waiting for the bus

----------------------------------------------------

when swait_for_bus=>

ncdone <= pcdone;

nhdone <= phdone;

nhits_output_enablen <= '1';

hits_read_req <= '0';

zvcs_read_req <= '0';

nhc_busy <= '1';    --  keep requesting the bus

----------------------------------------------------

--   IF the bus is available THEN upload hits ?

--		ELSE wait in this state

----------------------------------------------------

if(bus_available = '1') then

nhc_busy <= '1';

ns <= swrite_hits;

end if;

----------------------------------------------------

-- Reads hits from the internal HIT-FIFOs

----------------------------------------------------

when swrite_hits =>

ndv <= '1';		-- Setting Data_valid signal to indicate valid hit word.

-- Valid data word appears before end of this clock cycle.

if(pctype = hits_type) then

hits_read_req <= '1';

-- asynchronous, thus Data should appear on the next cycle

zvcs_read_req <= '0';

else

hits_read_req <= '0';

zvcs_read_req <= '1';  -- asynchronous, thus Data should appear on the next cycle

end if;

nhits_output_enablen <= '0'; --  Turn on output bus.

nhwr <= '0';

ns <= sdummy_wait;

----------------------------------------------------

-- dummy state to allow data to appear on the bus

----------------------------------------------------

when sdummy_wait =>

ns <= swrite;

hits_read_req <= '0';   -- bring read request low

zvcs_read_req <= '0';

nhits_output_enablen <= '0'; --   keep buffer on

nhwr  <= '1';    -- the data will appear written into control logic during next cycle

----------------------------------------------------

-- Data is available on the bus, and being written

----------------------------------------------------

when swrite =>   --- write into control logic during this state

hits_read_req <= '0';

zvcs_read_req <= '0';

nhits_output_enablen <= '0';

nhwr <= '0';    --  turn off hc wr line

nhc_busy <= '1'; --   keep bus

----------------------------------------------------

--  			Checking for more hits/Centroids

----------------------------------------------------

case pctype is

when hits_type =>

if(hits_available = '1') then

hits_read_req <= '1';    --  bring read request line high

zvcs_read_req <= '0';

ns <=swait_for_bus;  -- go write the next word

else

----------------------------------------------------

-- 		No more hits, Give up the bus

--        		Turn off output buffers and then release bus on next cycle

----------------------------------------------------

nhits_output_enablen <= '1';

nhc_busy <= '0';

----------------------------------------------------

--    If at the end of hits THEN, reset all values

--		ELSE wait for more hits

----------------------------------------------------

if(end_of_hits = '1') then

ns <= swait_for_start;

nhdone <= '1';

nhwr <= '0';

nhc_busy <= '0';

nhits_output_enablen <= '1';

else

ns <= swait_for_hits;

nhdone <= '0';

nhwr <= '0';

nhc_busy <= '0';

nhits_output_enablen <= '1';

end if;

end if;

when zvcs_type =>

if(zvcs_available = '1') then

hits_read_req <= '0';    --  bring read request line high

zvcs_read_req <= '1';

ns <=sdummy_wait;  -- go write the next word

else

----------------------------------------------------

-- 		No more Centroids, Give up the bus

--        		Turn off output buffers and then release bus on next cycle

----------------------------------------------------

nhits_output_enablen <= '1';

nhc_busy <= '0';

----------------------------------------------------

--    If at the end of centroids THEN, reset all values

--			ELSE wait for more centroids

----------------------------------------------------

if(end_of_zvcs = '1') then

ns <= swait_for_start;

ncdone <= '1';

nhwr <= '0';

nhc_busy <= '0';

nhits_output_enablen <= '1';

else

ns <= swait_for_centroids;

ncdone <= '0';

--               ndv <= '0';

nhwr <= '0';

nhc_busy <= '0';

nhits_output_enablen <= '1';

end if;

end if;

when others =>

null;

end case;  -- case pctype

when others =>

null;

end case;   --- case ps

end process;

------------------------------------------------------------------------------------------

--  Register process block

------------------------------------------------------------------------------------------

process(clk,reset,ns, ndv,nhdone,ncdone,nhits_output_enablen,nhwr,nhc_busy,nctype)

begin

if(reset = '0') then

ps <= sreset;

pdv <= '0';

phdone <= '0';

pcdone <= '0';

phits_output_enablen <= '0';

phwr <= '0';

pevent_busy <= '0';

phc_busy <= '0';

pctype <= '0';

elsif(clk'event and clk = '1') then

ps <= ns;

pdv <= ndv;

phdone <= nhdone;

pcdone <= ncdone;

phits_output_enablen <= nhits_output_enablen;

phwr <= nhwr;

pevent_busy <= nevent_busy;

phc_busy <= nhc_busy;

pctype <= nctype;

end if;

end process;

------------------------------------------------------------------------------------------

--    This process block determines event_busy

--    It goes high when event_start_int goes high

--    It goes low with end of hits

------------------------------------------------------------------------------------------

process(event_start_int,end_of_hit_event,end_of_l3_event,pevent_busy)

variable end_of_event: std_logic;

begin

--------------------------------------------

--    Check for end_of_event, if we are currently in an event otherwise assume default

--     condition is we have completed the previous event.

--------------------------------------------

end_of_event := end_of_hit_event and end_of_l3_event;

if(pevent_busy = '1') then

--------------------------------------------

--   we are current in an event.  Have we finished?

--   Are we at end_of_event?

--------------------------------------------

if(end_of_event = '1') then

nevent_busy <= '0';     -- yes.Reset, event_busy

else

nevent_busy <= '1'; -- no. Keep event_busy high

end if;

else

--------------------------------------------

--   			ck for event start

--------------------------------------------

if(event_start_int = '1') then

nevent_busy <= '1';    --  yes, set event_busy

else

nevent_busy <= '0';     -- no, reset event busy

end if;

end if;

end process;

------------------------------------------------------------------------------------------

--    Use multiplexer to select between hits and centroids

------------------------------------------------------------------------------------------

with pctype select

hc_data_out <=  hits_in when hits_type,

centroids_in when zvcs_type,

hits_in when others;

-- attach registers to outputs

hits_output_enablen <= phits_output_enablen;

dv <= pdv;

hc_busy <= phc_busy;

hdone <= phdone;

cdone <= pcdone;

hc_wr <= phwr;

event_busy <= pevent_busy;

end architecture;

Road-Set Generator:

-------------------------------------------------------------------------------

--  File 	: Road_Set_Generator.vhd

--  Created on 01/22/02 by Arvindh Lalam.

--	Modified on 01/26/02 by Arvindh Lalam.

--			Adding estimation of 'words with don't cares' from UPPER ROAD

-------------------------------------------------------------------------------

-- 	Author	: Vindi Lalam

--	1. Based on algorithm discussed in the Thesis work.

--	2. Generates road-sets for given roads and stores them in a CAM

--  3. Data words belonging to the road-set may contain 'DON'T CARES'

--	4. It takes a maximum of 50 Cycles to generate coded words for a given road.

-----------------------------------------------------------------------

--	clk				IN	: Clock

--	reset			IN	: Reset

--	min[6..0]		IN	: Lower road

--	max[6..0]		IN	: Upper road

--	activate		IN 	: Activates this module

--	read_in			IN	: Read signal for checking if data exits in CAM

-- 	centroid[6..0]	IN	: centroid calculated by Centroid finder.

--

--	cam_wr			OUT	: Cam write signal

--	cam_rd			OUT	: Latched Read signal (required to sync with centroid)

--	cam_word[6..0]	OUT	: output word

--	wrx_word[6..0]	OUT	: wrx for don't care.

--	adrr[4..0]		OUT	: CAM address.

--	ostate[4..0]	OUT	: FSM state.

-- 	err				OUT	: Indicates err in 'CASE STATEMENT'

--	return_state_out[4..0]	OUT	: Brings out internal 'return_state'

-----------------------------------------------------------------------

--

-----------------------------------------------------------------------

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

-------------------------------------------------------------------------------

-- Device ports

-------------------------------------------------------------------------------

entity road_set_generator is

port	( 	clk		: in std_logic;

reset	: in std_logic;

min		: in unsigned(6 downto 0);

max		: in unsigned(6 downto 0);

activate	: in std_logic;

read_in		: in std_logic;

centroid	: in unsigned(6 downto 0);

cam_wr		: out std_logic;

cam_rd		: out std_logic;

cam_word	: out unsigned(6 downto 0);

wrx_word	: out unsigned(6 downto 0);

addr		: out unsigned(4 downto 0);

ostate		: out std_logic_vector(4 downto 0);

return_state_out	: out std_logic_vector(4 downto 0);

err			: out std_logic);

end road_set_generator;

-------------------------------------------------------------------------------

-- Architecture Body Begins

-------------------------------------------------------------------------------

architecture behaviour of road_set_generator is

type mystates is (sreset, sinitialize, sbit0, sbit1, sbit2, sbit3, sbit4,

sbit5, sbit6, sbit0_rev, sbit1_rev, sbit2_rev, sbit3_rev,

sbit4_rev, sbit5_rev, sbit6_rev, sread, swrite, sdummy1,

sdecide);

signal ns, ps, nreturn_state, preturn_state	: mystates;

signal ncam_wr, pcam_wr, ncam_rd, pcam_rd	: std_logic;

signal ncam_word, pcam_word, nmin, pmin, nmax, pmax, nwrx_word,

pwrx_word : unsigned (6 downto 0);

signal ntemp, ptemp		: unsigned(6 downto 0);

signal nstatus,pstatus	: unsigned(6 downto 0);

signal naddr, paddr		: unsigned(4 downto 0);

constant zero7: unsigned(6 downto 0) := "0000000";

constant zero8: unsigned(7 downto 0) := "00000000";

begin

-------------------------------------------------------------------------------

-- Starting the Process

-------------------------------------------------------------------------------

process (pmin, pmax, pcam_word, pwrx_word, activate, ntemp, ptemp, pcam_wr,

pcam_rd, clk, pstatus, ps, max, min, preturn_state, paddr, read_in,

centroid)

variable diff		:unsigned(7 downto 0);

variable diff_xor	:unsigned(6 downto 0);

variable final_state, final_state_rev:mystates;

variable i			: integer;

begin

-------------------------------------

-- Initialization in the FSM

-------------------------------------

ns 			<= ps;

ncam_word 	<= pcam_word;

nmin		<= pmin;

nmax		<= pmax;

nwrx_word	<= pwrx_word;

ncam_wr		<= pcam_wr;

ncam_rd		<= pcam_rd;

nstatus		<= pstatus;

nreturn_state <= preturn_state;

ntemp		<= ptemp;

naddr		<= paddr;

err <= '0';

diff(7) := '0';

-------------------------------------

-- Finds the range of words. diff_xor <= MAX (xor) MIN

-- starting at bit 0, all bits after the first occurrence of 1 are made '1'

-- 0010101 <= 0010010 (xor) 0000111    ; 0011111 ;

-------------------------------------

for i in 0 to 6 loop

diff_xor(6-i) := min(6-i) xor max(6-i);

diff(6-i) := diff(7-i) or diff_xor(6-i);

end loop;

-------------------------------------

-- Find the highest-bit and thus the last state to go into:

-- a value of 0011111 indicates final_state = sbit4 AND

--								final_state_rev = sbit4_rev

-------------------------------------

case pstatus is

when "0000001" =>

final_state := sbit0;

final_state_rev := sbit0_rev;

when "0000011" =>

final_state := sbit1;

final_state_rev := sbit1_rev;

when "0000111" =>

final_state := sbit2;

final_state_rev := sbit2_rev;

when "0001111" =>

final_state := sbit3;

final_state_rev := sbit3_rev;

when "0011111" =>

final_state := sbit4;

final_state_rev := sbit4_rev;

when "0111111" =>

final_state := sbit5;

final_state_rev := sbit5_rev;

when "1111111" =>

final_state := sbit6;

final_state_rev := sbit6_rev;

when others	=>

final_state := sbit6;

final_state_rev := sbit6_rev;

end case;

-------------------------------------

-- Redirect : 	If ps = final_state,

--				THEN start from UPPER_ADDRESS.

--				ELSE continue

-------------------------------------

if(ps = final_state) then

ns <= sbit0_rev;

ntemp <= pmax;

naddr <= "00110";

else

case ps is

-------------------------------------

-- RESET State: Initializes all the values

-------------------------------------

when sreset =>

nmin <= zero7;

nmax <= zero7;

ntemp <= zero7;

ncam_word <= zero7;

nwrx_word <= zero7;

ncam_wr <= '0';

ncam_rd	<= '0';

ns	 <= sinitialize;

nstatus	<= zero7;

err <= '0';

nreturn_state <= sreset;

naddr	<= "00000";

-------------------------------------

-- SINITIALIZE : Stays in this state until activate = '1',

--							or read_in = '1'.

-------------------------------------

when sinitialize =>

if (activate='1') then			-- Goto state SBIT0

nmin <= min;

ntemp <= min;

nmax <= max;

ncam_word	<= zero7;

nwrx_word	<= zero7;

ns <= sbit0;

ncam_wr	<= '0';

ncam_rd	<= '0';

nstatus	<= diff(6 downto 0);

else

nmin <= pmin;

nmax <= pmax;

ntemp <= ptemp;

nwrx_word	<= pwrx_word;

ns <= sinitialize;

ncam_wr <= pcam_wr;

nstatus <= pstatus;

if(read_in='1') then 		-- Goto state SREAD

ncam_word <= centroid;

ns <= sread;

ncam_rd	<= '1';

else			-- Wait for change in 'activate'/'read_in'

ncam_word <= pcam_word;

ns <= sinitialize;

ncam_rd <= '0';

end if;

end if;

-------------------------------------

-- Estimate CAM words starting from LOWER_ADDRESS,

-- to represent the given range DONT CARES are used to represent ranges.

-------------------------------------

-- BIT0	: Checks bit'0'.

-------------------------------------

when sbit0 =>

ns <= swrite;

nreturn_state <= sbit1;

if (ptemp(0) = '1') then

ntemp <= ptemp + "0000001";

ncam_word <= ptemp;

nwrx_word <= zero7;

ncam_wr <= '1';

else

ns <= sbit1;

end if;

-------------------------------------

-- BIT1 	: Checks bit 1.

-------------------------------------

when sbit1 =>

ns <= swrite;

nreturn_state <= sbit2;

if (ptemp(1) = '1') then

ntemp <= ptemp + "0000010";

ncam_word <= ptemp;

nwrx_word <= "0000001";

ncam_wr <= '1';

else

ns <= sbit2;

end if;

-------------------------------------

-- BIT2 	: Check bit2

-------------------------------------

when sbit2 =>

ns <= swrite;

nreturn_state <= sbit3;

if (ptemp(2) = '1') then

ntemp <= ptemp + "0000100";

ncam_word <= ptemp;

nwrx_word <= "0000011";

ncam_wr <= '1';

else

ns <= sbit3;

end if;

-------------------------------------

-- BIT3	: Check bit3

-------------------------------------

when sbit3 =>

ns <= swrite;

nreturn_state <= sbit4;

if (ptemp(3) = '1') then

ntemp <= ptemp + "0001000";

ncam_word <= ptemp;

nwrx_word <= "0000111";

ncam_wr <= '1';

else

ns <= sbit4;

end if;

-------------------------------------

-- BIT4 	: Check bit4

-------------------------------------

when sbit4 =>

ns <= swrite;

nreturn_state <= sbit5;

if (ptemp(4) = '1') then

ntemp <= ptemp + "0010000";

ncam_word <= ptemp;

nwrx_word <= "0001111";

ncam_wr <= '1';

else

ns <= sbit5;

end if;

-------------------------------------

-- BIT5 	: Check bit5

-- 	This state behaves differently from others SBIT_ states.:

--	Going through next state(sbit6) is redundant, thus..

--	Redirect to SBIT0_REV; Reinitialize PTEMP to UPPER_ADDRESS (pmax).

-------------------------------------

when sbit5 =>

ns <= swrite;

nreturn_state <= sbit0_rev;

ntemp <= pmax;

if (ptemp(5) = '1') then

ncam_word <= ptemp;

nwrx_word <= "0011111";

ncam_wr <= '1';

else

ns <= sbit0_rev;

end if;

-------------------------------------

-- SBIT0_REV	: Start from Bit'0'.

-------------------------------------

when sbit0_rev =>

ns <= swrite;

nreturn_state <= sbit1_rev;

ncam_word <= ptemp;

if (ptemp(0) = '0') then

ntemp <= ptemp - "0000001";

nwrx_word <= zero7;

ncam_wr <= '1';

else

ns <= sbit1_rev;

end if;

-------------------------------------

-- In the last state, then check if bit ='1'

-------------------------------------

if (ps = final_state_rev) then

nreturn_state <= sreset;

if(ptemp(0) = '1') then

ntemp <= ptemp;

nwrx_word <= "0000001";

ncam_wr <= '1';

ns <= swrite;

else

ntemp <= ptemp;

ncam_wr <= '0';

ns <= sdecide;

end if;

end if;

-------------------------------------

-- SBIT1_REV	: Start from Bit'1'.

-------------------------------------

when sbit1_rev =>

ns <= swrite;

nreturn_state <= sbit2_rev;

ncam_word <= ptemp;

if (ptemp(1) = '0') then

ntemp <= ptemp - "0000010";

nwrx_word <= "0000001";

ncam_wr <= '1';

else

ns <= sbit2_rev;

end if;

if (ps = final_state_rev) then

nreturn_state <= sreset;

if(ptemp(1) = '1') then

ntemp <= ptemp;

nwrx_word <= "0000011";

ncam_wr <= '1';

ns <= swrite;

else

ntemp <= ptemp;

ncam_wr <= '0';

ns <= sdecide;

end if;

end if;

-------------------------------------

-- SBIT2_REV	: Start from Bit'2'.

-------------------------------------

when sbit2_rev =>

ns <= swrite;

nreturn_state <= sbit3_rev;

ncam_word <= ptemp;

if (ptemp(2) = '0') then

ntemp <= ptemp - "0000100";

nwrx_word <= "0000011";

ncam_wr <= '1';

else

ns <= sbit3_rev;

end if;

if (ps = final_state_rev) then

nreturn_state <= sreset;

if(ptemp(2) = '1') then

ntemp <= ptemp;

nwrx_word <= "0000111";

ncam_wr <= '1';

ns <= swrite;

else

ntemp <= ptemp;

ncam_wr <= '0';

ns <= sdecide;

end if;

end if;

-------------------------------------

-- SBIT3_REV	: Start from Bit'3'.

-------------------------------------

when sbit3_rev =>

ns <= swrite;

nreturn_state <= sbit4_rev;

ncam_word <= ptemp;

if (ptemp(3) = '0') then

ntemp <= ptemp - "0001000";

nwrx_word <= "0000111";

ncam_wr <= '1';

else

ns <= sbit4_rev;

end if;

if (ps = final_state_rev) then

nreturn_state <= sreset;

if(ptemp(3) = '1') then

ntemp <= ptemp;

nwrx_word <= "0001111";

ncam_wr <= '1';

ns <= swrite;

else

ntemp <= ptemp;

ncam_wr <= '0';

ns <= sdecide;

end if;

end if;

-------------------------------------

-- SBIT4_REV	: Start from Bit'4'.

-------------------------------------

when sbit4_rev =>

ns <= swrite;

nreturn_state <= sbit5_rev;

ncam_word <= ptemp;

if (ptemp(4) = '0') then

ntemp <= ptemp - "0010000";

nwrx_word <= "0001111";

ncam_wr <= '1';

else

ns <= sbit5_rev;

end if;

if (ps = final_state_rev) then

nreturn_state <= sreset;

if(ptemp(4) = '1') then

ntemp <= ptemp;

nwrx_word <= "0011111";

ncam_wr <= '1';

ns <= swrite;

else

ntemp <= ptemp;

ncam_wr <= '0';

ns <= sdecide;

end if;

end if;

-------------------------------------

-- SBIT5_REV	: Start from Bit'5'.

-------------------------------------

when sbit5_rev =>

ns <= swrite;

nreturn_state <= sbit6_rev;

ncam_word <= ptemp;

if (ptemp(5) = '0') then

ntemp <= ptemp - "0100000";

nwrx_word <= "0011111";

ncam_wr <= '1';

else

ns <= sbit6_rev;

end if;

if (ps = final_state_rev) then

nreturn_state <= sreset;

if(ptemp(5) = '1') then

ntemp <= ptemp;

nwrx_word <= "0111111";

ncam_wr <= '1';

ns <= swrite;

else

ntemp <= ptemp;

ncam_wr <= '0';

ns <= sdecide;

end if;

end if;

-------------------------------------

-- SBIT6_REV	: Start from Bit'6'

-- If the FSM comes to this state, it may mean that either all

--		the bits are DON'T CARES (if this bit = '1')

-------------------------------------

when sbit6_rev =>

nreturn_state <= sreset;

ncam_word <= ptemp;

if(ptemp(6) = '1') then

ntemp <= ptemp;

nwrx_word <= "1111111";

ncam_wr <= '1';

ns <= swrite;

else

ntemp <= ptemp;

ncam_wr <= '0';

ns <= sdecide;

end if;

-------------------------------------

-- SREAD	: Start from Bit'0'.

-------------------------------------

when sread =>

ns <= sdummy1;

ncam_rd <= '0';

nreturn_state <= preturn_state;

-------------------------------------

-- SWRITE	: Store the ROAD-SET word.

-------------------------------------

when swrite	=>

ns <= sdummy1;

nreturn_state <= preturn_state;

when sdummy1 =>

ns <= sdecide;

nreturn_state <= preturn_state;

-------------------------------------

-- SDECIDE	: find if this is the first or the second of the

---			two road-sets that can be stored in each ESB

-------------------------------------

when sdecide =>

if (preturn_state = sreset and paddr(4)='0') then

ns <= sinitialize;

naddr <= "10000";

else

ns <= preturn_state;

naddr <= paddr + 1;

end if;

ncam_wr <= '0';

when others=>

err <= '1';

ns <= sreset;

end case;

end if;

end process;

-----------------------------------------------------------------

-- Register Block

-----------------------------------------------------------------

process (clk, reset, nmin, nmax, ntemp, ncam_word, ncam_wr, ncam_rd,

ns, nreturn_state, naddr)

begin

if (reset='0') then

pmin <= zero7;

pmax <= zero7;

ptemp <= zero7;

pcam_word <= zero7;

pwrx_word <= zero7;

pcam_wr <= '0';

pcam_rd <= '0';

ps	 <= sreset;

pstatus <= zero7;

preturn_state <= sreset;

paddr	<= "00000";

elsif(clk='1' and clk'event) then

pmin <= nmin;

pmax <= nmax;

ptemp <= ntemp;

pcam_word	<= ncam_word;

pwrx_word	<= nwrx_word;

pcam_wr <= ncam_wr;

pcam_rd <= ncam_rd;

ps <= ns;

pstatus <= nstatus;

preturn_state <= nreturn_state;

paddr	<= naddr;

end if;

end process;

-------------------------------------

-- Connect Output ports to the register outuputs.

-------------------------------------

cam_wr <= pcam_wr;

cam_rd <= pcam_rd;

cam_word <= pcam_word;

wrx_word <= pwrx_word;

addr	<= paddr;

-------------------------------------

-- The encoded state for debugging purposes

-------------------------------------

with ps select

ostate <= 	"00000" when sreset,

"00001" when sinitialize,

"00010" when sbit0,

"00011" when sbit1,

"00100" when sbit2,

"00101" when sbit3,

"00110" when sbit4,

"00111" when sbit5,

"01000" when sbit6,

"01001" when sbit0_rev,

"01010" when sbit1_rev,

"01011" when sbit2_rev,

"01100" when sbit3_rev,

"01101" when sbit4_rev,

"01110" when sbit5_rev,

"01111" when sbit6_rev,

"10000" when sread,

"10001" when swrite,

"10010" when sdummy1,

"10011" when sdecide,

"11111" when others;

-------------------------------------

-- The encoded "return state" for debugging purposes

-------------------------------------

with preturn_state select

return_state_out <="00000" when sreset,

"00001" when sinitialize,

"00010" when sbit0,

"00011" when sbit1,

"00100" when sbit2,

"00101" when sbit3,

"00110" when sbit4,

"00111" when sbit5,

"01000" when sbit6,

"01001" when sbit0_rev,

"01010" when sbit1_rev,

"01011" when sbit2_rev,

"01100" when sbit3_rev,

"01101" when sbit4_rev,

"01110" when sbit5_rev,

"01111" when sbit6_rev,

"10000" when swrite,

"10001" when sdummy1,

"10010" when sdecide,

"11111" when others;

end behaviour;

Comparator Block

----------------------------------------------------------------------------

--File:comparator.vhd

-- To compare the strip pointer with the data value from the

-- road data

-- Originally created by : Shweta Lolage

-- 05/15/2000 Shweta Lolage on  : Changes made according to the new approach

-- 7/7/2001 rjp   Add road_select signal to allow road_write signal to

--					directly load comparators

-- 7/31/2001 Arvindh Lalam: Using loadroad instead of road_write

--------------------------------------------------------------------

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

----------------------------------------------------------------------------------

--  Entity Declaration

---------------------------------------------------------------------------------

entity comparator is

port (clock,reset : in std_logic;

centroid : in unsigned (10 downto 0);  -- the centroid

uroaddata : in unsigned (10 downto 0); -- the upper road_data address

lroaddata : in unsigned (10 downto 0); -- the lower road)data address

road_select: in std_logic;  --  This signal is used to select the comparator.

loadroad: in std_logic;  -- This signal will load the current road data

-- into the comparator, if selected

hit : out std_logic      -- Used to return the comparator output

);

end entity comparator;

--------------------------------------------------------------------------------

--  Architecture Body

--------------------------------------------------------------------------------

architecture behavior of comparator is

-- internal signals for the process blocks

signal ndata,pdata, compare1, compare2 : std_logic;

signal nroaddatau,proaddatau,nroaddatal,proaddatal : unsigned (10 downto 0);

signal  write_enable : std_logic;

constant zero11 : unsigned ( 10 downto 0) := "00000000000";

begin

---------------------------------------------

-- this block is used for the comparison of the road-data with the given centroid

---------------------------------------------

process(proaddatau,proaddatal,centroid)

begin

nroaddatau<= proaddatau;

nroaddatal<= proaddatal;

compare1 <= '0';

compare2 <= '0';

if (proaddatau >= centroid ) then

compare1 <= '1';

else

compare1 <= '0';

end if;

if (proaddatal <= centroid) then

compare2 <= '1';

else

compare2 <= '0';

end if;

end process;

---------------------------------------------

-- this block gives the result of the comparison

---------------------------------------------

process (compare1,compare2)

begin

--  output is logical and of compare1 and compare2

ndata <= compare1 and compare2;

end process;

---------------------------------------------

-- the process block to register the road-data signals

---------------------------------------------

process(ndata,clock,reset)

begin

if ( reset ='0') then

pdata <= '0';

elsif ( clock'event and clock='1') then

pdata <= ndata;

end if;

end process;

---------------------------------------------

-- the process block to register the road-data signals

---------------------------------------------

process( nroaddatau,nroaddatal,reset,road_select,loadroad,lroaddata,uroaddata)

begin

write_enable <= loadroad and road_select;

if ( reset ='0') then

proaddatau <= zero11;

proaddatal <= zero11;

elsif (clock'event and clock='1') then

if(write_enable = '0') then

proaddatau <= nroaddatau;

proaddatal <= nroaddatal;

else

proaddatau <= uroaddata;

proaddatal <= lroaddata;

end if;

end if;

end process;

---------------------------------------------

-- final output from the comparator

---------------------------------------------

hit <= pdata;

end architecture behavior;

Hit-Word generator

--	File	: hit_generator.vhd

--  Author	: Arvindh Lalam.

--  Dated	: March 06, 2002.

----------------------------------------------------------------------

--	This module works in conjunction with two CAM blocks. CAM1 (31

--		locations) and CAM2 (15 locations) are used to store information

--		of all the 46 roads.

--

--	This module adds 31 to the address location of CAM2 to find the actual

--		ROAD number.

--

--	** Using async signals for DONE (HITS_DONE & VDONE), START, NEXT1, NEXT2.

----------------------------------------------------------------------

--  1. Encodes the "road number" that the centroid falls in.

--	2. Adds other information to generate the complete HIT to be stored.

--	3. Controls two external CAMs.

----------------------------------------------------------------------

--  Modified:

--  03/12/02 (Vindi)	: Completed debugging, transfering to Quartus.

----------------------------------------------------------------------

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

-------------------------------------------------------------------------------------

-- The Device PORTS

-------------------------------------------------------------------------------------

entity hit_generator is

PORT (

--** INPUTS

clock					: in std_logic;

reset					: in std_logic;

l3_trailer				: in unsigned(31 downto 0);

-- Contains EVENT_ID ;

-- Received from strip_reader_control_edf_final thru Monitor blcok.

pulsearea				: in unsigned(2 downto 0);

centroid				: in unsigned(12 downto 0);

-- Above two values are read from Centroid RAM of Centroid finder block

hitreg_valid			: in std_logic;

-- Implies new centroid at input ; Generated by Hit_control_block ;

hits_eof				: in std_logic;

-- Momentary pulse indicating END OF EVENT.

-- This is found one clock period after DONE goes low.

event_frc				: in unsigned(7 downto 0);

-- Event latched at FRC_START

cam_out1				: in unsigned(4 downto 0);

cam_out2				: in unsigned(3 downto 0);

-- Encoded CAM output and count of number of matches.

cam_found1, cam_found2	: in std_logic;

-- Goes HIGH if a match is found.

-- ** OUTPUTS

hits_done	: out std_logic;

-- Pulse of HITS_DONE indiacates that hits for a centroids are stored;

--	Next centroid can be read

hit_word				: out unsigned(31 downto 0);

hit_wr					: out std_logic;

-- Hit word and the async write signal

cam_start				: out std_logic;

cam_next1,cam_next2		: out std_logic;

-- CAM control signals.

-- ** DEBUG SIGNALS

hitf_ostate				: out std_logic_vector(3 downto 0);

err						: out std_logic

);

end hit_generator;

---------------------------------------------------------------------------------------

-- Architecture Body

---------------------------------------------------------------------------------------

architecture behaviour of hit_generator is

type mystates is (sreset, sinitialize, scam_start1, sdummy1, scam_next1, sstore_hits1,

sstore_hits2, sdummy2, scheck_eof, strailer_write);

signal nstate, pstate			: mystates;

signal nhit_word, phit_word		: unsigned(31 downto 0);

signal nhit_wr, phit_wr			: std_logic;

signal nhits, phits				: unsigned(7 downto 0);

constant zero5	: unsigned(4 downto 0) := "00000";

constant zero6  : unsigned(5 downto 0) := "000000";

constant zero8	: unsigned(7 downto 0) := "00000000";

constant zero32 : unsigned(31 downto 0) := "00000000000000000000000000000000";

constant trailer_head	: unsigned(4 downto 0)	:= "11110";

BEGIN

------------------------------------------

-- The Process starts

------------------------------------------

read_cam : process (clock,reset,l3_trailer,pulsearea,centroid,hitreg_valid,hits_eof,event_frc,

cam_out1,cam_out2,cam_found1, cam_found2, pstate, phit_wr,phit_word,

event_frc, phits)

variable vstart, vnext1, vnext2, vdone	: std_logic;

-- signals to control various CAMs.

variable veerr							: std_logic;

-- Mismatch of "EVENT number" between FRC and SMT.

variable vevent, vevent_frc      		: unsigned(7 downto 0);

-- SMT and FRC event IDs

variable verr_bits						: unsigned(3 downto 0);

-- SEERR, MM, RERR,EERR

variable vroad, vcam_temp1, vcam_temp2	: unsigned(5 downto 0);

-- temporary CAM address variables

BEGIN

----------------------------------------

-- Variable Initializations

----------------------------------------

vstart  := '0';

vnext1  := '0';

vnext2	:= '0';

vdone   := '0';

vcam_temp1 := '0' & cam_out1;

vcam_temp2 := "00" & cam_out2;

nhit_wr   	<= '0';

nhit_word 	<= phit_word;

nhits		<= phits;

vevent		:= l3_trailer(7 downto 0);

vevent_frc	:= event_frc;

veerr		:= '0';

--	Compares EVENT_IDs received from SMT & FRC and sets EERR in case of an error.

if(vevent = vevent_frc) then

veerr := '0';

else

veerr := '1';

end if;

--	ERROR bits			SERR + MM			+ RERR+ EERR

verr_bits	:= l3_trailer(26 downto 25) & '0' & veerr;

case pstate is

-- RESET state: Resets all the signals and waits for "GO" signal from HIT_CONTROL_BLOCK.

when sreset	=>

vstart  := '0';

vnext1  := '0';

vnext2	:= '0';

vdone   := '0';

vroad	:= zero6;

vcam_temp1 := zero6;

vcam_temp2 := zero6;

nhit_wr <= '0';

err		<= '0';

nhit_word <= zero32;

if(hitreg_valid = '1') then

nstate <= scam_start1;

else

nstate <= sreset;

end if;

----------------------------------------

-- Initiates the CAM1

----------------------------------------

when scam_start1 =>

vstart := '1';

nstate <= sdummy1;

----------------------------------------

-- Waits for CAM iniatialization.

----------------------------------------

when sdummy1 =>

vstart := '0';

nstate <= scam_next1;

----------------------------------------

-- Activates reading of multiple CAM locations.

----------------------------------------

when scam_next1	=>

vnext1 := '1';

nstate <= sstore_hits1;

----------------------------------------

-- Reads CAM1 address and writes HIT word.

-- If there are no matching contents, FSM goes to SSTORE_HITS2.

----------------------------------------

when sstore_hits1 =>

vnext1 := '1';

vnext2 := '0';

vroad := vcam_temp1;

--			6		3		<-------------	[5 		+ 	2]	+ 3	 -------------->	 13

--			road		area		SEQ	(bit3 of 1-8 is left out)		HDI				centroid

nhit_word <= vroad & pulsearea & l3_trailer(18 downto 14) & l3_trailer(12 downto 8) & centroid;

if (cam_found1 = '1') then

vnext1 := '1';

vnext2 := '0';

nhit_wr <= '1';

nhits	<= phits + 1;	 		-- Number of hits counted for an event

nstate <= sstore_hits1;

else

vnext1 	:= '0';

vnext2 	:= '1';

nhit_wr <= '0';

nhits	<= phits;

nstate 	<= sstore_hits2;

end if;

----------------------------------------

-- Reads CAM2 address, transforms into proper ROAD NUMBER and writes HIT word.

----------------------------------------

when sstore_hits2 =>

vroad := vcam_temp2 + "011111";

--				6		3		<-------------	[5 		+ 	2]	+ 3	 -------------->	 13

--			road		area		SEQ	(bit3 of 1-8 is left out)		HDI			centroid

hit_word <= vroad & pulsearea & l3_trailer(18 downto 14) & l3_trailer(12 downto 8) & centroid;

if (cam_found2 = '1') then

vnext2 	:= '1';

nhit_wr <= '1';

vdone	:= '0';

nhits	<= phits + 1;		  	-- Number of hits counted for an event.

nstate 	<= sstore_hits2;

else

vnext2 	:= '0';

nhit_wr	<= '0';

vdone	:= '1';

nhits	<= phits;

nstate	<= scheck_eof;

end if;

----------------------------------------

-- Checks for EOF and writes trailer if EOF.

----------------------------------------

when scheck_eof =>

vdone := '0';

if (hits_eof = '1') then

nhit_wr		<= '1';

--						5		  3		  8			 8		 					4			4

--			header		NULL	EVENT_ID	No. of hits of event 	   ERR bits		NULL

nhit_word 	<= trailer_head & "000" & vevent & phits 					& verr_bits & "0000";

nstate 	<= strailer_write;

else

nhit_wr		<= '0';

nhit_word	<= phit_word;

nstate 	<= sreset;

end if;

----------------------------------------

-- DUMMY state that the FSM stays in while writing TRAILER.

----------------------------------------

when strailer_write =>

nhit_wr	<= '0';

nstate 	<= sreset;

nhits	<= zero8;

when others =>

nstate 	<= sreset;

err 	<= '1';

end case;

----------------------------------------

--sstore_hits1, sstrore_hits2, sstore_hits2, sdummy2, scheck_eof, strailer);

----------------------------------------

cam_start	<= vstart;

cam_next1	<= vnext1;

cam_next2	<= vnext2;

hits_done   <= vdone;

END process read_cam;

----------------------------------------

-- REgister Block

----------------------------------------

latch	: process (clock, reset, nstate, nhit_word, nhit_wr, nhits)

BEGIN

if(reset='0') then

pstate 		<= sreset;

phit_wr		<= '0';

phit_word 	<= zero32;

--				phits_done	<= '1';

phits		<= zero8;

elsif (clock='1' and clock'event) then

pstate		<= nstate;

phit_wr 	<= nhit_wr;

phit_word 	<= nhit_word;

--				phits_done	<= nhits_done;

phits		<= nhits;

end if;

END process latch;

hit_wr 		<= phit_wr;

hit_word 	<= phit_word;

----------------------------------------

-- Encoded FSM states for debugging

----------------------------------------

with pstate select

hitf_ostate <= 	"0000" when sreset,

"0001" when sinitialize,

“0010" when scam_start1,

"0011" when sdummy1,

"0100" when scam_next1,

"0101" when sstore_hits1,

"0110" when sstore_hits2,

"0111" when sdummy2,

"1000" when scheck_eof,

"1001" when strailer_write,

"1111" when others;

end behaviour;

L3 Readout Control

------------------------------------------------------------------------------------

--	Program: l3_readout_control_edf.vhd

--	Created: by Arvindh Lalam (10/21/2001); used some of the code written

--				by Shawn Roper

--	Description: This is the control block for the l3 readout

--	Remarks	: Since the complexity of the program the FPGA express is used

--				synthesize the program.

--

-----------------------------------------------------------

--		Signal I/O

-----------------------------------------------------------

--	signal name		direction		description

--		clk			  input			  clock line

--		reset		  input			  reset line

--		done		  input			  done processing signal from control logic

--	  	Start_L3	  input			  event start for readout

--		INH			  input			  inhibit signals

--		start		  output		  event start for control logic

--		L3_DONE		  output		  high if done processign

--		L3_BUSY		  output		  high if busy processign

--		L3_BLOCK	  output		  selector for final data mux

-----------------------------------------------------------

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

------------------------------------------------------------------------------------

--	Declaration of the device ports

------------------------------------------------------------------------------------

entity l3_readout_control_edf is

port (clk,reset	: in std_logic;

smt_id	: in std_logic_vector(2 downto 0);

-- Smt ID of the channel.

l3_master_inhibit: in std_logic;

-- this is the master l3_busy bit

start_L3	: in std_logic_vector(9 downto 0);

-- start_l3 bits from main logic

l3_busy_inhibit	: in std_logic_vector(6 downto 0);

-- busy inhibit bits from other channels

l3_block_inhibit  : in std_logic_vector(6 downto 0);

-- block inhibit bits from other channels

l3_done_in: in std_logic_vector(4 downto 0);

--  indicates that a l3 buffer has finished block

l3_available: in std_logic_vector(4 downto 0);

-- indicates that a l3 buffer has data available

l3_wr_in:  in std_logic_vector(4 downto 0);

-- l3 write strobe out

data0,data1,data2,data3,data4: in std_logic_vector(31 downto 0);

l3_start_out      : out std_logic_vector(4 downto 0);

--	  l3_write_strobe   : out std_logic;   -- or'ed l3_wr's from buffers

l3_write			: out std_logic;	-- EXTERNAL L3 write signal.

l3_oen			: out std_logic;	-- Enables output bus of TRI_BUS module

l3_data_out       : out std_logic_vector(31 downto 0);

L3_BUSY_out		: out std_logic;

L3_BLOCK_out		: out std_logic;

l3_done_out       : out std_logic;

------------------------------------------

--	Debug pins.

------------------------------------------

l3_ctrl_ostate	: out std_logic_vector(2 downto 0);

vl3_done_out		: out std_logic;

l3_ready_out		: out std_logic;

vstart_l3_out		: out std_logic;

ifstat			: out std_logic_vector(1 downto 0);

l3_buffer_ready_out	: out std_logic_vector(4 downto 0)

);

end entity l3_readout_control_edf;

------------------------------------------------------------------------------------

--	Architecture Body Begins

------------------------------------------------------------------------------------

architecture behavior of l3_readout_control_edf is

type mystates is (s0,s1,s2,s3);

type myreadstates is (sinit,sdetermine,swait, sdone);

signal nreadstate,preadstate: myreadstates;

signal ns,ps : mystates;

signal nstart,pstart: std_logic_vector(4 downto 0);

signal nStart_L3, pStart_L3: std_logic_vector(9 downto 0);

signal l3_channel_busy,l3_ready: std_logic;

signal nl3_busy, pl3_busy,nreadout_l3,preadout_l3,pl3_block,nl3_block: std_logic;

signal preadout_done,nreadout_done: std_logic;

signal pl3_event,nl3_event,pend_l3,nend_l3: std_logic;

--  This bit is used to indicate a current l3 event

signal pend_l3_event, nend_l3_event: std_logic;

signal l3_buffer_ready:std_logic_vector(4 downto 0);

signal nbuffer_select,pbuffer_select:std_logic_vector(2 downto 0);

--	signal nl3_done_out, pl3_done_out: std_logic;

signal int_l3_data: std_logic_vector(31 downto 0);

signal rotate_bit		:std_logic;

signal channel			:std_logic_vector(2 downto 0);

signal l3_buffers_done: std_logic;

signal nl3_write, pl3_write	: std_logic;

constant zero32:std_logic_vector(31 downto 0) :="00000000000000000000000000000000";

constant zero10: std_logic_vector(9 downto 0) := "0000000000";

constant trailer_seq : std_logic_vector(4 downto 0) := "11110";

begin

------------------------------------------

--	This block checks to see if the channel is free.

--	Look at l3_master_inhibits and l3_channel_inhibits

------------------------------------------

process(l3_master_inhibit,l3_busy_inhibit,l3_block_inhibit)

variable i:integer;

variable vtemp: std_logic;

begin

vtemp := l3_master_inhibit;

for i in 0 to 6 loop

vtemp := (vtemp or (l3_busy_inhibit(i) or l3_block_inhibit(i)));

end loop;

l3_channel_busy <= vtemp;

end process;

------------------------------------------

--	This process block will check if l3 data is available in buffers

------------------------------------------

process(l3_available, l3_ready)

variable vl3_ready: std_logic;

variable i: integer;

begin

vL3_ready := '0';

for i in 0 to 4 loop

vL3_ready := vL3_ready or l3_available(i);

end loop;

l3_ready <= vL3_ready;

l3_ready_out <= l3_ready;

end process;

------------------------------------------

--	This FSM determines if the bus is available.

------------------------------------------

process(ps,preadout_l3, pStart_L3,l3_channel_busy, pl3_busy,l3_ready,

preadout_done,pl3_event)

begin

------------------------------------------

--	set defaults

------------------------------------------

ns <= ps;

nl3_busy <= pl3_busy;

nreadout_l3 <= preadout_l3;

nl3_busy	<= pl3_busy;

nl3_block	<= pl3_block;

case ps is

when  s0 =>

------------------------------------------

--	Are we in an L3 event with L3 data ready

--	Yes, request the channel by setting l3_busy high,  go to s1, else stay here

------------------------------------------

if((l3_ready = '1') and (pl3_event='1')) then

ns <= s1;

nl3_busy <= '1';

else

ns <= s0;

nl3_busy <= '0';

end if;

when s1 =>

------------------------------------------

--	we have data and have requested the channel by setting l3_busy high

--	wait for l3_channel to go low indicating that l3 channel is free

------------------------------------------

if(l3_channel_busy = '1') then

nl3_busy <= '1';  -- channel is still busy, keep request high

nreadout_l3 <= '0';  -- this signal is for another FSM to

-- start the readout process

ns <= s1;  		-- stay in this state

else

nl3_block <= '1'; --   we have the channel, set nl3_block high

nreadout_l3 <= '1';    --  tell readout FSM to start

ns <= s2;

end if;

when s2 =>

------------------------------------------

--	Channel is free.  Stay here until readout FSM is done

------------------------------------------

if(preadout_done = '1') then

------------------------------------------

--	There are no more channels with data ready to go..

--	So set nl3_busy and nl3_block low

------------------------------------------

nl3_busy <= '0';

nl3_block <= '0';

nreadout_l3 <= '0';   -- ok to set this to 0.

ns <= s0;   -- go back to s0 to wait for new data

else

------------------------------------------

--	More L3 data available, so stay here

------------------------------------------

nl3_busy <= '1';

nl3_block <= '1';

nreadout_l3 <= '0';   -- ok to set this to 0.

ns <= s2;

end if;

when others =>   -- ERROR, goto RESET state

nl3_busy <= '0';

nl3_block <= '0';

nreadout_l3 <= '0';

ns <= s0;

end case;

end process;

------------------------------------------

--	This process block controls the individual l3 buffers.

--	It determines which one should have the internal l3 bus,

--	It waits for "readout_l3" signal before doing anything

------------------------------------------

process(preadstate, pstart_l3,preadout_l3,l3_done_in,pl3_event,pend_l3_event,

start_l3, rotate_bit, channel, smt_id, preadout_done, pbuffer_select,

pstart, int_l3_data, l3_available, l3_buffer_ready)

variable vl3_done,vendl3,vStart_L3,vpstart_l3: std_logic;

variable vtemp, vptemp			:std_logic;

variable vl3_centroid_buffer, vl3_ext_wr			: std_logic;

variable i,j,iinc,iwr: integer;

begin

------------------------------------------

--	Initialize the signals

------------------------------------------

vStart_L3 := '0';

vtemp	:= '0';

vpstart_l3 := '0';

vptemp	:= '0';

ifstat <= "11";

nl3_event <= pl3_event;

-- indicates we are in a l3_event

nend_l3_event <= pend_l3_event;

-- indicates that current l3_event should end

rotate_bit <= start_l3(6);

channel	<= start_l3(9 downto 7);

------------------------------------------

--	This checks start l3 bits latched during

--	the current l3 event.  See ** below

------------------------------------------

for i in 0 to 5 loop

vtemp := vtemp or START_L3(i);

vptemp := vptemp or pStart_L3(i);

end loop;

if((rotate_bit = '1') and (channel = smt_id)) then

vStart_l3 := vtemp;

vpStart_l3 := vptemp;

ifstat <= "00";

elsif (rotate_bit = '0') then

vStart_l3 := vtemp;

vpStart_l3 := vptemp;

ifstat <= "01";

else

vStart_L3 := '0';

vpstart_l3 := '0';

ifstat <= "11";

end if;

vstart_l3_out <= vstart_l3;				--** DEBUG pin.

------------------------------------------

--	Checks if the l3 data transfer is complete.

------------------------------------------

if(preadout_done  = '1') then  --- should we end this l3 event;

nend_l3_event <= '0';  -- reset bit to prepare for the next l3 event.

nl3_event <= '0';

nStart_L3 <= zero10;

elsif(pl3_event = '0') then

-- are we currently waiting to start a l3 event

nl3_event <= vStart_L3;

-- yes, if any bit is high, we enter a l3 event

nStart_L3 <= Start_L3;     -- register start_l3

nend_l3_event <= '0';      -- help synthesis tool

else

nl3_event <= pl3_event;   -- we are in an l3 event.

nStart_L3 <= pstart_L3;   -- ** check pstart_l3 bits,

-- if all zeros then leave this l3 event

nend_l3_event <= not vpStart_L3;

--  on next cycle. Use vpStart_l3 generated above

end if;

------------------------------------------

--	Generate l3_buffer_ready signals.   These are equal to

--	l3_available = '1' i.e. the buffer has data AND

--	start_l3 = '1' i.e. we need the l3 data for this event

--	If this is one, then output its content, need a separate signal for each buffer

--	L3_avaiable should be registered by individual l3 buffers

------------------------------------------

nreadstate <= preadstate;

vendl3 := '0';

vl3_centroid_buffer := '0';

------------------------------------------

--	This calculates the parameters for the bits 1-3 of STRAT_L3

--	 which stand for centroids.

------------------------------------------

for i in 0 to 4 loop

iinc := i + 1;

vendl3 := vendl3 or pstart_l3(iinc);

l3_buffer_ready(i) <= pstart_l3(iinc) and l3_available(i);

end loop;

l3_buffer_ready_out <= l3_buffer_ready;

------------------------------------------

--	All of the done bits are ANDed to determine if

--	 the last start has been completed

------------------------------------------

vl3_done := '1';

for i in 0 to 4 loop

vl3_done := vl3_done and l3_done_in(i);

end loop;

l3_buffers_done<= vl3_done;

vl3_done_out <= vl3_done;

nreadout_done <= '0';

nbuffer_select <= pbuffer_select;

------------------------------------------

--	This logic is used to set l3_done_out

--	If we are in a l3_event, l3_done should be 0

--	If end_l3_event goes high, l3_done should be 1

--	L3_done should stay high, until reset by entering a new l3_event

------------------------------------------

--	nl3_done_out <= pl3_done_out;

l3_done_out <= '1';

if(pl3_event = '1') then

l3_done_out <= '0';

if(vl3_done = '1') then

l3_done_out <= '1';

end if;

end if;

------------------------------------------

--	Generating the external L3 write signal to send out of STC.

--	pbuffer_select bits are ORed to generate WR and OEN.

------------------------------------------

vl3_ext_wr := '0';

for iwr in 0 to 2 loop

vl3_ext_wr := vl3_ext_wr or pbuffer_select(iwr);

end loop;

if( int_l3_data(31 downto 27) = trailer_seq) then

nl3_write <= '0';

else

nl3_write	<= vl3_ext_wr;

end if;

l3_oen		<= not vl3_ext_wr;

------------------------------------------

--	The FSM starts here.

------------------------------------------

case preadstate is

------------------------------------------

--	SINTI State

------------------------------------------

when sinit =>

if(preadout_l3 = '1') then

nreadstate <= sdetermine;

else

nreadstate <= sinit;

end if;

------------------------------------------

--	SDETERMINE state

--  Need to determine which buffer gets to dump bus.

--  need to use if else to implement priority.

------------------------------------------

when sdetermine =>

nreadstate <= swait;

nstart(0) <= pstart(0);

nstart(1) <= pstart(1);

nstart(2) <= pstart(2);

nstart(3) <= pstart(3);

nstart(4) <= pstart(4);

nstart_l3(0) <= pstart_l3(0);

nstart_l3(1) <= pstart_l3(1);

nstart_l3(2) <= pstart_l3(2);

nstart_l3(3) <= pstart_l3(3);

nstart_l3(4) <= pstart_l3(4);

------------------------------------------

--	Check to see if a buffer is ready.

--	Must use if then else since priority is assumed.

------------------------------------------

if(l3_buffer_ready(0) = '1') then

nstart(0) <= '1';   --  yes

nstart_l3(0) <= '0';  --  reset this start_l3 bit;

nreadstate <= swait;    -- go to wait state;

nbuffer_select <= "001";

elsif(l3_buffer_ready(1) = '1') then

nstart(1) <= '1';

nstart_l3(1) <= '0';  --  reset this start_l3 bit;

nreadstate <= swait;    -- go to wait state;

nbuffer_select <= "010";

elsif(l3_buffer_ready(2) = '1') then

nstart(2) <= '1';

nstart_l3(2) <= '0';  --  reset this start_l3 bit;

nreadstate <= swait;    -- go to wait state;

nbuffer_select <= "011";

elsif(l3_buffer_ready(3) = '1') then

nstart(3) <= '1';

nstart_l3(3) <= '0';  --  reset this start_l3 bit;

nreadstate <= swait;    -- go to wait state;

nbuffer_select <= "100";

elsif(l3_buffer_ready(4) = '1') then

nstart(4) <= '1';

nstart_l3(4) <= '0';  --  reset this start_l3 bit;

nreadstate <= swait;    -- go to wait state;

nbuffer_select <= "101";

else

------------------------------------------

--	If we get here, then either all start_l3 bits have been reset

--	OR there is no more L3 content

------------------------------------------

nreadstate <= sdone;  -- go to sdone to "clean up"

nbuffer_select <= "000";

end if;

when swait =>  -- we started one of the l3 buffers,

--wait for done signal

nbuffer_select <= pbuffer_select;

if(vl3_done = '0') then

nreadstate <= swait;

else

------------------------------------------

--  "done" bit has been returned.  Turn reset all nstart bits and

--  return to sdetermine to see if we have other buffers to output

------------------------------------------

nstart(0) <= '0';

nstart(1) <= '0';

nstart(2) <= '0';

nstart(3) <= '0';

nstart(4) <= '0';

nreadstate <= sdetermine;

end if;

when sdone =>

------------------------------------------

--   Tell main FSM that we are done

------------------------------------------

nreadout_done <= '1';

nreadstate <= sinit;  -- go wait for a new preadout signal

nbuffer_select <= "000";

when others =>   -- should never come here

nreadout_done <= '1';

nbuffer_select <= "000";

nreadstate <=  sinit;

end case;

end process;

------------------------------------------------------------------------------------

-- Register Block

------------------------------------------------------------------------------------

reg: process (clk,reset,ns,nreadstate,nstart_l3,nl3_busy,nreadout_l3,nreadout_done,

nl3_event, nend_l3_event)

begin

if(reset = '0') then

ps <= s0;

preadstate <= sinit;

pl3_busy <= '0';

pl3_block <= '0';

preadout_l3 <= '0';

preadout_done <= '0';

pl3_event <= '0';

pstart <= "00000";

pstart_l3 <= "0000000000";

pbuffer_select <= "000";

pl3_write <= '0';

elsif(clk'event and clk = '1') then

ps <= ns;

preadstate <= nreadstate;

pl3_busy <= nl3_busy;

pl3_block <= nl3_block;

preadout_l3 <= nreadout_l3;

preadout_done <= nreadout_done;

pl3_event <= nl3_event;

pstart <= nstart;

pstart_l3 <= nstart_l3;

pbuffer_select <= nbuffer_select;

pl3_write <= nl3_write;

end if;

end process reg;

------------------------------------------

-- Assign outputs

------------------------------------------

l3_busy_out <= pl3_busy;

l3_block_out <= pl3_block;

l3_start_out <= pstart;

------------------------------------------

--   use CASE to select one of the data_outs.

------------------------------------------

with pbuffer_select select

int_l3_data <= data0 when "001",

data1 when "010",

data2 when "011",

data3 when "100",

data4 when "101",

zero32   when others;

with preadstate select

l3_ctrl_ostate <=  "000" when sinit,

"001" when sdetermine,

"010" when swait,

"011" when sdone,

"111" when others;

l3_data_out <= int_l3_data;

l3_write	<= pl3_write;

--  send l3_write_strobe unregistered

--	     l3_write_strobe <= l3_wr_in(0) or l3_wr_in(1) or l3_wr_in(2) or l3_wr_in(3) or l3_wr_in(4);

end architecture behavior;

L3 Write Control

------------------------------------------------------------------------------------

-- File	:	 raw_data.vhd

-- Created by Arvindh Lalam (09/24/01)

-- Description : Loads data into the FIFO and controls the counter.

------------------------------------------------------------------------------------

--  Modified on 02/21/02 Arvindh Lalam.

------------------------------------------------------------------------------------

library altera;

use altera.maxplus2.all;

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

------------------------------------------------------------------------------------

--	Declaration of the Device ports

------------------------------------------------------------------------------------

entity raw_data is

port (	clock, reset	: in std_logic;

-- Clock and reset signals.

event_start_int	: in std_logic;

-- Internal Event_start generated in Active channels.

ctrl_wr			: in std_logic;

-- WRREQ generated by data block sources.

eof				: in std_logic;

-- EOF generated by data block source.

enable			: in std_logic;

-- Enables RAW_DATA fifo depending on config bits.

l3_data			: in std_logic_vector(17 downto 0);

-- Input L3 data.

l3_data_out		: out std_logic_vector(31 downto 0);

-- DATA that goes into FIFO.

wrreq			: out std_logic;

-- WRREQ for the actual FIFO.

inc				: out std_logic;

-- INC increments counter.

ostate			: out std_logic_vector(2 downto 0)

-- Encoded State for Debugging.

);

end entity raw_data;

------------------------------------------------------------------------------------

--	Architecture Body Begins

------------------------------------------------------------------------------------

architecture behaviour of raw_data is

type raw_states is (sreset, sdecide, swait_for_start, strailer);

signal ns, ps	: raw_states;

constant zero14 : std_logic_vector(13 downto 0) := "00000000000000";

constant zero23 : std_logic_vector(22 downto 0) := "00000000000000000000000";

constant zero32 : std_logic_vector(31 downto 0) := "00000000000000000000000000000000";

begin

process(ctrl_wr,enable, eof,l3_data,event_start_int)

begin

------------------------------------------

--	Initializing the asynchronous signals

------------------------------------------

wrreq <= '0';

inc <= '0';

--	done <= '1';

l3_data_out <= zero32;

case ps is

------------------------------------------

--	Waits for event_start

------------------------------------------

when sreset =>

--		done <= '1';

if(event_start_int = '1') then

ns <= sdecide;

else

ns <= sreset;

end if;

------------------------------------------

--	Checks if any of the FIFOs are enabled.

------------------------------------------

when sdecide =>

--		done <= '1';

if(enable='1') then		            -- Yes : Proceed with storing data.

ns <= swait_for_start;

wrreq <= '1';

l3_data_out <= "10000" & "0000" & zero23;

else                                -- NO :  Go back to SRESET and wait

-- for next EVENT_START

ns <= sreset;

wrreq <= '0';

l3_data_out <= zero32;

end if;

------------------------------------------

--	Waits for WRREQ from the source block.

------------------------------------------

when swait_for_start =>

--		done <= 0;

ns <= swait_for_start;

if(ctrl_wr = '1') then

wrreq <= '1';

l3_data_out <= zero14 & l3_data;

if(eof='0') then

ns <= swait_for_start;

else

ns <= strailer;

end if;

end if;

------------------------------------------

-- clock 		  |	  |	  |    |

-- EOF			___|---|________

-- ctrl_wr		____________|---|___________

-- data			____________DDDDD_____________

-- WRREQ		____________|----|______

------------------------------------------

--	Writes the trailer for current event.

------------------------------------------

when strailer =>

--		done <= '0';

wrreq <= '1';

inc <= '1';

l3_data_out <= "11110" & "000" & "0" & zero23;

ns <= sreset;

when others =>

--	 	done <= '1';

ns <= sreset;

end case;

end process;

------------------------------------------------------------------------------------

-- Register Block

------------------------------------------------------------------------------------

process (clock, reset,ps)

begin

if(reset='0') then

ps <= sreset;

elsif(clock='1' and clock'event) then

ps <= ns;

end if;

end process;

------------------------------------------

-- Encode the State Information

------------------------------------------

with ps select

ostate	<=  "000" when sreset,

"001" when sdecide,

"010" when swait_for_start,

"011" when strailer,

"111" when others;

end architecture behaviour;